3GPP TSG-SA WG2 Meeting #168	S2-2503271

Goteborg, Sweden, April 07 – 11, 2025

Source:	China Telecom

Title:	Analysis of issues to be considered for end-to-end sensing service

Document for:	Discussion

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This paper presents an analysis of the issues that should be considered when designing end-to-end sensing service architecture, also points out the common aspects that can be studied in Q2.

1. Introduction

In SA#107, Rel-20 SID FS_Sensing_ARC was approved (see SP-250401), and the guidance on this SID (see SP-250418) is as following:

- Architecture Enhancement to support Integrated Sensing and Communication

  - Work can start on the study in Q2 2025

  - Any study work on Sensing in Q2 should be limited to common aspects for the study without any RAN specific or UE specific sensing modes. Any UE/RAN aspects will be aligned with RAN after TSG#108

Therefore, this paper proposes an analysis of issues that should be taken into consideration when designing end-to-end sensing service architecture, which can be used as a guidance for defining Key Issues and discussing detailed solutions for common aspects in Q2.

2. Discussion

2.1 Overall architecture design

Since Rel-20 FS_Sensing_ARC is a 5G-Advanced study item, the designing of sensing service architecture should use the current 5GC architecture as the baseline and focus on the essential functionalities. Considering future proof, sensing dedicated network function(s) (e.g., a Sensing Function with separated control plane and user plane entity) should be introduced to management the sensing service, collect and process sensing data from gNB or UE, and the overall architecture should be scalable to support different sensing modes/sensing scenarios, in which service consumer (e.g., AF or UE), sensing granularity, sensing transmitter(s) and sensing receiver(s) may be different.

In our view, the common part can be studied in Q2 are as follows:

- How the AF, NEF and the Sensing Function interact to support sensing service authorization/ invocation, sensing service revocation and sensing result exposure/report?

- To enable efficient collection of sensing data (e.g., via a UDP-based GTP-U tunnel to collect sensing data from gNB node) and flexible deployment of sensing network function(s), how to support separated Sensing Function by splitting it into a Sensing Service Control/Management Function and a Sensing Result Calculation Function?

2.2 Functionalities to enable end-to-end sensing

To enable end-to-end sensing services, the essential functionalities contain sensing service related information configuration and authorization, discovery and selection of sensing entities, sensing data collection and processing, sensing result exposure and sensing-related policy provisioning.

Figure 2.1: High level flow chart showing the end-to-end sensing service

Based on SA1 requirements of TS 22.137 and work tasks of the R20 Sensing SID, the detailed analysis of these essential functionalities are as follows:

### 1. Configuration and authorization

The configuration and authorization functionality may include the following:

1) Authorization of the sensing service requests provided by the service consumers:

When a service consumer (e.g., external AF/UE/internal NF) requests the sensing service from the network (e.g. 5GC), the network shall authorize the request(s) based on configured sensing operation related information (e.g., sensing operation area and/or the not allowed area).

2) Authorization of sensing service level registration of sensing transmitter(s)/receiver(s) to the CN:

When the sensing transmitter(s) and sensing receiver(s) need to register to the network as a sensing measurement participant, the network may need to authorize the sensing service level registration operation based on operator’s policies and/or the transmitter/receiver information (e.g., user consent if it is a normal UE).

Therefore, the following issues need to be considered:

- What and how sensing operation information will be used for the authorization of sensing service, and where to configure them accordingly?

- How to perform the authorization of sensing service request(s) from a service consumer?

- How to revoke the sensing service e.g., if the service consumer cancels the sensing service?

- When the configured sensing operation information in the network changes, whether and how to update/revoke the sensing service?

- How to perform the authorization of sensing service level registration of different sensing transmitter(s) and sensing receiver(s)?

- When the transmitter/receiver information (e.g., user consent if it is a normal UE) in the network changes, whether and how to update their registration information or de-register the transmitter/receiver in the network?

The above issues in bullet 1, 2 and 3, which are sensing mode transparent, can be addressed in Q2.

### 2. Discovery and selection of sensing entities

The sensing entities in 5GS consist of both the Core Network sensing entities (e.g., Sensing Function entity) and the sensing transmitter(s) and sensing receiver(s) (e.g. RAN and/or UE) and we assume the sensing transmitter(s) and sensing receiver(s) will perform sensing service level registration to core network sensing entities.

To achieve the discovery and selection of different sensing entities, the following issues need to be considered:

- Whether and how to define the capability of different sensing entities (e.g., the capability of SF entity, the capability of sensing transmitter(s) and sensing receiver(s))?

- Whether and how to register the capabilities of different sensing entities to the network?

- Whether and how to select and modify sensing entities based on the capabilities of sensing entities?

- Whether and how to select and modify sensing modes based on sensing service request(s) and the capabilities of sensing entities?

In our view, the selection of Sensing Function entities can be studied in Q2 (e.g., when NEF receives a sensing request from AF, how to select the appropriate SF). The selection of Sensing Functions can be discussed along with the consideration of control/management plane and user plane separation (i.e., how to select a sensing control/management entities).

### 3. Sensing data collection and processing

For sensing data collection, sensing receiver(s) may collect sensing data based on the requirements in sensing service request(s). The following aspects need to be considered:

- Input/output content and format of sensing data when transmitted from the sensing receiver to the CN.

- Data transmission paths and protocols, e.g., whether it can be transmitted via the control plane or user plane; what protocol it can use for data transmission.

For sensing data processing, the following aspects need to be considered:

- How to select the processing network function/entity in 5GC for 3GPP sensing data sensing data processing?

- How to select and configure transmitter/receiver for sensing data collection?

- How to process sensing data based on sensing service requests and derive desired content or format for exposure?

Since the sensing data collection part is related to specific sensing mode and has strong RAN dependency, it can be discussed after the final scope is determined.

The selection of sensing processing network function/entity in 5GC can be discussed along with the consideration of separation of sensing control/management plane and user plane.

### 4. Sensing result exposure

From our side, the following aspects need to be considered for sensing result exposure:

- What kind of information will be included in the sensing result for exposure?

- Whether sensing result exposure can be performed via control plane and/or user plane?

The above issues can be addressed in Q2.

### 5. Sensing-related policy provisioning

Policy information may be required in sensing service for sensing control and sensing data transmission. The following aspects need to be investigated:

- Whether and how sensing or policy related functions determine sensing-related policies and provides related configuration information to gNB/UE?

- Whether and how the network performs sensing data processing and sensing result exposure based on differentiated QoS requirement information of sensing service request?

This functionality is also related to specific sensing mode and has dependency to RAN, therefore, not in a hurry to be discussed in Q2.

3. Conclusion

Proposal 1: It is proposed to start the study on the overall architecture design, sensing service related information configuration, the procedure of sensing service authorization, revocation and sensing result exposure, and discovery and selection of sensing entities of core network in Q2.