3GPP TSG-WG SA2 Meeting #168	S2-2503442

Gothenburg, SE, 7th April – 11th April, 2025

Source:	NTT DOCOMO, T-Mobile USA

Title:	Discussion on assistance information to support command

Document for:	Discussion

Agenda Item:	19.14.2

Work Item / Release:	Architecture support of Ambient power-enabled Internet of Things (AmbientIoT-ARC) / Rel-19

Abstract: This paper discusses the need for assistance data to support command

1. Introduction

This paper discusses potential issues to support command, namely when there is a time delay between the AIOT NAS Inventory response and the delivery of a AIOT NAS Command request, the case where inventory and command occur in parallel and multiple AIOT RAN(s) are involved.

2. Discussion

2.1	Case 1: Inventory first, then Command

Figure 2.1-1 Ambient IoT service operation

2.1.1	AIOTF AIOT RAN during inventory

Figure 2.1-1 illustrates an Application Function requesting a Command service operation. To perform the command, the AIOTF first triggers an inventory request to find the candidate Ambient IoT device(s).

The AIOTF forwards the inventory request to the AIOT RAN that executes an Ambient IoT paging to trigger candidate Ambient IoT device(s) to initiate Random access.

In step 5, the candidate Ambient IoT device(s) provides an Inventory response with the Ambient IoT device ID to the AIOT RAN, AIOT RAN forwards the Inventory response to the AIOTF.

Observation 1: If the inventory has a subsequent command, the AIOT RAN should have some kind of mapping to be able to deliver the pending command to the correct Ambient IoT device. If it is only an inventory without subsequent command, the AIOT RAN should not create a mapping and may release the session towards the Ambient IoT device.

Proposal 1: The AIOTF indicates to AIOT RAN, as part of assistance data, whether or not a pending command is expected with the Ambient IoT device(s) so that the AIOT RAN can avoid creating unnecessary mappings and blockage of radio resources. This would address the Editor’s Note in TS 23.XYZ 020

Editor's note:	Whether the Inventory Request sent to AIoT RAN includes indication about whether there will be a follow up command or not needs to be determined.

2.1.2 AIOTF AIOT RAN during command

Figure 2.1.2-1 Ambient IoT command operation

Figure 2.1.2-1 illustrates an example of the call flow for performing a command, first inventory is performed including pending command assistance information, then a command is sent from the core to the AIOT RAN.

2.2	Case 2: Inventory and Command in parallel

During SA2#167 Athens it was discussed whether or not a command towards several devices can operate in a parallel mode, e.g., whilst collecting inventory responses, start command procedure for the devices that have already responded. With the assumption that the core network is likely to require more than one AIoT RAN to reach target Ambient IoT device(s) supporting inventory and command in parallel may require some thought to handle network resources responsibly. The following Editor’s Notes are captured in 23.xyz 020:

Editor's note:	It is FFS how to handle the scenarios that the Inventory Response received for Command Request sent for many individual devices are in parallel.

Editor's note:	Further clarification and enhancement on step 7 to step 11 of procedure for Inventory specified in clause 6.2.2 is FFS.

Figure 2.2-1 Ambient IoT command operation across multiple AIOT RAN

Figure 2.2-1 illustrates an example where the AIOTF relies on two AIOT RANs to fulfil the command. Since the AIOT RAN#2 is unaware the AIOT RAN#1 has successfully reached the Ambient IoT device, the AIOT RAN#2 continues to attempt to fulfil the inventory request.

Observation 2: When inventory and command are executed in parallel, there needs to be coordination to avoid spending unnecessary resources.

Proposal 2: It is proposed to extend the correlation ID to also comprise of a status for inventory and command. Status can be pending and complete. In scenarios where the AIOTF is certain all devices have been reached by the paging, the status for inventory can be set to “complete”. In a scenario where the target devices are not exactly specified e.g., an area, the AIOT RAN can set inventory to “complete” once all paging is done.

Proposal 3: At the end of the command service, the AIOTF can set command status to “complete” and send it to involved AIOT RAN(s) so that the involved AIOT RAN(s) can clear up any pending mappings for the correlation ID. Since several AIOT RAN(s) may have prepared to deliver the command to the Ambient IoT device due to overlapping responses during inventory.

3. Conclusion and proposal(s)

Based on the observations in clause 2, it is proposed to include information in the correlation ID as assistance data that indicates an inventory is to be followed by a subsequent command and the current status of the inventory and subsequent command as proposed in S2-2503466 and S2-2503469.