3GPP TSG-SA WG2#168	S2-2503713

Gothenburg, Sweden, 7-11 April 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	New Solution to WT#2: Enabling dynamic QoS handling for energy efficiency lterby use of Energy Saving QoS Profiles

Document for:	Discussion/Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes to enable dynamic QoS handling based on whether energy efficiency mechanisms are being used, by defining an Energy Efficiency QoS profile.

# 1.	Discussion

This contribution proposes to enable dynamic QoS handling by define Energy Saving QoS profiles.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-67:

# >>>>BEGINNING OF CHANGES (all new text) <<<<

## Y.X	New Solution to WT#2: Enabling dynamic QoS handling for energy efficiency by use of Energy Saving QoS Profiles

## Y.X.1	Description

This solution addresses WT#2.

This solution proposes that PCF generates the Energy Saving QoS profile for the service flow. AF may provide the Energy Saving Service Requirements in addition to the QoS Reference, QoS parameters or Alternative Service Requirements for a service flow. PCF authorizes the service information from the AF and generates the PCC rule which includes the list of Energy Saving QoS profile and the priority based on AF’s request. If AF does not provide the Energy Saving Service Requirements, PCF may generate the Energy Saving QoS profile and the priority based on local configuration.

The PCF shall enable QoS Notification Control and include the derived Energy Saving QoS profile with other QoS profile in the PCC rule sent to the SMF. SMF shall provide the Energy Saving QoS profile together with other QoS profile to the NG-RAN.

This Energy Saving QoS profile is only used when the specific QoS flow is subject to degradation of the QoS parameters due to the energy saving in NG-RAN. The Energy Saving QoS profile is applied to both GBR and non GBR bearer.When the Energy Saving QoS profile is applied for the certain QoS flow due to energy saving mechanism, NG-RAN will notify the SMF that the Energy Saving QoS profile is applied. SMF may indicate the used Energy Saving QoS profile to PCF and also forwards to UPF for charging purpose.

## Y.X.2	Procedures

AF provides the Energy Saving Service Requirements to PCF by reusing the setting up an AF session with required QoS procedure as described in clause 4.15.6.6 in TS 23.502, with the following addition:

- AF may provid Energy Saving Service Requirements to PCF.

- PCF generates a list of Energy Saving QoS profiles and the priority list for the QoS flow based on AF’s request and local configuration. the Energy Saving QoS profile can be applied to both GBR and non-GBR QoS flows.

PCF provides the Energy Saving QoS profile to SMF and NG-RAN by reusing the PDU session establishment as described in clause 4.3.2 in TS 23.502 and PDU session modification in clause 4.3.3 in TS 23.502 with the following addition:

- PCF provides the Energy Saving QoS profile and the priority list along with other QoS profile to SMF.

- SMF provides the Energy Saving QoS profile and the priority list along with other QoS profile to NG-RAN.

NG-RAN sends the notification control with Energy Saving QoS profile as described in clause 5.7.2.4.1b in TS 23.501 and the AN initiated modification as described in clause 4.3.3.2 and clause 4.3.3.3 in TS 23.502 with the addition:

- NG-RAN sends the notification with the selected Energy Saving QoS profile

- SMF sends the selected Energy Saving QoS profile to PCF.

- SMF sends the selected Energy Saving QoS profile to UPF for charging purpose.

## Y.X.3	Impacts

PCF

-	New PCC rules related to  Energy Saving QoS profile due to energy efficiency

SMF

- Indicate Energy Saving QoS Profile to NG-RAN

- Receive notification of selected Energy Saving QoS Profile from NG-RAN, and propagating that information to PCF and UPF.

NG-RAN

-	Use of Energy Saving QoS Profile for energy efficiency if Energy saving is applied to the QoS flow.

-	Notification of use of Energy Saving QoS Profile to SMF.

UPF

- Indication of Energy Saving QoS Profile usage in CDR.

# >>>>END CHANGE<<<<