3GPP TSG-SA2 Meeting #168	S2-2503800

Goteborg, Apr 7th – 11th 2025

Source:	Huawei, HiSilicon

Title:	Call flow of the sensing study

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Providing on the general call flow of the sensing study for FS_Sensing_ARC is proposed.

# 1. Introduction

This pCR proposes the general call flow for sensing, which in general includes all KIs regarding the study. Specifically, a Sensing Function, SF, is introduced to manage the sensing related functionalities within 5GC as much as possible, so as to provide a future-proof resolution.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X | X | X | X | X | X | X |

# * * * * Second change (All Text New) * * * *

## 6.X	Solution #X: General framework for sensing

### 6.X.1	Description

This solution aims to provide the general call flow of the sensing.

The main principles of the procedure are the following:

-	AF as the service consumer initiates the sensing service by sending the sensing service subscription request to the 5GC, e.g., to NEF or to a selected Sensing Function (SF).

-	NEF determines an appropriate SF according to the sensing service request.

-	The SF continues the steps with the following aspects:

-	Discovers the corresponding downstream nodes (e.g. Sensing Transmitter/Receiver) and provides the configuration parameters to them;

-	if the reporting of 3GPP Sensing Data is performed via user plane, the SF establishes the path for the reporting of the Sensing Data from the Sensing Transmitter(s)/Receiver(s).

-	The 3GPP Sensing Data is transmitted to the SF, which further process it to evaluate the Sensing Result and exposes the Sensing Result to the AF via the NEF.

### 6.X.2	Procedure of the general sensing framework

Figure 6.X.2-1: Procedure of the sensing general framework.

1.	The AF requests to the NEF to obtain the Sensing Result from the network via Nnef_SensingExposure_Subscribe request.

NOTE 1:	Detailed information of sensing service requirement, sensing service type and corresponding information will be addressed by the solution of Key Issue #2.

2.	Upon receiving request from AF, the NEF authorizes the AF for usage of the sensing service.

3. 	The NEF may discover and select the candidate Sensing Function(s).

NOTE 2:	Details regarding NEF authorising the AF will be addressed by the solution of Key Issue #2.

4.	The NEF requests to the Sensing Function, and the request message contains information received in step 1.

5.	Optionally, the Sensing Function may perform further authorisation for the usage of the sensing service.

NOTE 2:	SF and/or NEF may respond to the upstream nodes indicating the result of sensing service subscription and optionally the reason.

NOTE 3:	Details regarding Sensing Function discovery will be addressed by the solution of Key Issue #3.

6.	If SF performs authorisation, it may indicate whether the request has been authorised or rejected via Nsf_SensingSubscription Response.

7. 	The NEF notifies the AF whether the subscription has been authorised or rejected via Nnef_SensingSubscription Response. If the SF is not involved in authorisation, this step can be performed after step 2.

8	The Sensing Function performs the downstream sensing entities discovery and selection, based on e.g., the sensing service requirement, and sensing capability supported by the downstream sensing entities (e.g., Sensing Transmitter(s)/Receiver(s)).

NOTE 4:	The details regarding downstream sensing entities discovery and selection will be addressed by the solution of Key Issue #3.

9.	The Sensing Function determines the information for the subsequent Sensing measurement and sends the sensing service request message including the configuration parameters to the downstream nodes (e.g., Sensing Transmitter(s)/Receiver(s)).

NOTE 5:	Details regarding Sensing Function sending the configuration parameters will be addressed by the solution of Key Issue #1 and Key Issue 6.

10.	Upon receiving the sensing service request message from the Sensing Function, the Sensing Transmitter(s)/Receiver(s) may create the path for 3GPP Sensing Data reporting if the 3GPP Sensing Data transmission is performed via user plane.

11.	The Sensing Transmitter(s)/Receiver(s) performs the Sensing measurement and obtains 3GPP Sensing Data.

12.	The 3GPP Sensing Data(s) are transmitted to the Sensing Function.

NOTE 5:	Details regarding the 3GPP Sensing Measurement Data and the reporting will be addressed by the solution of Key Issue #4.

13.	After receiving the 3GPP Sensing Data, the Sensing Function determines the Sensing Result.

14.	The Sensing Function sends the Sensing result to the NEF via Nsf_Sensing_notify.

15.	The NEF provides the Sensing result to the AF via via Nnef_Sensing_notify.

NOTE 6: The steps 11-15 may be performed one-time or a periodically or based on event trigger depending by the sensing service request from AF.

NOTE 7:	Details regarding the Sensing result will be addressed by the solution of Key Issue #5.

Editor’s note: the Service message names are intended as reference and they might be further detailed in any specific solution.

Editor’s note: the detailed procedure between the Sensing Function and the Sensing Transmitter(s)/Receiver(s) from step 8 to 11 requires further study.

### 6.X.3	Impacts on services, entities and interfaces

NEF:

-	Selection of Sensing Function.

-	Interaction with AF and Sensing Function for sensing authorization, invocation and revocation.

-	Provision the Sensing Result retrieved from Sensing Function to AF.

Sensing Function:

-	Interaction with NEF for exposure of Sensing Results and reception of sensing requests.

-	Interaction with downstream sensing entities for requesting to perform sensing measurement.

-	Reception of 3GPP Sensing Data;

-	Determine the Sensing Result based on the 3GPP Sensing data.

AF:

-	Interaction with 5GC for requesting sensing service to obtain the Sensing Result(s).

-	Acquisition Sensing Result(s) from 5GC.

# * * * * End of changes * * * *