3GPP TSG-SA2 Meeting #168	S2-2503802

Goteborg, SE, Apr 7th – 11th 2025

Source:	Huawei, HiSilicon

Title:	KI#3, New Sol: Discovery and selection of sensing entities

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The Discovery and selection of sensing entities for FS_Sensing_ARC are proposed.

# 1. Introduction

This pCR proposes the solution for Key issue#3 Discovery and selection of sensing entities

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x1]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  |  | X |  |  |  |

# * * * * Second change (All Text New) * * * *

## 6.X	Solution #X: Discovery and selection of Sensing Function

### 6.X.1	Description

NOTE:	This sub-clause provides the details of the discovery and selection of Sensing function procedure, and corresponds to step 1 (in this procedure, step 2) and 3 (in this procedure, step 1, 3-6) of the sensing general framework.

This solution aims to address key issue#3 on how to perform Sensing function discovery and selection.

Specifically, the NEF requests NRF for Sensing Function discovery and then select Sensing Function based on the candidate Sensing Function from NRF. To ensure that the selected Sensing Function suitable for the required sensing service, the NEF determines based on the supported service area of the Sensing Function and Target Sensing Service Area requested by the AF.

The Sensing function may register itself to the NRF, update its information, or deregister itself, as defined in clause 4.17.1.1, 4.17.2, and 4.17.3 of TS 23.502 [x1], respectively. In case of registration, the Sensing Function sends the Nnrf_NFManagement_NFRegister with additionally contains the Sensing Function capability information, e.g., Supported Sensing Service area.

After the NEF authorise the request from the AF, the NEF performs the discovery of Sensing Function as defined in TS 23.502 [x1] clause 4.17.4, where the request message sent to NRF contains the sensing service requirement derived from the request sent by AF and sensing capability (e.g., if support the List of objects within the Target Area, Crossing border of an area, Path monitoring and tracking).

The NRF authorizes the request and determines whether the NEF is allowed to discover the expected Sensing Function instance(s). Then the NRF determines a set of Sensing Function instance(s) matching the information within the Nnrf_NFDiscovery_Request message and the internal policies.

The NEF determines the Sensing Function instance(s) based on the supported Sensing Service area of the Sensing Function, sensing capability, etc.

The following information are included in the Sensing Function profile in the NRF:

-	Supported Sensing Service area.

-	Sensing Capabilities (e.g., if support the List of objects within the Target Area, Crossing border of an area, Path monitoring and tracking, etc)

The following factors may be considered for Sensing Function discovery and selection by NEF:

-	Supported Sensing Service area.

- 	Sensing Capabilities.

-	Sensing service requirement sent by AF.

### 6.X.4	Impacts on services, entities and interfaces

NEF:

-	Support Sensing Function discovery and selection

NRF:

-	Support Sensing Function capability registration.

# * * * * End of changes * * * *