3GPP TSG-WG SA2 Meeting #168	S2-2503846

Goteborg, SE, 7th April – 11th April, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	pCR for TS 23.369: Procedure for AIOTF and AIoT Reader Communication

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: The procedure for AIOTF and AIoT Reader Communication is added to specify how AIoT Information is transferred between AIOTF and AIoT Reader.

1. Introduction/Discussion

In this release, there are two topologies for the connectivity between AIOTF and NG-RAN. Since how AIOTF and NG-RAN handle the service operation makes no difference in these two topologies, it is recommended to specify how the AIoT information is transferred and routed in these two architectures separately and make it decoupled with the Inventory procedure and Command procedure for better understanding.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service requirements for Ambient power-enabled IoT".

[3]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[6]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[7]	GS1 TDS Release 2.1: "EPC Tag Data Standard".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[x]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

# * * * * First change * * * *(All new text below)

6.X	Procedures for AIOTF and NG-RAN Communication

6.X.1	General

Ambient IoT Supports different paths for communication between NG-RAN and AIOTF, as defined in clause 4.2.2. The procedures defined in clause 6.2 use these procedures for communication between the AIOTF and NG-RAN.

The handling of the different communication paths is described in the flowing clauses.

6.X.2	Procedures for Direct Connectivity between AIOTF and NG-RAN

When the AIOTF and NG-RAN use the N116 interface then the AIOTF uses NGAP to communicate with NG-RAN as defined in TS 38.413 [x].

6.X.3	Procedures for Indirect Connectivity between AIOTF and NG-RAN

An AIOTF connects indirectly to NG-RAN via an AMF in the Indirect Connectivity path, as described in clause 4.2.2.3, with the AIOTF using services provided by an AMF to route information to and from NG-RAN. The additional steps used for Indirect Connectivity between AIOTF and NG-RAN are shown in Figure 6.X.3-1.

Figure 6.3.3-1: AIOTF and NG-RAN Indirect Connectivity via an AMF

1.	When an AIOTF sends information to NG- RAN via an AMF, it includes information and parameters in an AIoT Container for the required operation. The IE(s) included in the AIoT Container is the same as the one sent from AIOTF to NG-RAN when an AIOTF and NG-RAN use Direct Connectivity. The AIOTF sends Namf_AIoT_MessageDelivery Request to the AMF which has an N2 interface towards NG-RAN, including AIOT RAN ID to identify the target NG-RAN node and the AIoT Container. After receiving the Namf_AIoT_MessageDelivery Request, the AMF sends Namf_AIoT_MessageDelivery Response to the AIOTF indicating whether the message will be delivered.

2.	The AMF sends an NGAP message to the target NG-RAN, and includes the AIOTF ID and the AIoT Container from the AIOTF.

3.	When receiving an AIOT NGAP message, the NG-RAN shall store the mapping for the requested operation, AIOTF ID and the AMF which sent the NGAP message.

4.	When NG-RAN sends responses to AIOTF via an AMF the NG-RAN includes an AIoT Container with request specific responses, and the AIOTF ID received in the request in step 2. NG-RAN determines the AIOTF ID and AMF based on the mapping stored in step 3.

5.	AMF sends the Namf_AIoT_Notify message to the AIOTF identified by the AIOTF ID received in step 4, including the AIOT RAN ID and the AIoT Container received from the NG-RAN.

NOTE:	The details of message type and IE used between AIOTF and NG-RAN and between NG-RAN and AMF are defined in TS 38.413 [x].

# * * * * End of changes * * * *