3GPP TSG-WG SA2 Meeting #168	S2-2503847

Goteborg, SE, 7th April – 11th April, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	pCR for TS 23.369: Support of AIoT Device Temporary ID

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: the discussion paper provided observation on SA2 scope to support AIoT device temporary ID, and proposed solution principles to support AIoT device temporary ID..

1. Discussion

SA3 concluded in TR 33.713, key issue #3 Privacy by protecting AIoT device identifiers, that following aspects and principles are agreed for the conclusion on KI#3:

-	A mechanism to protect AIoT device ID based on the use of temporary ID shall be supported.

-	Mechanism shall allow unambiguous identification of the AIoT device

-	A mechanism to re-synchronize de-synchronized temporary IDs shall be supported.

Editor’s note: Additional conclusions on solution are FFS.

SA2 WID also included “device temporary ID”, it is necessary to work with SA3 to specify the overall support on device temporary ID.

2. SA2 work on temporary ID

SA2 should focus on the following two aspects:

- temporary ID allocation

- use of temporary ID in the inventory procedure

The other aspects belong to SA3 scope and SA2 will align with SA3 later.

2.1 Temporary ID allocation

Two options of temporary ID allocation are described in SA3 TR:

- (Network allocated option) Network generates the temporary ID and provisions to the AIoT device, or

- (Device generated option) AIoT device autonomously generates the temporary ID locally, based on the pre-configured parameter e.g. pre-configured key and the AIoT device permanent ID

In case of network allocated option, AIOTF supports the temporary ID generation and provisioning. The command procedure (write operation) is used by the AIOTF to provision the temporary ID to the device. Temporary ID sent in the AIoT NAS message is encrypted. The trigger of allocation of the temporary ID is determined by the AIOTF.

AIoT device stores the allocated temporary ID in the non-volatile memory, and acknowledge the network. If the AIoT device has an old temporary ID, it should overwrite the old temporary ID with the new temporary ID.

AIOTF considers the success of temporary ID when receiving the acknowledgement from the AIoT device. If the AIOTF does not receive the acknowledgement from the AIoT device, the AIOTF may re-try the allocation for a configured time. When the allocation is successful, the AIOTF only stores the newly allocated temporary ID in the device data profile, but in the unsuccessful case the AIOTF stores both the new and old temporary ID in the device data profile.

The AIoT device and the network may be pre-stored the same temporary ID, which can be updated later.

In case of device generated option, no standalone procedure is needed to provision the AIoT device with the temporary ID.

2.2 Use of temporary ID in the inventory procedure

Two scenarios are considered: group inventory and single device inventory.

Group inventory means the AIOTF provides the filtering information (e.g. MASK) to reader for paging. The filtering information is generated based on AIoT device permanent ID.

Single device inventory means the AIOTF provides device temporary ID to reader for paging.

- Group inventory

In case of network allocated option:

When receiving the filtering information (e.g. MASK) in the paging message, the AIoT device matches the filtering information with the AIoT permanent ID, and if matched the AIoT Device reports its temporary ID to the network.

The AIOTF derives the device permanent ID from the device temporary ID, based on the mapping stored in the device data profile.

In case of device generated option:

The AIoT device matches the filtering information with the AIoT permanent ID and generates the AIoT temporary ID locally. The generation by AIoT device can be based on the pre-configured parameter e.g. pre-configured key, and the AIoT device permanent ID. The AIoT device reports its temporary ID to the network.

As an implementation, the AIoT device can generate the temporary ID when receiving the paging message, and remove the device temporary ID after reporting to the network. This approach is beneficial for the device to save the device permanent memory (non-volatile memory).

The AIOTF also generates the device temporary ID using the same approach as the AIoT device. When receiving the reported AIoT temporary ID, the AIOTF filters the device ID based on the logic to construct the filtering information.

- Single device inventory

In case of network allocated option:

The AIOTF provides the device temporary ID to the reader for paging.

When receiving the AIoT device matches the filtering information with the provisioned AIoT temporary ID, and report its temporary ID to the network.

In case of device generated option:

The AIOTF provides the device temporary ID (hashed) to the reader for paging.

The AIoT device firstly locally generated AIoT temporary ID. The generation by AIoT device can be based on the pre-configured parameter e.g. pre-configured key, and the AIoT device permanent ID.

The AIoT Device matches the filtering information with the locally generated AIoT temporary ID.

As an implementation, the AIoT device can generate the temporary ID when receiving the paging message, and remove the device temporary ID after reporting to the network. This approach is beneficial for the device to save the device permanent memory (non-volatile memory).

3. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change (all new texts) * * * *

5.7.X	AIoT Device Temporary Identifier

5.7.X.1	General

The AIoT Device Temporary Identifier is used to protect the AIoT Device Permanent Identifier to be transmitted on interfaces within the AIoT system.

5.7.X.2	Allocation of AIoT Device Temporary Identifier

An AIoT Device Temporary Identifier may be allocated to the AIoT Device using one of the following options:

- Option 1: AIOTF generates the AIoT Device Temporary Identifier and provisions to the AIoT Fevice.

- Option 2: AIoT Device generates the AIoT Device Temporary Identifier locally, using the parameter provided by the AIOTF together with the locally stored parameters (AIoT Device Permanent Identifier and security parameter).

For option 1, AIOTF initiates a command procedure (write operation) to provision the AIoT Device Temporary Identifier.

For option 2, AIOTF, in addition to the other parameters in an inventory request message, provides additional parameters to enable the AIOT Device to generate a AIoT Device Temporary Identifier.

NOTE X: For option 1, how the AIOTF generates the AIoT Device Temporary Identifier is implementation specific, and depends on operator policy.

NOTE Y: For option 1, the trigger for AIoT Device Temporary Identifier (re-)allocation depends on the AIOTF implementation. For example, the trigger can be the AIoT Device Temporary Identifier is updated in the inventory and command procedure.

Editor’s note: It is needed to align with SA3 in terms of the allocation option.

Editor’s note: In case of allocation option 2, parameter(s) provided by the AIOTF is pending SA3 decision. Also, security parameter locally stored in the AIoT Device is pending SA3 decision.

5.7.X.3	Use of AIoT Device Temporary Identifier

During the inventory procedure, AIOTF may use a AIoT Device Temporary Identifier to target a single AIoT Device.

NOTE X: In case of group paging, filtering information is used for paging.

When receiving an AIoT Device Temporary Identifier in a paging message, the AIoT Device may either match it with the AIoT Device Temporary Identifier or with the locally generated AIoT Device Temporary Identifier.

Once the match is successful, the AIoT Device reports its AIoT Device Temporary Identifier to the AIOTF in the inventory response.

# * * * * End of changes * * * *