TSG SA WG2 Meeting #168	S2-2503910

07 - 11 April, 2025, Goteborg, SE

Title:	[Draft] Reply LS on Transport Level Marking on N3/N9

Response to:	LS on Transport Level Marking on N3/N9s (C4-250629/S2-2502815)

Release:	Release 19

Work Item:	XRM_Ph2

Source:	SA2

To:	CT4

Cc:

Contact Person:

Name:	Lizhuo Zheng

E-mail Address:	zhenglz4 AT lenovo DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR6230(TS23.501)

1. Overall Description:

SA2 has discussed CT4(C4-250629/S2-2502815) LS on Transport Level Marking on N3/N9 and approved the attached CR. The following principles apply:

1] If I-SMF/I-UPF is inserted in the PDU session, the I-UPF shall not check the PSI value of the incoming packets.

2] If I-SMF is notified that the QoS flow is related to XRM traffic, e.g., by receiving PDU set QoS parameters, then no transport level packet marking value will be included in the FAR to I-UPF. Under this case, I-UPF will put the transport level packet marking value of the incoming N9 downlink packet into the outgoing N3 packets.

2. Actions:

To CT4:

ACTION: 	SA2 kindly asks CT4 to take the above into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #169		19-23 May 2025	Fukuoka, Japan

TSG-SA2 Meeting #170		25-29 August 2025	Goteborg, Sweden