SA WG2 Meeting #168	S2-2503960

07 - 11 April, 2025, Goteborg	(Revision of S2-2502992r2)

Source:	SK Telecom, China Mobile, China Telecom, Deutsche Telekom, KPN N.V., LG Uplus, Verizon, Rakuten Mobile

Title:	WT#2 use case : AI/ML-assisted user plane traffic pattern and behaviour analysis to support efficient performance of User Plane

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This paper proposes a use case to cover Work Task#2 of the FS_AIML_CN_Ph2 SID.

1	Discussion

This paper discusses a use case for FS_AIML_CN_Ph2 (SP-250413) Work Task #2 (WT#2), as described below.

Whether and how to enhance the 5GC analytics to support efficient performance user plane. Note 4: This may be based on the analysis of the user plane traffic (e.g. traffic patterns, packet information) and the reaction of the network per the analysis is based on the existing policy framework. Note 5: The study will not address how UPF detects wrong or malformed or duplicated packets. This WT shall have neither RAN or UE impacts. Solutions shall minimize the load on Control Plane NFs due to too much reporting from UPF.

A related use case for the WT#2 is provided in accordance with the SA2 guidance (SP-250418).

2	Proposal

It is proposed to include the following changes in TR 23.700-04 V.0.0.

* * * * Start of Changes – All New Text * * * *

5.X	Use cases #x: AI/ML-assisted user plane traffic pattern and behaviour analysis to support efficient performance of User Plane

The UPF is responsible for managing substantial volumes of uplink and downlink application traffic originating from UEs or application servers (ASs). It must be robust and resilient against abnormal data traffic from UEs/ASs, for examples, large amount of abnormal or malicious IP packets, flows, bursts, syn-ack signaling. Without actions to mitigate the impact of such potential abnormal or malicious traffic pattern in the user plane, the UPF may experience performance degradation which can lead to resource inefficiency and an unreliable 5G network. Use of AI/ML can be studied to provide analytics on user plane traffic patterns and UE or Application communication behaviours and how to use the Analytics to apply mitigation strategies with the goal to improve UPF performance and/or to avoid malicious user plane traffic.

* * * * End of Changes * * * *