3GPP TSG-WG SA2 Meeting #168	S2-2504284

Goteborg, SE, 7th April – 11th April, 2025	(revision of S2-2503834, 4064)

Source:	Huawei, HiSilicon

Title:	pCR for TS 23.369: Update Inventory Procedure

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: Clause 6.2.2 is updated to add more details about inventory procedure.

# 1. Introduction/Discussion

This contribution updates the Inventory procedure to add more details of parameters and clarify how to use the parameters accordingly. In addition, there are several Editor’s Notes in the clauses which are to be addressed by this contribution.

Editor's note:	The parameters for the inventory service operation need further definition.

For this Editor’s note, further clarification on the parameters of target area information, AIoT Device ID identification information and the approximate number of AIoT Devices are added.

The target area information is used for reader selection, so the procedure is updated to refer to that procedure instead of duplicating the information.

AIoT Device ID identification information may include partial AIoT device identifier (e.g., the value(s) of one or more fields of the AIoT device identifier) or include one AIoT device identifier.

The approximate number of AIoT Devices, if provided by the AF, is used by AIOTF to estimate the potential number of AIoT Devices responding to this inventory service operation, and is sent by AIOTF to the AIoT RAN in the assistance information for NG-RAN in step 7 for proper radio resource allocation.

Editor's note:	How the NEF selects the AIOTF needs further definition.

It is clarified that the NEF will use the target area information and/or AIoT Device ID identification information (if provided) for AIOTF selection. This is referred to the AIOTF selection dedicated clause to avoid duplication.

Editor's note:	The authorisation split between the NEF and AIOTF needs alignment with the other clauses in this TS.

Further clarifications are added in step 2 and step 4 to align with clause 5.6.

Editor's note:	AIoT Device Identification information needs further definition.

The clarification on AIoT Device Identification information (information about the target AIoT Device(s) is added in step 1. After received by the AIOTF, the AIoT Device Identification information will be further authorized and transform to AIoT Device Identification MASK which will be included in the paging message broadcast by the NG-RAN. Since the two information may or may not be same, in order to differentiate them, new terminology name is given to the one sent by AIOTF to NG-RAN which is AIoT Device Identification MASK. AIoT Device will determine whether its Device ID matches this AIoT Device Identification MASK in order to determine whether to respond to the paging message.

Editor's note:	Whether the Inventory Request sent to AIoT RAN includes indication about whether there will be a follow up command or not needs to be determined.

Since this is the procedure for inventory only, there will be no command in the following steps. Therefore, the Editor’s Note can be deleted and the further clarification on the command indication can be added in the command procedure accordingly.

Editor's note:	Whether only one, or more than one report is generated by the AIOTF to send to the AF needs clarification.

Whether AIOTF generates one or more reports may depend on the aggregation information sent from AF (if provided). Therefore, the clarification is added. For the messages sent by NEF to AF, this depends on the message(s) received by NEF from AIOTF. In other words, as long as the NEF receives the message from AIOTF, it will send the notify to AF accordingly.

Apart from the text related to solving the Editor’s notes, two additional steps are added regarding how to stop the service operation and release the context related to the service operation. If NG-RAN detects that there is no more AIoT Devices performing RAN inventory procedure, NG-RAN can send Inventory Complete message to the AIOTF including the corresponding Correlation ID indicating that the Inventory service operation associated with the Correlation ID is completed. If the AIOTF considers that the service operation is completed, it shall send Service End message including the Correlation ID to the NG-RAN, indicating that the Inventory service operation associated with the Correlation ID is completed. When the Inventory service operation associated with the Correlation ID is completed, both the AIOTF and the NG-RAN release the context associated with the Correlation ID and it can no longer be used.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change * * * *

### 6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.X.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory(AF ID, [External Target Area information], information about the target AIoT Device(s), [Approximate number of AIoT Devices]) service operation request to the NEF.

The External Target Area information, is specified in clause 5.3.

Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.x, or include complete AIoT Device Identifier(s).

The approximate number of AIoT Devices, if provided, is used to determine the number of AIoT Devices expected to respond to this inventory service operation, which is sent by AIOTF to the NG-RAN in the assistance information for NG-RAN in step 7 for proper radio resource allocation.

Editor's note:	The parameters for the inventory service operation need further definition.

2.

The NEF may further authorize the AF request as specified in clause 5.6.

The NEF determines the Target Area information, and selects one or multiple AIOTF(s) to handle the request as specified in clause 5.X.

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, [Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices]) service operation towards to the selected AIOTF.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. The AIOTF may perform authorization as specified in clause 5.6. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a Correlation ID corresponding to this AF service operation request.

The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.X, or a single AIoT Device Identifier.

AIOTF performs Reader Selection, see clause 5.3.

The AIOTF may also use the last serving Reader to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device.

The AIOTF determines assistance information as described in clause 5.4.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT Inventory service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message including the Correlation ID, the AIoT Identification Information to be included in the paging message, and assistance information to the selected AIoT RAN.

Editor's note:	Whether the Inventory Request sent to AIoT RAN includes indication about whether there will be a follow up command or not needs to be determined.

8.	The AIoT RAN sends an Inventory Response to the AIOTF with the Correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

9.	Upon reception of the Inventory Request message from the AIOTF, the AIoT Reader(s) will execute the inventory operation. The AIoT Reader(s) broadcast the paging message that includes the AIoT Identification Information.

The AIoT Device determines whether it matches the AIoT Identification Information, as described in clause 5.x.

If an AIoT device matches the AIoT Identification Information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT Device ID.

Editor's note:	Whether and how the Device ID is concealed or encrypted will be determined and aligned with SA WG3.

10.	AIoT RAN sends one or more Inventory Report messages to the AIOTF including the Correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s). The AIOTF stores the mapping between the Reader ID and AIoT Device ID(s).

NOTE X: When to erase the stored mapping between the Reader Index and AIoT device ID(s) is up to implementation and local configuration.

11.	The AIOTF validates the results, using local stored device information or device profile data retrieved from the ADM. The AIOTF may aggregate the results.

12.	Optionally, if the NG-RAN detects that no more AIoT Devices will respond to the inventory procedure, the NG-RAN informs the AIOTF that no further Inventory Report messages will be sent and the procedure is completed.

Editor’s Note: The details how interactions between NG-RAN and AIOTF to indicate no further Inventory Reports will be sent and completion of the procedure need to be aligned with RAN.

13.	The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the Naiotf_AIoT_Notify message including a list of AIoT Device Permanent Identifier(s). The AIOTF may send multiple reports. The AIOTF in the final Naiotf_AIoT_Notify message indicates that the procedure has completed.

14.	When receiving the Naiotf_AIoT_Notify message from AIOTF, the NEF informs the AF of the outcome of the AIoT_Inventory request by sending the Nnef_AIoT_Notify message including the AIoT Device Permanent Identifier(s). The NEF in the final Nnef_AIoT_Notify message indicates that the procedure has completed.

# * * * * End of changes * * * *