3GPP TSG-WG2 Meeting #168	S2-2504359

# Stor-Göteborg, Sweden, 7th - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6275 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment with ATSSS Capability IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CT1 approved C1-251196 that updates the ATSSS Capability IE and introduces elementary capabilities for steering functionalities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align UE capability indication for ATSSS with TS 24.501. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 23.501 is misaligned with TS 24.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.2, 5.32.5.1, 5.32.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 5.32.2	Multi Access PDU Sessions

A Multi-Access PDU (MA PDU) Session is managed by using the session management functionality specified in clause 5.6, with the following additions and modifications:

-	When the UE wants to request a new MA PDU Session:

-	If the UE is registered to the same PLMN over 3GPP and non-3GPP accesses, then the UE shall send a PDU Session Establishment Request over any of the two accesses. The UE also provides Request Type as "MA PDU Request" in the UL NAS Transport message. The AMF informs the SMF that the UE is registered over both accesses and this triggers the establishment of user-plane resources on both accesses and two N3/N9 tunnels between PSA and the RAN/AN.

-	If the UE is registered to different PLMNs over 3GPP and non-3GPP accesses, then the UE shall send a PDU Session Establishment Request over one access. The UE also provides Request Type as "MA PDU Request" in the UL NAS Transport message. After this PDU Session is established with one N3/N9 tunnel between the PSA and (R)AN established, the UE shall send another PDU Session Establishment Request over the other access. The UE also provides the same PDU Session ID and Request Type as "MA PDU Request" in the UL NAS Transport message. Two N3/N9 tunnels and User-plane resources on both accesses are established.

-	If the UE is registered over one access only, then the UE shall send a PDU Session Establishment Request over this access. The UE also provides Request Type as "MA PDU Request" in the UL NAS Transport message. One N3/N9 tunnel between the PSA and (R)AN and User-plane resources on this access only are established. After the UE is registered over the second access, the UE shall establish user-plane resources on the second access.

-	In the PDU Session Establishment Request that is sent to request a new MA PDU Session, the UE shall provide also its ATSSS capabilities, which indicate the steering functionalities and the steering modes supported in the UE. These functionalities are defined in clause 5.32.6.

-	If the UE indicates it is capable of supporting:

-	the "ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported" capability (as specified in clause 5.32.6.1);

and the network accepts to activate this functionality, then the network may provide to UE Measurement Assistance Information (see details in clause 5.32.5) and shall provide to UE one or more ATSSS rules.

NOTE 1:	As specified in Table 5.32.8-1 and in Table 5.8.5.8-1, the ATSSS-LL functionality cannot be used together with the Redundant steering mode. When the UE indicates it is capable of supporting the ATSSS-LL functionality with any steering mode, it is implied that the UE can support the ATSSS-LL functionality with any steering mode except the Redundant steering mode.

-	If the UE indicates it is capable of supporting:

-	the "MPTCP functionality with any steering mode supported" capability and the "ATSSS-LL functionality with only the Active-Standby steering mode supported" capability (as specified in clause 5.32.6.1);

and the network accepts to activate these functionalities, then the network provides MPTCP proxy information to UE and allocates to UE (a) one IP address/prefix for the MA PDU session (as defined in clause 5.8.2.2) and (b) two additional IP addresses/prefixes, called "MPTCP link-specific multipath" addresses. Further details are provided in clause 5.32.6.2.1. In addition, the network may provide to UE Measurement Assistance Information and shall provide to UE one or more ATSSS rules. The network shall provide to UE ATSSS rule(s) for non-MPTCP traffic. The ATSSS rule(s) for non-MPTCP traffic shall use the ATSSS-LL, MPQUIC-UDP and/or MPQUIC-IP steering functionality(ies) depending on what is supported and selected for the MA PDU Session.

If the network accepts to activate only ATSSS-LL functionality with only the Active-Standby steering mode, the network shall provide to the UE an ATSSS rule that uses the ATSSS-LL functionality and the Active-Standby Steering Mode, to indicate how the traffic shall be transferred across the 3GPP access and the non-3GPP access in the uplink direction.

-	If the UE indicates it is capable of supporting

-	the "MPQUIC-UDP functionality with any steering mode supported" capability and the "ATSSS-LL functionality with only the Active-Standby steering mode supported" capability (as specified in clause 5.32.6.1);

and the network accepts to activate these functionalities, then the network provides MPQUIC-UDP proxy information to UE and allocates to UE (a) one IP address/prefix for the MA PDU session (as defined in clause 5.8.2.2) and (b) two additional IP addresses/prefixes, called "MPQUIC link-specific multipath" addresses. Further details are provided in clause 5.32.6.2.2. In addition, the network may provide to UE Measurement Assistance Information and shall provide to UE one or more ATSSS rules. The network shall provide to UE an ATSSS rule(s) for non-MPQUIC-UDP traffic. The ATSSS rule(s) for non-MPQUIC-UDP traffic shall use the ATSSS-LL, MPTCP and/or MPQUIC-IP steering functionality(ies) depending on what is supported and selected for the MA PDU Session.

If the network accepts to activate only ATSSS-LL functionality with only the Active-Standby steering mode, the network shall provide to the UE an ATSSS rule that uses the ATSSS-LL functionality and the Active-Standby Steering Mode, to indicate how the traffic shall be transferred across the 3GPP access and the non-3GPP access in the uplink direction.

-	If the UE indicates it is capable of supporting:

-	the "MPQUIC-IP functionality with any steering mode supported" capability and "ATSSS-LL functionality with only Active-Standby steering mode supported" capability (as specified in clause 5.32.6.1);

and the network accepts to activate these functionalities, then the network provides MPQUIC proxy information to UE and allocates to UE (a) one IP address/prefix for the MA PDU session (as defined in clause 5.8.2.2) and (b) two additional IP addresses/prefixes, called "MPQUIC link-specific multipath" addresses. Further details are provided in clause 5.32.6.2.2. In addition, the network may provide to UE Measurement Assistance Information and shall provide to UE one or more ATSSS rules.

If the network accepts to activate only ATSSS-LL functionality with only the Active-Standby steering mode, the network shall provide to the UE an ATSSS rule that uses the ATSSS-LL functionality and the Active-Standby Steering Mode, to indicate how the traffic shall be transferred across the 3GPP access and the non-3GPP access in the uplink direction.

-	If the UE indicates it is capable of supporting:

-	the "MPQUIC-E functionality with any steering mode supported" capability and "ATSSS-LL functionality with only Active-Standby steering mode supported" capability (as specified in clause 5.32.6.1);

and the network accepts to activate MPQUIC-E functionality, then the network provides MPQUIC proxy information to UE and allocates to UE two IP addresses/prefixes, called "MPQUIC link-specific multipath" addresses. Further details are provided in clause 5.32.6.2.2. In addition, the network shall provide to UE one or more ATSSS rules.

If the network accepts to activate only ATSSS-LL functionality with only the Active-Standby steering mode, the network shall provide to the UE an ATSSS rule that uses the ATSSS-LL functionality and the Active-Standby Steering Mode, to indicate how the traffic shall be transferred across the 3GPP access and the non-3GPP access in the uplink direction. In addition, the network may provide to UE Measurement Assistance Information.

-	If the UE indicates it is capable of supporting:

-	the "MPQUIC-IP functionality with any steering mode supported" capability (as specified in clause 5.32.6.1);

NOTE x:	This capability does not require the UE to support the "ATSSS-LL functionality with only Active-Standby steering mode supported" capability.

and the network accepts to activate MPQUIC-IP functionality, then the network provides MPQUIC-IP proxy information to UE and allocates to UE (a) one IP address/prefix for the MA PDU session (as defined in clause 5.8.2.2) and (b) two additional IP addresses/prefixes, called "MPQUIC link-specific multipath" addresses. In addition, the network shall provide to UE one or more ATSSS rules.

NOTE 2:	The "MPTCP link-specific multipath" addresses and the "MPQUIC link-specific multipath" addresses can be the same.

-	If the UE requests an S-NSSAI, this S-NSSAI should be allowed on both accesses. Otherwise, the MA PDU Session shall not be established.

-	The SMF determines the ATSSS capabilities supported for the MA PDU Session taking into consideration one or more of the following aspects based on the ATSSS capabilities provided by the UE and per DNN configuration on SMF:

a)	If the UE includes in its ATSSS capabilities the "MPTCP functionality with any steering mode supported" capability and the "ATSSS-LL functionality with only Active-Standby steering mode supported" capability (as specified in clause 5.32.6.1), then:

i)	If the DNN configuration allows MPTCP and ATSSS-LL with any steering mode (i.e. any Steering Mode allowed for ATSSS-LL), including RTT measurement without using PMF protocol, the MA PDU Session is capable of (1) MPTCP and ATSSS-LL with any steering mode (i.e. any Steering Mode allowed for ATSSS-LL) in the downlink and (2) MPTCP and ATSSS-LL with Active-Standby mode in the uplink.

NOTE 3:	In this case, it is assumed that ATSSS-LL with "Smallest Delay" steering mode is selected for the downlink only when the UPF can measure RTT without using the PMF protocol, e.g. by using other means not defined by 3GPP such as using the RTT measurements of MPTCP.

ii)	If the DNN configuration allows MPTCP and ATSSS-LL with any steering mode (i.e. any Steering Mode allowed for ATSSS-LL), but not RTT measurement without using PMF protocol, the MA PDU Session is capable of (1) MPTCP in the downlink (2) ATSSS-LL with any steering mode except Smallest Delay steering mode (i.e. any Steering Mode allowed for ATSSS-LL except Smallest Delay steering mode) in the downlink and (3) MPTCP and ATSSS-LL with Active-Standby mode in the uplink.

iii)	If the DNN configuration allows MPTCP with any steering mode and ATSSS-LL with only Active-Standby steering mode, the MA PDU Session is capable of MPTCP and ATSSS-LL with Active-Standby mode in the uplink and in the downlink.

iv)	If the DNN configuration does not allow MPTCP steering functionality and the DNN configuration allows ATSSS-LL functionality with at least the Active-Standby steering mode, then the MA PDU Session is capable of ATSSS-LL with only Active-Standby steering mode in the uplink and in the downlink.

b)	If the UE includes in its ATSSS capabilities the "MPQUIC-UDP functionality with any steering mode supported" capability and the "ATSSS-LL functionality with only Active-Standby steering mode supported" capability (as specified in clause 5.32.6.1), then:

i)	If the DNN configuration allows MPQUIC-UDP and ATSSS-LL with any steering mode (i.e. any Steering Mode allowed for ATSSS-LL), including RTT measurement without using PMF protocol, the MA PDU Session is capable of (1) MPQUIC-UDP and ATSSS-LL with any steering mode (i.e. any Steering Mode allowed for ATSSS-LL) in the downlink and (2) MPQUIC-UDP and ATSSS-LL with Active-Standby mode in the uplink.

NOTE 4:	In this case, it is assumed that ATSSS-LL with "Smallest Delay" steering mode is selected for the downlink only when the UPF can measure RTT without using the PMF protocol, e.g. by using other means not defined by 3GPP such as using the RTT measurements of MPQUIC.

ii)	If the DNN configuration allows MPQUIC-UDP and ATSSS-LL with any steering mode (i.e. any Steering Mode allowed for ATSSS-LL), but not RTT measurement without using PMF protocol, the MA PDU Session is capable of (1) MPQUIC-UDP in the downlink (2) ATSSS-LL with any steering mode except Smallest Delay steering mode (i.e. any Steering Mode allowed for ATSSS-LL except Smallest Delay steering mode) in the downlink and (3) MPQUIC-UDP and ATSSS-LL with Active-Standby mode in the uplink.

iii)	If the DNN configuration allows MPQUIC-UDP with any steering mode and ATSSS-LL with only Active-Standby steering mode, the MA PDU Session is capable of MPQUIC-UDP and ATSSS-LL with Active-Standby mode in the uplink and in the downlink.

iv)	If the DNN configuration does not allow MPQUIC-UDP steering functionality and the DNN configuration allows ATSSS-LL functionality with at least the Active-Standby steering mode, then the MA PDU Session is capable of ATSSS-LL with only Active-Standby steering mode in the uplink and in the downlink.

c)	If the UE includes in its ATSSS capabilities the "ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported" capability (as specified in clause 5.32.6.1) and the DNN configuration allows ATSSS-LL with any steering mode allowed for ATSSS-LL, the MA PDU Session is capable of ATSSS-LL with any steering mode allowed for ATSSS-LL in the uplink and in the downlink.

d)	If the UE includes in its ATSSS capabilities the "MPQUIC IP functionality with any steering mode supported" capability and the "ATSSS-LL functionality with only Active-Standby steering mode supported" capability (as specified in clause 5.32.6.1) and the DNN configuration allows MPQUIC IP functionality with any steering mode, the MA PDU Session is capable of MPQUIC IP with any steering mode in the uplink and in the downlink. If the DNN configuration does not allow MPQUIC-IP steering functionality and the DNN configuration allows ATSSS-LL functionality with at least the Active-Standby steering mode, then the MA PDU Session is capable of ATSSS-LL with only Active-Standby steering mode in the uplink and in the downlink.

e)	If the UE includes in its ATSSS capabilities the "MPQUIC-E functionality with any steering mode supported" capability and the "ATSSS-LL functionality with only Active-Standby steering mode supported" capability (as specified in clause 5.32.6.1) and the DNN configuration allows MPQUIC-E functionality with any steering mode, the MA PDU Session is capable of MPQUIC-E functionality with any steering mode in the uplink and in the downlink. If the DNN configuration does not allow MPQUIC-E steering functionality and the DNN configuration allows ATSSS-LL functionality with at least the Active-Standby steering mode, then the MA PDU Session is capable of ATSSS-LL with only Active-Standby steering mode in the uplink and in the downlink.

f)	If the UE includes in its ATSSS capabilities the "MPQUIC-IP functionality with any steering mode supported" capability (as specified in clause 5.32.6.1) and the DNN configuration allows MPQUIC-IP functionality with any steering mode, the MA PDU Session is capable of MPQUIC-IP functionality with any steering mode in the uplink and in the downlink. If the DNN configuration does not allow MPQUIC-IP steering functionality, then the SMF rejects the MA PDU Session request.

NOTE 5:	As described above, an example of how SMF determines the ATSSS capabilities of a MA PDU Session is as follows:

-	If the UE indicates that it supports the "MPTCP functionality with any steering mode supported" capability, the "ATSSS-LL functionality with only Active-Standby steering mode supported" capability, the "MPQUIC-UDP functionality with any steering mode supported" capability and the "MPQUIC-IP functionality with any steering mode supported" capability and the DNN configuration allows only MPTCP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode, then the MA PDU Session is capable of MPTCP and ATSSS-LL with Active-Standby mode in the uplink and in the downlink.

NOTE 6:	MPQUIC-E functionality is enabled only when the type of the MA PDU Session is Ethernet. The MPTCP, MPQUIC-UDP and MPQUIC-IP functionalities are not enabled when the type of the MA PDU Session is Ethernet.

For ATSSS-LL with only Active-Standby steering mode, the UE and the UPF needs to support Access Availability/Unavailability Report as specified in clause 5.32.5.3.

The SMF provides the ATSSS capabilities of the MA PDU Session to the PCF during PDU Session Establishment.

-	The PCC rules provided by PCF include MA PDU Session Control information (see TS 23.503 [45]). They are used by SMF to derive ATSSS rules for the UE and N4 rules for the UPF. When dynamic PCC is not used for the MA PDU Session, the SMF shall provide ATSSS rules and N4 rules based on local configuration (e.g. based on DNN or S-NSSAI).

-	The UE receives ATSSS rules from SMF, which indicate how the uplink traffic should be routed across 3GPP access and non-3GPP access. Similarly, the UPF receives N4 rules from SMF, which indicate how the downlink traffic should be routed across 3GPP access and non-3GPP access.

-	When the SMF receives a PDU Session Establishment Request and a "MA PDU Request" indication and determines that UP security protection (see clause 5.10.3) is required for the PDU Session, the SMF shall only confirm the establishment of the MA PDU session if the 3GPP access network can enforce the required UP security protection. The SMF needs not confirm whether the non-3GPP access can enforce the required UP security protection.

-	The UE indicates during MA PDU Session Establishment to the AMF whether it supports non-3GPP access path switching, i.e. whether the UE can transfer the non-3GPP access path of the MA PDU Session from a source non-3GPP access (N3IWF/TNGF) to a target non-3GPP access (a different N3IWF/TNGF). If the UE has indicated support for non-3GPP access path switching and the AMF supports non-3GPP access path switching, the AMF selects an SMF that supports non-3GPP access path switching, if such an SMF is available. If the AMF supports to maintain two N2 connections for non-3GPP access during the Registration procedure and the selected SMF supports non-3GPP path switching, the AMF indicates whether the UE supports non-3GPP path switching to the SMF. The SMF indicates support for non-3GPP path switching to the UE in the PDU Session Establishment Accept message.

NOTE 7:	If the AMF selects an SMF not supporting non-3GPP access path switching, the non-3GPP access path switching can still be performed with the AMF triggering release of the old user plane resources before new user plane resources are established.

-	After the MA PDU Session establishment:

-	At any given time, the MA PDU session may have user-plane resources on both 3GPP and non-3GPP accesses, or on one access only, or may have no user-plane resources on any access.

-	The AMF, SMF, PCF and UPF maintain their MA PDU Session contexts, even when the UE deregisters from one access (but remains registered on the other access).

-	When the UE deregisters from one access (but remains registered on the other access), the AMF informs the SMF to release the resource of this access type in the UPF for the MA PDU Session. Subsequently, the SMF notifies the UPF that the access type has become unavailable and the N3/N9 tunnel for the access type are released.

-	If the UE wants to add user-plane resources on one access of the MA PDU Session, e.g. based on access network performance measurement and/or ATSSS rules, then the UE shall send a PDU Session Establishment Request over this access containing PDU Session ID of the MA PDU Session. The UE also provides Request Type as "MA PDU Request" and the same PDU Session ID in the UL NAS Transport message. If there is no N3/N9 tunnel for this access, the N3/N9 tunnel for this access is established.

-	If the UE wants to re-activate user-plane resources on one access of the MA PDU Session, e.g. based on access network performance measurement and/or ATSSS rules, then the UE shall initiate the UE Triggered Service Request procedure over this access.

-	If the network wants to re-activate the user-plane resources over 3GPP access or non-3GPP access of the MA PDU Session, the network shall initiate the Network Triggered Service Request procedure, as specified in clause 4.22.7 of TS 23.502 [3].

-	If the UE wants to move the non-3GPP user-plane resources of the MA PDU Session from a source non-3GPP access (e.g. source N3IWF or TNGF) to a target non-3GPP access (e.g. target N3IWF or TNGF), the UE initiates a Mobility Registration Update via the target non-3GPP access as described in TS 23.502 [3], clause 4.22.9.5. This procedure may also be used to move the non-3GPP user-plane resources of single access PDU Session(s).

NOTE 8:	The UE can request activation of single access PDU Session(s) over the target non-3GPP access while performing Mobility Registration Update procedure according to the existing procedure.

-	The SMF may add, remove or update one or more individual ATSSS rules of the UE by sending new or updated ATSSS rules with the corresponding Rule IDs to the UE.

A MA PDU Session may be established either:

a)	when it is explicitly requested by an ATSSS-capable UE; or

b)	when an ATSSS-capable UE requests a single-access PDU Session but the network decides to establish a MA PDU Session instead. This is an optional scenario specified in clause 4.22.3 of TS 23.502 [3], which may occur when the UE requests a single-access PDU Session but no policy (e.g. no URSP rule) and no local restrictions in the UE mandate a single access for the PDU Session.

A MA PDU Session may be established during a PDU Session modification procedure when the UE moves from EPS to 5GS, as specified in clause 4.22.6.3 of TS 23.502 [3].

The AMF indicates as part of the Registration procedure whether ATSSS is supported or not. When ATSSS is not supported, the UE shall not

-	request establishment of a MA PDU Session (as described in clause 4.22.2 of TS 23.502 [3]); or

-	request addition of User Plane resources for an existing MA PDU Session (as described in clause 4.22.7 of TS 23.502 [3]); or

-	request establishment of a PDU Session with "MA PDU Network-Upgrade Allowed" indication (as described in clause 4.22.3 of TS 23.502 [3]); or

-	request PDU Session Modification with Request Type of "MA PDU request" or with "MA PDU Network-Upgrade Allowed" indication after moving from EPC to 5GC (as described in clause 4.22.6.3 of TS 23.502 [3]).

The AMF indicates as part of the Registration procedure whether it supports non-3GPP access path switching. When the AMF does not indicate support of non-3GPP access path switching, the UE shall not perform the Mobility Registration Update procedure for non-3GPP access path switching, i.e. to switch traffic from a source non-3GPP access to a target non-3GPP access. The SMF indicates as part of the PDU Session Establishment procedure whether it supports non-3GPP access path switching. If the UE has one or more PDU sessions and at least one serving SMF for the PDU Sessions supports non-3GPP access path switching, the UE may include ("Non-3GPP path switching while using old AN resources") indication when the UE performs the Mobility Registration Update procedure for non-3GPP access path switching. If the UE is registered to different PLMNs over 3GPP and non-3GPP accesses, the UE shall use the capability received over non-3GPP access to determine whether to perform the Mobility Registration Update procedure for non-3GPP path switching and whether to include ("Non-3GPP access path switching while using old AN resources") indication.

NOTE 9:	If the AMF receives ("Non-3GPP path switching while using old AN resources") indication from Mobility Registration Update procedure and the serving SMF(s) for PDU Session(s) is not supporting non-3GPP access path switching, the non-3GPP access path switching can still be performed with the AMF triggering for each PDU Session the release of the old user plane resources before new user plane resources are established.

An ATSSS-capable UE may decide to request a MA PDU Session based on the provisioned URSP rules. In particular, the UE should request a MA PDU Session when the UE applies a URSP rule, which triggers the UE to establish a new PDU Session and the Access Type Preference component of the URSP rule indicates "Multi-Access" (see TS 23.503 [45]).

**** Next Change ****

#### 5.32.5.1	General principles

When an MA PDU Session is established, the network may provide the UE with Measurement Assistance Information. This information assists the UE in determining which measurements shall be performed over both accesses, as well as whether measurement reports need to be sent to the network.

Measurement Assistance Information shall include the addressing information of a Performance Measurement Function (PMF) in the UPF, the UE can send PMF protocol messages to:

-	For a PDU Session of IP type, Measurement Assistance Information contains one IP address for the PMF, one UDP port associated with 3GPP access and another UDP port associated with non-3GPP access. PMF messages sent by UE to one of these UDP ports, shall be transmitted to UPF via the QoS Flow associated with the default QoS rule.

-	For a PDU Session of Ethernet type, Measurement Assistance Information contains one MAC address associated with 3GPP access and another MAC address associated with non-3GPP access. PMF messages sent by UE to one of these MAC addresses, shall be transmitted to UPF via the QoS Flow associated with the default QoS rule.

NOTE 1:	To protect the PMF in the UPF (e.g. to block DDOS to the PMF), the IP addresses of the PMF are only accessible from the UE IP address via the N3/N9 interface.

NOTE 2:	After the MA PDU Session is released, the same UE IP address/prefix is not allocated to another UE for MA PDU Session in a short time.

If the SMF determines that access performance measurements per QoS Flow shall be applied for the MA PDU Session, then the Measurement Assistance Information shall also include a list of QoS Flows on which access performance measurements may be performed. For each QoS Flow in this list, the following information is included:

-	The QFI of the associated QoS Flow.

-	For a PDU Session of IP type, one UDP port associated with 3GPP access and another UDP port associated with non-3GPP access. PMF messages sent by UE to one of these UDP ports, shall be transmitted to UPF via the associated QoS Flow.

-	For a PDU Session of Ethernet type, one MAC address associated with 3GPP access and another MAC address associated with non-3GPP access. PMF messages sent by UE to one of these MAC addresses, shall be transmitted to UPF via the associated QoS Flow.

The QoS rules and the N4 rules provided by SMF to UE and to UPF respectively shall include information (e.g. packet filters containing the UDP port or the MAC address associated with a QoS Flow), which enables the UE and UPF to route a PMF message to a specific QoS Flow.

The UE and the UPF may need to perform access performance measurements in order to estimate the Round-Trip Time (RTT) and/or the Packet Loss Rate (PLR) that an SDF is expected to experience when transmitted on a certain access type. Based on these measurements and the provisioned ATSSS rules in the UE and MAR rules in the UPF, the UE and the UPF decide how to distribute the traffic of an SDF across the two accesses.

If the UE and the UPF decide to initiate access performance measurements to estimate the RTT and/or the PLR for an SDF, the access performance measurements shall be performed either

(a)	using the QoS Flow associated with the default QoS rule; or

(b)	using the target QoS Flow, which is the QoS Flow that the SDF traffic is transmitted on.

When the access performance measurements are using the target QoS Flow, it is termed that "access performance measurements per QoS Flow" are applied for the MA PDU Session.

The UE shall indicate in its ATSSS capabilities that it supports access performance measurements per QoS Flow. Based on this UE capability and other information (such as local policy), the SMF determines whether access performance measurements per QoS Flow shall be applied for the MA PDU Session or not. If the SMF determines that access performance measurements per QoS Flow shall be applied for the MA PDU Session, then:

-	The SMF determines a list of QoS Flows over which access performance measurements may be performed and provides this list to the UE (within the Measurement Assistance Information) and to the UPF.

-	The UE and the UPF may initiate access performance measurements on one or more of the QoS Flows included in this list. The UE and the UPF shall be able to receive and respond to PMF messages sent on any QoS Flow included in this list.

-	The SMF may update the list of QoS Flows over which access performance measurements may be performed during the lifetime of a MA PDU Session, e.g. when a new PCC rule that could benefit from PMF access performance measurements is bound to a QoS Flow.

NOTE 3:	The SMF can e.g. add a QoS Flow into the list when at least one PCC Rule is bound to that QoS Flow that is using one of the Steering Modes where performance measurements via PMF are applicable, such as Lowest Delay Steering Mode or a Steering Mode where threshold values have been provided.

The UE shall perform access performance measurements per QoS Flow only when this is explicitly indicated in the Measurement Assistance Information, i.e. only when the UE receives the list of QoS Flows over which access performance measurements may be performed. Otherwise, the UE shall perform access performance measurements based on the QoS Flow associated with the default QoS rule. The UPF shall perform access performance measurements per QoS Flow only when this is explicitly indicated by SMF, i.e. only when the UPF receives the list of QoS Flows over which access performance measurements may be performed. Otherwise, the UPF shall perform access performance measurements based on the QoS Flow associated with the default QoS rule. In this case the UPF learns what QoS Flow to use as described in TS 29.244 [65].

The UE and the UPF may decide not to initiate access performance measurements using PMF over a certain target QoS Flow, when they already have access performance measurements for another target QoS Flow which they determine can be reused.

NOTE 4:	How the UE and UPF determine that the performance measurements using a certain target QoS Flow apply to another target QoS Flow is based on implementation, e.g. AN resource to QoS Flow mapping in the UE or getting similar access measurements results with other QoS Flow.

When access performance measurements for an SDF are performed based on the target QoS Flow, the UE needs to be able to determine the QoS Flow a downlink packet arrives on. In order to enable this, the SMF shall include downlink Packet Filter information in the QoS rule provided to UE matching this SDF, unless Reflective QoS is used for the SDF.

NOTE 5:	For example, if a QoS Flow requires to activate Reflective QoS, the SMF does not need to provide downlink QoS Flow information for the QoS Flow to minimize usage of packet filters. When a data packet is received over a QoS Flow, the UE can decide whether to check the downlink QoS Flow information based on the existence of SDAP header for the QoS Flow.

The addressing information of the PMF in the UPF is retrieved by the SMF from the UPF during N4 session establishment. If the UPF receives from the SMF, during N4 session establishment or modification procedure, a list of QoS Flows over which access performance measurements may be performed, the UPF allocates different UDP ports per QoS Flow per access for IP PDU sessions, or allocates different MAC addresses per QoS Flow per access for Ethernet PDU sessions. For IP PDU sessions, the UPF sends the PMF IP addressing information and the UDP ports with the QFI of the associated QoS Flow to the SMF. For Ethernet PDU sessions, the UPF sends the MAC addresses with the QFI of the associated QoS Flow to the SMF.

The following PMF protocol messages can be exchanged between the UE and the UPF:

-	Messages to allow for Round Trip Time (RTT) measurements, i.e. when the "Smallest Delay" steering mode is used or when either "Priority-based", "Load-Balancing" or "Redundant" steering mode is used with RTT threshold value being applied;

-	Messages to allow for Packet Loss Rate (PLR) measurements, i.e. when steering mode is used either "Priority-based", "Load-Balancing" or "Redundant" steering mode is used with PLR threshold value being applied;

-	Messages for reporting Access availability/unavailability by the UE to the UPF.

-	Messages for sending UE-assistance data to UPF. Such messages may be sent from the UE to UPF only when the UE receives the UE-assistance indicator in an ATSSS rule, as specified in clause 5.32.8. Further details are provided in clause 5.32.5.5.

-	Messages for sending Suspend Traffic Duplication and Resume Traffic Duplication from UPF to UE to suspend or resume traffic duplication as defined in clause 5.32.5.6.

Since steering modes can be different in up-link and down-link, the UE needs to be able to handle PMF protocol messages for RTT and PLR measurements received from UPF even if it is not using one of the steering modes associated with the RTT and PLR measurements (and vice versa).

The PMF protocol is specified in TS 24.193 [109].

The PMF protocol messages used for access availability/unavailability reports shall be sent on the QoS Flow associated with the default QoS rule. The PMF protocol messages used for access performance measurements shall be sent either on the QoS Flow associated with the default QoS rule, or on the target QoS Flow, as specified above.

The QoS Flow associated with the default QoS rule for MA PDU Session is Non-GBR QoS Flow.

The UE shall not apply the ATSSS rules and the UPF shall not apply the MAR rules for the PMF protocol messages.

When the UE requests a MA PDU session and indicates it is capable to support one or more of the steering functionalities below:

-	the "MPTCP functionality with any steering mode supported" capability and the "ATSSS-LL functionality with only the Active-Standby steering mode supported" capability (as specified in clause 5.32.6.1);

-	the "MPQUIC-UDP functionality with any steering mode supported" capability and the "ATSSS-LL functionality with only the Active-Standby steering mode supported" capability (as specified in clause 5.32.6.1);

-	the "MPQUIC-IP functionality with any steering mode supported" capability and the "ATSSS-LL functionality with only the Active-Standby steering mode supported" capability;

-	the "MPQUIC-E functionality with any steering mode supported" capability and the "ATSSS-LL functionality with only the Active-Standby steering mode supported" capability;

the network may send Measurement Assistance Information for the UE to send Access availability/unavailability reports to the UPF. In this case, the UE and UPF shall not perform RTT and PLR measurements using PMF as the UE and UPF can use measurements available at the MPTCP layer and/or at the MPQUIC layer.

**** Next Change ****

#### 5.32.6.1	General

The functionality in an ATSSS-capable UE that can steer, switch and split the MA PDU Session traffic across 3GPP access and non-3GPP access, is called a "steering functionality". An ATSSS-capable UE may support one or more of the following types of steering functionalities:

-	Steering functionalities based on multi-path protocols:

-	Steering functionality that applies the MPTCP protocol (IETF RFC 8684 [81]) and is called "MPTCP functionality" (see clause 5.32.6.2.1). This steering functionality can be applied to steer, switch and split the TCP traffic flows identified in the ATSSS/N4 rules. The MPTCP functionality in the UE may communicate with an associated MPTCP Proxy functionality in the UPF, by using the MPTCP protocol over the 3GPP and/or the non-3GPP user plane.

-	Steering functionalities based on MPQUIC that apply the QUIC protocol (IETF RFC 9000 [166], IETF RFC 9001 [167], IETF RFC 9002 [168]) and its multipath extensions (draft-ietf-quic-multipath [174]). The MPQUIC functionality in the UE may communicate with an associated MPQUIC Proxy functionality in the UPF, by using the QUIC protocol and its multipath extensions over the 3GPP and/or the non-3GPP user plane. The following MPQUIC steering functionalities are supported:

-	"MPQUIC-UDP" steering functionality that can be applied to steer, switch and split UDP traffic flows identified in the ATSSS/N4 rules.

-	"MPQUIC-IP" steering functionality that can be applied to steer, switch and split IP traffic identified in the ATSSS/N4 rules.

-	"MPQUIC-E" steering functionality that can be applied to steer, switch and split Ethernet traffic identified in the ATSSS/N4 rules.

-	Steering functionalities based on data switching:

-	A steering functionality called "ATSSS Low-Layer functionality", or ATSSS-LL functionality (see clause 5.32.6.3.1): This steering functionality can be applied to steer, switch and split all types of traffic, including TCP traffic, UDP traffic, Ethernet traffic, etc.

NOTE:	Filters used in ATSSS rules related with a MA PDU Session of type Ethernet can refer to IP level parameters such as IP addresses and TCP/UDP ports.

The UE indicates to the network its supported steering functionalities and steering modes by including in the UE ATSSS Capability one or more of the following capabilities:

1)	The "ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported" capability.

In this case, the UE indicates that it is capable to steer, switch and split all traffic of the MA PDU Session by using the ATSSS-LL functionality with any steering mode allowed for ATSSS-LL, as specified in clause 5.32.8.

2)	The "MPTCP functionality with any steering mode supported" capability and the "ATSSS-LL functionality with only Active-Standby steering mode supported" capability. The MPTCP functionality shall be used only when the type of the MA PDU Session is IPv4, IPv6, or IPv4v6.

In this case, the UE indicates that:

a)	it is capable to steer, switch and split the TCP traffic of the MA PDU Session by using the MPTCP functionality with any steering mode, as specified in clause 5.32.8; and

b)	it is capable to steer and switch all other traffic (i.e. the non-MPTCP traffic) of the MA PDU Session by using the ATSSS-LL functionality with the Active-Standby steering mode, as specified in clause 5.32.8.

3)	The "MPQUIC-UDP functionality with any steering mode supported" capability and the "ATSSS-LL functionality with only Active-Standby steering mode supported" capability. The MPQUIC-UDP functionality shall be used only when the type of the MA PDU Session is IPv4, IPv6, or IPv4v6.

In this case, the UE indicates that:

a)	it is capable to steer, switch and split the UDP traffic of the MA PDU Session by using the MPQUIC-UDP functionality with any steering mode, as specified in clause 5.32.8; and

b)	it is capable to steer and switch all other traffic (i.e. the non-MPQUIC traffic) of the MA PDU Session by using the ATSSS-LL functionality with the Active-Standby steering mode, as specified in clause 5.32.8.

4)	The "MPQUIC-IP functionality with any steering mode supported" capability and the "ATSSS-LL functionality with only Active-Standby steering mode supported" capability. The MPQUIC-IP functionality shall be used only when the type of the MA PDU Session is IPv4, IPv6, or IPv4v6.

In this case, the UE indicates that:

a)	it is capable to steer, switch and split all traffic of the MA PDU Session, by using the MPQUIC-IP functionality with any steering mode, as specified in clause 5.32.8. and

b)	it is capable to steer and switch all traffic of the MA PDU Session by using the ATSSS-LL functionality with the Active-Standby steering mode, as specified in clause 5.32.8.

5)	The "MPQUIC-E functionality with any steering mode supported" capability and the "ATSSS-LL functionality with only Active-Standby steering mode supported" capability. MPQUIC-E functionality shall be used only when the type of the MA PDU Session is Ethernet.

In this case, the UE indicates that:

a)	it is capable to steer, switch and split all traffic of the MA PDU Session of type Ethernet, by using the MPQUIC-E functionality with any steering mode, as specified in clause 5.32.8. and

b)	it is capable to steer and switch all traffic of the MA PDU Session by using the ATSSS-LL functionality with the Active-Standby steering mode, as specified in clause 5.32.8.

6)	The "MPQUIC-IP functionality with any steering mode supported" capability. The MPQUIC-IP functionality shall be used only when the type of the MA PDU Session is IPv4, IPv6, or IPv4v6. This capability shall be provided when the UE supports only MPQUIC-IP functionality.

In this case, the UE indicates that it is capable to steer, switch and split all traffic of the MA PDU Session by using the MPQUIC-IP functionality with any steering mode, as specified in clause 5.32.8.

The above steering functionalities are schematically illustrated in the Figure 5.32.6.1-1, which shows an example model for an ATSSS-capable UE supporting the MPTCP functionality, the MPQUIC functionality(ies) and the ATSSS-LL functionality. The MPTCP flows and the MPQUIC flows in this figure represent the traffic of the applications for which MPTCP protocol can be applied and for which MPQUIC protocol can be applied respectively. The five different IP addresses illustrated in the UE are further described in clause 5.32.6.2.1 and in clause 5.32.6.2.2. When the MPTCP functionality and the MPQUIC functionality(ies) are both applied, the addresses (IP@1, IP@2) used for MPTCP functionality may be the same as the addresses (IP@4, IP@5) used for MPQUIC functionality(ies).

Figure 5.32.6.1-1: Steering functionalities in an example UE model

Within the same IP type MA PDU Session, it is possible to simultaneously steer different packet flows by different steering functionalities. For the same packet flow, only one steering functionality shall be used. For the same Ethernet type MA PDU Session, either ATSSS-LL steering functionality or MPQUIC-E steering functionality is enabled.

All steering functionalities in the UE shall take ATSSS decisions (i.e. decide how to steer, switch and split the traffic) by using the same set of ATSSS rules. Similarly, all ATSSS decisions in the UPF shall be taken by applying the same set of N4 rules, which support ATSSS. The ATSSS rules and the N4 rules supporting ATSSS are provisioned in the UE and in the UPF respectively, when the MA PDU Session is established.

If the UE supports multiple steering functionalities, it shall use the provisioned ATSSS rules (see TS 23.503 [45]) to decide which steering functionality to apply for a specific packet flow.

**** End of Changes ****