3GPP TSG-SA WG2 Meeting #169	S2-2504602

Fukuoka, Japan, 19th May 2025 – 23rd May 2025

Source:	OPPO

Title:	New Solution for KI#2: Sensing Entity Authorization

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution on Sensing Entity Authorization.

- Discussion

KI#2 have been defined in TR 23.700-14 as below:

The key issue is to address the authorization and revocation for a particular sensing service, specifically: - authorization of sensing service requests and revocation of an on-going sensing service; and - authorization and revocation for the sensing entities if applicable. For the authorization of the sensing service request, and the revocation of an on-going sensing service, the study needs to address the following aspects: - whether and how to perform sensing service request authorization if required, and the criteria for the sensing service request authorization, based on the requirements from TS 22.137 [2]; - whether and how to perform sensing service revocation if required, and the criteria for the revocation, based on the requirements from TS 22.137 [2]; - whether, how and which entity(/entities) performs the sensing service request authorization and sensing service revocation if required; - whether and where to store the sensing service request authorization information and sensing service revocation information, and when and how to update the authorization and revocation information if required. For the authorization and revocation for the sensing entities if applicable, the study needs to address the following aspects: - whether, when and which entity performs the authorization and revocation of the sensing entity(/entities); - if needed, the granularity of the sensing entity authorization, e.g. based on TS 22.137 [2]; - if needed, storage and management of the authorization information for the sensing entities. NOTE: The authorization and revocation for the sensing entities can be related to the key issue #3 on sensing entities discovery and selection.

Network authorization is mandatory for sensing-capable entities (e.g., gNBs, UEs) to act as Sensing Transmitters, Receivers, or both. Unauthorized operations pose critical risks:

- Unauthorized devices may intercept or transmit sensitive environmental/object data (e.g., tracking individuals, monitoring restricted zones), violating privacy regulations or enabling malicious surveillance. Authorization ensures only trusted entities access such data. Attackers could exploit rogue transmitters/receivers to spoof signals, jam networks, or steal infrastructure details. Authorization blocks unauthorized devices, safeguarding network integrity.

- Sensing signals share spectrum with communication services. Uncoordinated sensing operations risk interference, degrading network performance (e.g., reduced throughput, increased latency). Authorization enables operators to harmonize resource allocation, prioritizing mission-critical traffic.

- Governments often impose strict RF usage rules (e.g., power levels, frequency bands). Authorization ensures compliance with local regulations (e.g., radio management laws), preventing illegal spectrum occupation.

Thus, networks must first authenticate and authorize sensing entities before permitting sensing operations.

Additionally, requiring explicit verification from a UE (or its user) before enabling sensing measurement and result generation is also crucial, as frequent measurements and calculation drains battery and consumes computing resources.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-14.

* * * Start of Change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | Key Issue #1 | Key Issue #2 | Key Issue #3 | Key Issue #4 | Key Issue #5 | Key Issue #6 |
| #1 |  |  |  |  |  |  |
| #2 |  |  |  |  |  |  |
| #X |  | X |  |  |  |  |

* * * Next Change (All new text)* * * *

## 6.X	Solution #X: Sensing Entity Authorization

### 6.X.1	Description

This solution is for Key Issue#2 on Sensing Entity Authorization.

Before the data collection configuration to the Sensing Entity, 5GC network needs to select the one or multiple Sensing Entities to based on the sensing service requirements.

The Sensing Entities selected for sensing measurement and data collection have to be qualified to ensure security, privacy, and data quality while maintaining network performance and cost efficiency.

The Sensing Entities selected for sensing measurement and data collection have to be available to perform the measurments to ensure reasonable resource consumption.

The SSMF determines whether the selected sensing entity can perform sensing operations in the target service area based on the following information:

- Sensing authorization information of the Sensing Entity.

- Availability information of the Sensing Entity.

When the Sensing Entity is a UE, sensing authorization information is generated by the UDM using UE sensing subscription data stored in the UDM and information from the sensing UE authorization request message.

The UE sensing subscription data includes the following parameters:

-	Whether UE is allowed to perform sensing: as sensing transmitter and/or sensing receiver

-	PLMN

-	Sensing Service Area

-	Sensing request type: one time，periodic，event based

-	Time information: sensing measurement

-	Access Type

-	Sensing service type: Object detection and tracking；Environment monitoring；Motion monitoring

-	UE Availability

-	UE Availability verification indication

UE Availability information refers to whether the UE’s current status (e.g., battery level, CPU load, storage, mobility) permits sensing operations and whether the user consents to such operations.

Editor’s note: Whether it is necesorry to authurise gNB when it is selected as Sensing Entity is FFS.

### 6.X.2	Procedures

Figure 6.X.2-1: Sensing Entity (UE) authurization service flow

- The SSMF receives a sensing service request containing target sensing service area, Sensing request type, Time information, and Sensing service type.

- After determining the sensing mode, the SSMF discovers and selects sensing entities. For sensing modes involving UEs, it selects one or multiple UEs as sensing transmitters or sensing receivers.

- Upon identifying candidate sensing UEs, the SSMF sends a sensing UE authorization request to the UDM. The message includes UE ID(s) and for each UE ID, also includes:

- Sensing roles: sensing transmitter；sensing receiver or both

- PLMN

- Target sensing Service Area

- Sensing request type

- Time information: sensing measurement

- Access Type

- Sensing service type

- The UDM determines whether to authorize sensing for the UE(s) based on Information in the authorization request and UE sensing subscription data. It replies to the SSMF with a response message containing the following information per UE: if UE is permitted to perform sensing operations with the authurised Sensing roles, PLMN, target sensing service area, sensing request type, time information, Access Type, sensing service type, UE Validity Verification Indication (Whether the UE’s availability requires verification).

- If UE Validity Verification Indication is received in step 4, the SSMF verifies the UE’s current status (e.g., battery, mobility) and user consent before finalizing its selection as a sensing entity.

### 6.X.3	Impacts on services, entities and interfaces

SSMF:

  - Sensing Entity authurization

  - Sensing UE availability verification

UDM:

  - Sensing Entity authurization

UE:

  - availability verification

* * * End of Change * * * *