SA WG2 Meeting #169	S2-2504900

May 19 – 23, 2025, Fukuoka, Japan

Source:	Ericsson

Title:	MPQUIC-E and Ethernet Header Compression

Document for:	Discussion

Agenda Item:	19.49

Work Item / Release:	MASSS / Rel-19

Abstract of the contribution: The CT4 LS has brought up new questions that are related to the topic of using MPQUIC-E over Ethernet sessions and this contribution discusses the use of Ethernet Header Compression when MPQUIC-E is used.

# Background

When MPQUIC-E is used for a MA PDU Session, Ethernet frames are encapsulated in QUIC frames that are carried over UDP and IP between UE and UPF. Since Ethernet PDU Sessions are used, RAN may activate Ethernet Header Compression (EHC). However, when MPQUIC-E is used, since the lowest header in the PDU is an IP header instead of an Ethernet header, the EHC handling in UE and RAN may not work as expected. Due to this, when MPQUIC-E is used, it is already specified that SMF indicates to RAN that the PDU Session type is IP (IPv4, IPv6 or IPv4v6). This also has the benefit that IP header compression (ROCH) can be applied.

At SA2#168, related to a discussion about using MPQUIC-E with EPC IWK, it was discussed if this is really needed. It was e.g. suggested that the EHC algorithm could continue to operate even if the lowest header in the PDU is an IP header instead of an Ethernet header. Companies were asked to investigate this further.

# EHC background

For 5GS, EHC is defined in TS 38.323, Annex A and is part of the PDCP layer in the UE and RAN. As described in TS 36.323, it can also apply to Ethernet PDN Connections in EPS.

EHC operates roughly as follows:

- 	For a new Ethernet flow, the Compressor (i.e. RAN in DL, and UE in UL) assigns a Context ID (CID) and forwards the packet with the full header and also includes the CID.

- 	The Decompressor receiving the packet (i.e. UE in DL and RAN in UL) stores the header and the CID, and then sends an ack to the Compressor for this CID.

- 	The Compressor continues to send the full header and the CID until it receives the acknowledgement for the CID from the Decompressor.

- 	After receiving the acknowledgement, the Compressor forwards packets with the CID only (without the header fields). The receiver (Decompressor) adds back the stored header fields corresponding to the CID, before forwarding to higher layers.

A “call flow” is shown in Figure 1 (from TS 38.323). The packet before and after compression is shown in Figure 2.

Figure 1. EHC operation

Figure 2. Compression of Ethernet header

If the maximum number of EHC contexts is reached and the Compressor runs out of CIDs (i.e. if there are many Ethernet flows), and a new Ethernet flow is detected, the compressor needs to re-assign a CID already in use, and send the full header for the new flow, as described above.

# Discussion

With MPQUIC-E, the Ethernet frame will be embedded in a QUIC/UDP/IP packet. The IP packet header structure is shown in Figure 3.

Figure 3. IPv4 and IPv6 packet structure

From a pure algorithm perspective, the EHC mechanism does not seem to rely on any specific property of the Ethernet header, i.e. even if the packet has an IP header, or even a random set of bytes, the algorithm could in principle continue to operate as defined.

However, it can be noted that the IPv4 and IPv6 headers has a Length field within the first 8 bytes. Since the total length of an IP packet likely will be different between packets, it leads to that the EHC compressor will believe it is a new Ethernet flow with a new MAC address, and thus frequently assign a new CID and send the full header to the decompressor. Also, the DiffServ/ECN field may be different (e.g. in different QoS flows) and combining this with the Length fields thus results in additional combinations. Possibly the Length will be the same in several packets (e.g. for large flows with mostly full-length packets, i.e. length = MTU) and in that case the EHC process will lead to actual compression. However, in many cases the length will be different in every packet which will lead to constant CID (re-)assignments and that the number of EHC contexts continuously runs out, causing load in the compressor/decompressor, and actually leading to additional overhead due to the transmission of CIDs in addition to the full header in each packet and frequent CID feedback/acknowledgement messages.

Observation: Applying EHC to IP packets might work in principle but may lead to extra overhead over the air and load in the UE/gNB and this is not controllable or predictable by the operator.

A good EHC implementation should probably be able to operate even if every new “Ethernet frame” has a new MAC address, and continuously recycle CIDs, but it is not likely that such a traffic scenario has been tested in operator’s networks.

There are also other potential issues. Even though it is not described in standards, EHC implementations may e.g. decide to check the validity of Ethernet headers which risk causing problems if the first 14-20 bytes contain parts of an IP header instead.

# Conclusion

Applying EHC in UE and RAN even if the packet is an IP packet might in principle work, but risk negative side effects. It will likely lead to more load over the air (EHC feedback messages, additional packet overhead etc) and may also cause problems in implementations in the field. This is not controllable or predictable by the operator.

It therefore seems appropriate to not apply EHC to MPQUIC-E traffic.