3GPP TSG-SA2 Meeting #169	S2-2505030

# 19 - 23 May, 2025, Fukuoka, Japan                                                            rev of S2-2503598

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0721 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on privacy check for data collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The UE LCS privacy is defined in 23.273, where the following is defined and the privacy is performed by GMLC for obtaining UE's positioning using LCS procedure. UE LCS privacy is a feature which allows a UE and/or AF to control which LCS clients and AFs are and are not allowed access to UE location information. However, for data collection by LMF for model training and performance monitoring, it was agreed to use new procedure bypassing GMLC, to support LMF invoking UDM service for user consent check. As per above - GMLC is not involved in the procedures for data collection by LMF for model training and performance monitoring, the UE LCS privacy check is missing - LMF can invoke UDM service for user consent check. //added reason for change in r01 In S2-2504530, "SA3 agrees that LCS privacy profile defined in TS 23.273 and user consent in Annex V of TS 33.501 are independent procedures." Therefore, there is no need to mix LCS privacy profile check and user consent check together, and no need to chanllege the agreed conclusion to have user consent check for data collectio for model training and performance monitoring. //end of change for r01 And it is proposed to clarify to perform privacy check for the ground truth data collection from UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification for privacy check for the ground truth data collection from UE in clause 5.18.2 and clause 6.22.3. Add LMF as LCS for allowed to locate the particular UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No UE LCS privacy check when LMF collects UE’s location as ground truth data. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.0, 5.18.2, 6.22.3, 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

5.18.0	General

The LMF may calculate the UE location and estimate the achieved accuracy by using LMF-based AI/ML Positioning. When receiving the request from AMF for determining a UE location, the LMF selects an appropriate positioning method as described in clause 5.2 to determine the result of the positioning. The result of the positioning may be calculated by using LMF-based AI/ML Positioning ML model supported by LMF. The LMF collects input data from UE or NG-RAN for the LMF-based AI/ML Positioning to perform location calculation and provide the location to the consumer.

NOTE 1:	Whether to select LMF-based AI/ML Positioning for location result calculation is determined by LMF.

NOTE 2:	The specific measurement data collected by LMF from UE and NG-RAN for LMF-based AI/ML Positioning are in the scope of RAN specifications and not in the scope of this specification.

Editor's note:	What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs.

The ML model that is used for LMF-based AI/ML Positioning may be trained by LMF. The trigger for data collection required for model training in LMF is up to implementation. LMF collects input data from UE for ML model training as described in clause 6.22.2. The LMF collects input data from NG-RAN for ML model training as described in clause 6.22.3.

The LMF may also request a trained ML model for LMF-based AI/ML Positioning from NWDAF containing MTLF as described in clause 6.22.5. The LMF discovers a suitable NWDAF containing MTLF via NRF as described in clause 5.2 of TS 23.288 [37] with the following considerations:

-	The LMF may provide the positioning case information(e.g. UE assisted/LMF-based AI/ML Positioning case, NG RAN node assisted LMF-based AI/ML Positioning case, or both).

-	The LMF provides an Area of Interest, and may include the ML Model Interoperability indicator to discover an NWDAF that can provide an AI/ML Model.

-	The LMF may request a NWDAF with ML Model accuracy checking capability to report when the model is degraded (i.e. for ML model performance monitoring).

NOTE 3:	Other NWDAF discovery parameters listed in clause 5.2 of TS 23.288 [37] such as Analytics ID, FL capability type and related time period, S-NSSAI or any roaming capabilities are not included by LMF.

Editor's note:	The types of LMF-based AI/ML positioning model (e.g., per positioning case) is FFS, which needs to be coordinated with RAN WGs.

The LMF requests the NWDAF containing MTLF to provide an ML Model for LMF-based AI/ML Positioning as described in clause 6.2A of TS 23.288 [37] with the following considerations:

-	The LMF provides the following input parameters in the Nnwdaf_MLModelProvision_Subscribe or Nnwdaf_MLModelInfo_Request:

-	LMF-based AI/ML positioning indication.

-	Optionally, Vendor ID, ML Model Filter Information (e.g. Area of Interest), Positioning case information, Target of ML Model Reporting, ML Model Target Period, Time when model is needed, Inference Input Data information and ML Model Monitoring Information.

-	If vendor specific information is required , then the ML Model Interoperability Information is included.

-	If the LMF supports multiple AI/ML Models, indication of support for multiple ML Models, optionally with Number of ML Models and Accuracy level(s) of Interest.

The NWDAF containing MTLF collects input data to perform the ML model training as described in clause 6.22.4, and the NWDAF containing MTLF performs ML model provision to LMF as described in clause 6.2A of TS 23.288 [37] with the following considerations:

-	The NWDAF containing MTLF provides the ML Model identifier and ML Model Information for the ML Model for UE Positioning, and optionally, the following parameters:

-	ML Model Filter Information and/or Target of ML Model Reporting, if the ML Model provisioning request includes multiple ML Model Filter Information and/or Target of ML Model Reporting;

-	Indication of whether the ML Model identifier is updated (e.g. retrained ML model).

-	Validity period, Spatial validity, Training Input Data Information, ML Model accuracy Information.

Editor's note:	The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs.

Once the ML model for LMF-based AI/ML Positioning is trained and available in the LMF, the LMF may use it to perform UE Positioning after receiving a location determination request from AMF.

Editor's note:	Whether user consent for UE positioning calculation is needed apart from the existing GMLC check of the LCS privacy profile is FFS.

For ML model training, performance monitoring and UE positioning calculation using LMF-based AI/ML Positioning, the LMF checks with UDM the user consent status and UE LCS privacy before collecting UE related data, see clause 6.22.3 and clause 6.22.4.

Either LMF or NWDAF containing MTLF may perform performance monitoring for LMF-based AI/ML Positioning. When the ML model that is used for LMF-based AI/ML Positioning is trained by LMF the LMF monitors the performance of the ML model. When the ML model that is used for LMF-based AI/ML Positioning in LMF is trained by NWDAF containing MTLF, then the NWDAF containing MTLF monitors the performance of the ML model. LMF may determine whether to use the LMF-based AI/ML Positioning to perform location calculation based on the model performance monitoring result. LMF or the NWDAF containing MTLF may also trigger the ML model retraining based on the model performance monitoring result.

* * * Next Change * * *

5.18.2	AI/ML model performance monitoring for LMF-based AI/ML Positioning

When the AI/ML model that is used for LMF-based AI/ML Positioning is trained by LMF, the LMF may perform performance monitoring for AI/ML model by collecting the measurements data and ground truth data of PRU(s)/UE(s) by using the procedures of data collection for LMF-based AI/ML Positioning as specified in clause 6.22.2 or clause 6.22.3. Before collecting the data from UE(s), LMF needs to check the user consent and UE LCS privacy in UDM, however LMF does not need to check user consent and UE LCS privacy for data collected from PRU. LMF calculates the location information based on the collected measurement data by using AI/ML model. Based on the calculated location information and ground truth data, LMF evaluates the AI/ML model performance to generate the performance monitoring result. The result may trigger LMF to change the positioning method, e.g. from LMF-based AI/ML Positioning to the legacy positioning or retrain the AI/ML model in LMF.

When the AI/ML model that is used for LMF-based AI/ML Positioning in LMF is trained by NWDAF containing MTLF, then the NWDAF containing MTLF may monitor the performance of the AI/ML model as specified in clause 6.2E.4 of TS 23.288 [37] based on the data collection via the LMF as specified in clause 6.22.4. If the data collection is not granted, the LMF does not perform any data collection procedure for the ML model performance monitoring for this UE.

* * * Next Change * * *

6.22.3	Data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements

The procedure for data collection from NG-RAN is used to e.g. train the ML Model for LMF-based AI/ML positioning.

Figure 6.22.3-1: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements

1.	The LMF determines that data collection from the NG-RAN is required e.g. to train an ML Model for UE positioning for a number of UEs or to monitor the ML Model performance. The LMF may also initiate the data collection upon the request of an NWDAF containing MTLF as described in step 3 in figure 6.22.4-1.

2.	The LMF may know the SUPIs of the UEs for which to collect location measurement data from the NG-RAN, e.g. when training an ML model using the PRU(s) associated to this LMF. The LMF may optionally invoke an Nnrf_NFDiscovery Request service operation to an NRF to discover other PRU serving LMF(s) which has associated PRUs in the area of interest and send an Nlmf_Location_MeasurementData Request service operation to the selected PRU serving LMFs to collect the measurement data from the NG-RAN for the PRUs and the PRU locations.

If the LMF does not know the SUPIs of the UEs for which to collect location measurement data from the NG-RAN, the LMF tries to get the list of SUPIs from the AMF. Before that the LMF discovers the AMF(s) that serves the area of interest via the NRF using Nnrf_NFDiscoveryRequest.

3.	The LMF subscribes to the list of SUPIs in an area of interest from the AMF(s) using Namf_EventExposure_Subscribe request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest").

The AMF sends Namf_EventExposure_Subscribe response or Namf_EventExposure_Notify (list of SUPIs in the area of interest).

For each SUPI in the area of interest, the following steps are performed.

4.	The LMF checks whether the SUPI provided user consent for data collection for a purpose with UDM using Nudm_SDM_Get including subscription data type set to "User consent" for this SUPI. The LMF also checks the UE LCS privacy with UDM via invoking Nudm_SDM_Get service operation including subscription data type set to "LCS privacy" for the SUPI that is not PRU.

NOTE:	The User consent and UE LCS privacy information can be retrieved simultaneously in one Nudm_SDM_Get service operation invocation.

5.	The LMF subscribes to UDM to notifications of changes on subscription data type "User consent" and "LCS privacy" for this SUPI using Nudm_SDM_Subscribe. If user consent is provided and user consent resulte is allowed, then step 6 follows, otherwise no data is collected for this SUPI, i.e. Steps 6 to step 9 are not performed.

Editor's note:	Further details on user consent for data collection for a specific purpose, e.g. for model training and/or for performance monitoring for LMF-based AI/ML positioning will be aligned with SA WG3.

6.	The LMF requests input data for the UE from the NG-RAN.

7.	The LMF requests ground truth data from the UE.

8.	The LMF may determine that the UE is no longer in the area of interest, based on the AMF notification using Namf_EventExposure service, then the LMF performs step 10 and step 11, the LMF may unsubscribe to be notified on user consent updates and LCS privacy updates if the UE is not in the area of interest any longer.

9.	The UDM may notify the LMF on changes of user consent and/or changes LCS privacyat any time after step 5 using Nudm_SDM_Notification including SUPI and Subscription data type set to "User consent" and/or "LCS privacy". If user consent is no longer granted for a user or LCS privacy check result is not allowed for which data has been collected the LMF performs step 10 and step 11. The LMF may unsubscribe to be notified of user consent updates from UDM for each SUPI for which data consent has been revoked, using Nudm_SDM_Unsubscribe including SUPI and Subscription data type set to "User consent". The LMF may unsubscribe to be notified of LCS privacy updates from UDM for each SUPI for which LCS privacy profile has been revoked, using Nudm_SDM_Unsubscribe including SUPI and Subscription data type set to "LCS privacy".

10.		The LMF request NG-RAN to stop reporting input data for the target UE.

11.	The LMF stop any retrieval of ground truth data from the UE.

The measurements and ground truth data are used for ML model training. The UE location is derived from the measurements data by using LMF-based AI/ML Positioning. The derived UE location and ground truth data are used for ML model performance monitoring.

The LMF may initiate data collection for multiple UEs simultaneously, as such steps 6 and 7 may occur in parallel for a number of SUPIs as determined by the LMF.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

Editor's note:	Whether and how the LMF takes the RAN load into account in this procedure is FFS.

* * * Next Change * * *

7.1	UDM

For each UE subscriber the UDM stores LCS related data as part of the Subscriber Data Management (SDM) service as defined in clause 5.2.3.3.1 of TS 23.502 [19].

The privacy profile data is defined in table 7.1-1 containing data for the privacy classes for which location of the target UE is permitted. For the meaning of each LCS privacy profile data type and included data, refer to clause 5.4.2.

Table 7.1-1: LCS privacy profile data stored in the UDM for a UE Subscriber

| Privacy Profile Data Type | Presence | UDM data |
| --- | --- | --- |
| Location Privacy Indication | M O | Indication of one of the following mutually exclusive global settings: - Location is disallowed. - Location is allowed (default). Time period when the Location Privacy Indication is valid. |
| Call/session Unrelated Class | M O O O O O O O O O O O O O | For any LCS client or AF not in the external LCS client list or otherwise identified for the Call/session Unrelated Class, the following data may be present: - One of the following mutually exclusive options: - Location not allowed (default case) - Location allowed with notification. - Location with notification and privacy verification; location allowed if no response. - Location with notification and privacy verification; location restricted if no response. - Time period when positioning is allowed. - Geographical area where positioning is allowed. - Indication that codeword shall be checked in UE or one or more codeword values to be checked in GMLC. External LCS client list: a list of zero or more LCS clients, AFs and LCS Client groups with the following data for each entry: - One of the following mutually exclusive options: - Location allowed without notification (default case). - Location allowed with notification. - Location with notification and privacy verification; location allowed if no response - Location with notification and privacy verification; location restricted if no response - Time period when positioning is allowed - Geographical area where positioning is allowed Service types list: a list of one or more service types for which the LCS client is allowed to locate the particular UE. The possible service types are defined in TS 22.071 [2]. The following data may be present for each service type in the list: - One of the following mutually exclusive options: - Location allowed without notification (default case) - Location allowed with notification - Location with notification and privacy verification; location allowed if no response. - Location with notification and privacy verification; location restricted if no response. - Time period when positioning is allowed. - Geographical area where positioning is allowed. - Indication that codeword shall be checked in UE or one or more codeword values to be checked in GMLC. |
| PLMN Operator Class | O | LCS client list: a list of one or more generic classes of LCS client that are allowed to locate the particular UE. The following classes are distinguished: - LCS client broadcasting location related information. - O&M LCS client in the HPLMN. - O&M LCS client in the VPLMN. - LCS client recording anonymous location information. - LCS Client supporting a bearer service, teleservice or supplementary service to the target UE. - NWDAF in the HPLMN (when the UE is currently being served by the HPLMN). - NWDAF in the VPLMN. - LMF in the Serving PLMN. |
| User Plane Connection between UE and LCS Client or AF | O | Indication of one of the following mutually exclusive global settings: - UE is allowed to report periodic or triggered location events via user plane to an LCS Client or AF - UE is not allowed to report periodic or triggered location events via user plane to an LCS Client or AF (default). |
| Event report expected area | O | Presents a geographical area generated by UE, which is used by GMLC to determine event report allowed area for the UE. |
| Area usage indication | O | Indication of one of the following mutually exclusive global settings on using event report allowed area: - Inside reporting (default). - Outside reporting. |
| GMLC address list | O | Addresses of GMLC located in local network(s) which are allowed to be used for the UE positioning. The GMLC address is used to identify the local network for verification of location service in PNI-NPN. |

* * * End of Change * * *