3GPP TSG-WG SA2 Meeting #169	S2-2505420

# Fukuoka, JP, 19 - 23 May 2025											     (revision of S2-xxxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6300 | rev | 1 | Current version: | 17.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE radio capability for paging handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core, 5GS_Ph1, TEI17 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | #### For UE radio capability for paging handling, RAN2 replied LS R2-2501388/S2-2502821 and R2-2503172/S2-2504513. RAN3 LS R3-252276/S2-250xxxx questions further on the uploading of UE radio capability for paging and term “that PLMN” used in SA2. From the replied RAN2 LSs, it’s clear that the retrival and application of proper radio capability for paging for ceration functions/features mainly depends on the network deployment (e.g. same relase needs to be supported by gNB for certain features, with imapct to UE readio paging capability, to work properly). From the questions in the RAN3 LS, the meaing of “that PLMN” needs to be clarified. The uploading of the UE radio capability for paging from NG-RAN to AMF also needs to be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding NOTE to clarify the expected deployment of NG-RAN node to support certain functions. Clarify the serving PLMN. Clarify the uploading of the UE radio capability for paging to CN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear description on the UE radio capability for paging which can impact the functions/features that relys on proper handling of UE radio capability for paging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.4.4.3	Paging assistance information

The paging assistance information contains UE radio related information that assists the RAN for efficient paging. The Paging assistance information contains:

a)	UE radio capability for paging information:

-	The UE Radio Capability for Paging contains information derived by the NG-RAN node (e.g. band support information) from the UE Radio Capability information. The NG-RAN provides a single UE Radio Capability for Paging (including different NG-RAN RATs) to the AMF as specified in TS 38.413 [34] and the AMF stores this information without needing to understand its contents.

As the AMF only infrequently (e.g. at Initial Registration) prompts the NG-RAN to retrieve and upload the UE radio capabilities i.e. UE Radio Capability information to the AMF, and the AMF may be connected to more than one NG-RAN RAT, it is the responsibility of the NG-RAN to ensure that UE Radio Capability for Paging (which is derived by the NG-RAN node) contains information on NG-RAN RATs that the UE supports in the serving PLMN. To assist the NG-RAN in this task, as specified in TS 38.413 [34], the AMF provides its stored UE Radio Capability for Paging in every NG-AP INITIAL CONTEXT SETUP REQUEST message sent to the NG-RAN.

NOTE:	For functions (e.g. NR RedCap UE) relying on UE Radio Capability for Paging to work properly, it is expected that all deployed NG-RAN nodes in the serving PLMN support the UE Radio Capability for Paging specific to these functions.

-	The UE Radio Capability for Paging is maintained in the core network, even during AMF reselection, and is stored in the UCMF alongside the UE Radio Capability information associated to a UE Radio Capability ID.

b)	Information On Recommended Cells and RAN nodes For Paging:

-	Information sent by the NG-RAN, and used by the AMF when paging the UE to help determining the NG RAN nodes to be paged as well as to provide the information on recommended cells to each of these RAN nodes, in order to optimize the probability of successful paging while minimizing the signalling load on the radio path.

-	The RAN provides this information during N2 release.

# * * * End of changes * * * *