3GPP TSG-SA WG2 Meeting #169	S2-2505700

# Fukuoka, Japan, May 19 - 23, 2025											 (revision of S2-2505457)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1465 | rev | 7 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on support of DC interworking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In AC.7.9.2, the signaling flow of how to perform Real-time Screen Sharing from DCMTSI UE to MTSI UE is given. In this procedure, a new video media is added to UE#2 for converting screen sharing to video when the original call is a voice call. Considering that screen sharing is unidirectional, the video media shall be indicated as "sendonly", which also can avoid the problem that the user of UE#2 may be confused and reject the addition because of diswillingness of opening camera. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new description that the new added video media is for sendonly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The user may be confused and reject to use such service. Waste network resource. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | To be implemented after CR1539 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change ---

### AC.7.9.2	Signalling Procedure of Application Data Channel Interworking via MF

Figure AC.7.9.2-1 depicts a signalling flow diagram for establishing the application data channel when the originating DCMTSI UE initiates an IMS DC session to the terminating MTSI UE, providing interworking of application data channel to MTSI UE via MF. This signalling flow takes the data channel application which enables the transcode between the data channel media and the MMTEL media such as audio and video, e.g. Real-time Screen Sharing.

Figure AC.7.9.2-1: Application Data Channel set up signalling procedure for IMS data channel interworking with MTSI UE via MF

0.	The UE#1 initiates an IMS session setup procedure towards the UE#2 with initial SDP offer that includes audio media and bootstrap data channel description. The DCSF is aware that the terminating network or UE#2 does not support or does not want to use IMS data channel in the session, e.g. the SDP offer that includes bootstrap data channel description has been rejected by the terminating network or UE#2.

1-3.	After the IMS session with audio media between UE#1 and UE#2 as well as bootstrap data channel between UE#1 and network side has been established, the UE#1 initiates a re-INVITE request to establish an application data channel in which the application data channel and data channel application information (for instance, application ID) is included. The IMS AS validates user subscription data to determine whether this request should be notified to the DCSF as stated in clause AC.7.2.

4.	After receiving the notification, the DCSF determines the policy about how to process the application data channel establishment request based on the related parameters (i.e. associated DC application binding information as application ID) in the notification and/or DCSF service specific policy.

5.	According to step 0, the DCSF determines that interworking of the application data channel media to MTSI UE is needed and possible, based on the application information, i.e. application id.

6.	The DCSF invokes Nimsas_MediaControl_MediaInstruction to the IMS AS and instructs the IMS AS to create resources and association for the application data channels on UE#1 side and video media on UE#2 side. The DCSF also includes the instruction on how to transcode data channel media to video media. The DCSF includes the instruction that the video media on UE2#2 side is one-way from UE#1 side to UE#2 side.

7.	The IMS AS invokes Nmf_MediaResourceManagement to the MF to allocate resource and transcode data channel media to video media according to DCSF's instruction.

NOTE:	The media instruction enables the MF to perform transport protocol translation from DC protocol (UDP/DTLS/SCTP) on one side and video media (RTP/AVPF) on the other.

8-9.	IMS AS responds to the MediaInstruction request received in step 6. The response may include the atomic success result of operation and also includes negotiated data channel media resource information for MDC1. The DCSF stores the media resource information and responds to the request received in step 3.

10-11.	After the media resource reservation, the IMS AS sends a re-INVITE request with the SDP offer for anchoring the video media of UE#2 to the MF. According to the instruction received in step 6, the video media is associated with direction attribute setting to "sendonly" to disable the uplink video media from the UE#2 to the MF.

12-14.	After terminating network side negotiation, UE#2 and terminating network returns 200 OK for the re-INVITE.

15-16.	DCSF is notified about the terminating network media negotiation result.

17-21.	The IMS AS sends 200 OK for re-INVITE initiated by UE#1 and UE#1 returns ACK.

22.	UE#1 initiates SCTP association and DTLS connection establishment procedures and establishes the application data channels with the MF.

23-24.	The IMS AS reports media negotiation result to the DCSF and reports that anchoring video stream is successful. The DCSF sends a Call Control Request to the MMTel AS indicating it to continue the call process.

25.	The IMS AS sends a Media Resource Management Request to the MF to associate application data channel stream between UE#1 and the MF with the allocated video streams between UE#2 and the MF.

The application data channels between the MF and UE#1 are associated with the RTP video streams between the MF and UE#2. The RTP stream of UE#1's (e.g. screenshare sequence) is received by UE#2 from video streams.

## ---End of the Change---