SA WG2 Meeting #169	S2-2504837

Fukuoka, Japan, May 19 – 23, 2025

Source:	NTT DOCOMO

Title:	KI#1: New Sol: Energy Consumption estimation considering idle energy consumption

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes a solution for KI#1 where the estimation of the energy consumption by EIF considers the idle energy consumption of the node.

# 1	Discussion

According to the documented model/formula (in Annex T of TS 23.501) of energy consumption, to estimate EC of a given granularity level, e.g., UE level, the total node-level energy consumption i.e., EgNB or EUPF is multiplied by the ratio of data volume of the requested granularity level. But the issue is that the total node-level energy consumption also includes the energy consumed to power on the node without providing service. Contributing this amount of the consumed energy in the estimations that are exposed to the consumer would be confusing and misleading. This paper proposes a new energy consumption estimation model to eliminate the impact of the consumed energy.

# 2 Proposal

It is proposed to update TR 23.700-67 according to the following text.

********** First Change (All new text) **********

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |
| 1 | X |  |  |
| 2 | X | X |  |
| 3 |  | X |  |
| 4 |  | X |  |
| 5 |  | X |  |
| 6 |  | X |  |
| 7 |  | X |  |
| X | X |  |  |

## 6.x	Solution #X: Energy Aware Data Transfer Policy Negotiation

### 6.x.0	High-level solution principles

The high-level principles of the proposed solution are as follows:

-	The total node-level energy consumption is composed of two main parts: 1) Static/idle energy consumption to power on the node and 2) Dynamic energy consumption to handle UE traffic.

-	Including the static/idle energy consumption in the estimation of energy consumption at finer granularities, which are exposed to the consumer by the EIF, would be confusing and misleading, since that portion of energy is not consumed to handle UE traffic.

-	By taking into account the node's static/idle energy consumption (provided by OAM) in the energy consumption estimation models used by the EIF, the exposed energy-related information would be more accurate in terms of its dependency on traffic volume.

### 6.x.1	Description

According to the documented model/formula (in Annex T of TS 23.501) of energy consumption, to estimate EC of a given granularity level, the total node-level energy consumption i.e., EgNB or EUPF is multiplied by the ratio of data volume of the requested granularity level. However, the issue is that not all the EgNB or EUPF are consumed to serve the UE traffic volume. Indeed, the consumed energy includes two main terms:

- Static/Idle power consumption: the energy consumed to power up the node without any load (idle mode)

- Dynamic power consumption: the energy consumed to serve UE traffic

These two components are typically modeled in the literature with the following model:

[Vishwanath, Arun, et al. "Modeling energy consumption in high-capacity routers and switches." IEEE Journal on Selected Areas in Communications 32.8 (2014): 1524-1532.]

Using the total node-level energy (i.e., summation of the static and dynamic energy consumption) to estimate the EC of the given granularity may lead to a high influence of the static power consumption on the estimation and raises robustness/sensitivity issues explained by S2-2501605 at SA2#167; which is exemplified here again as follows.

Let’s assume:

- Static EC of UPF: 100

- EC per 1 Mbit of traffic: 10

- UE sends 1 Mbit in the given time window

The total power consumption of UPF measured by OAM will be:

EUPF=ECstatic+ECDynamic= ECstatic+load*EC(per unit of load)

Estimation of the EC for the UE in two cases for the same amount of traffic would be different as follows (for the sake of simplicity, energy consumption in RAN part is not considered):

- If total load of the UPF in the given time window is 10 Mbit:

  - Measured  EUPF=100+10 ×10=200

  - ECUE=EUPF×DVUEDVUPF= 200×110=20

- If total load of the UPF in the given time window is 1 Mbit:

  - Measured EUPF=100+1 ×10=110

  - ECUE=EUPF×DVUEDVUPF=110×11=110

These estimations/calculations are quite confusing for the consumer since for the transfer of the same amount of traffic by the UE, the estimation of energy consumption is substantially different (i.e., 20 vs 110).

To address the issue, it is proposed to exclude (deduct) the (estimation of) static/idle power consumption from the total energy consumption; i.e., instead of EUPF or EgNB in the formula, E'UPF=EUPF-EUPF,0 or E'gNB=EgNB-EgNB,0 are used where EUPF,0 or EgNB,0 are the (estimation) of the static/idle power consumption of the node.

To show the impact of the proposed change, let’s assume OAM provides an estimation of  EUPF,0 (which is the energy consumption of the UPF when it just powered on – i.e. idle condition).

Let’s assume:

- Actual Static EC of UPF: 100

- OAM provides estimation of EUPF,0=98 (it is assumed that it is a bit different from the actual EC due to measurement errors)

- EC per 1 Mbit of traffic: 10

- UE sends 1 Mbit in the given time window

Estimation of the EC for the UE in two cases for the same amount of traffic would be different as follows

- If total load of the UPF in the given time window is 10 Mbit:

  - Measured EUPF=100+10 ×10=200

  - ECUE=E'UPF×DVUEDVUPF=(200-98)×110=10.2

- If total load of the UPF in the given time window is 1 Mbit:

  - Measure EUPF=100+1 ×10=110

  - ECUE=E'UPF×DVUEDVUPF=(110-98)×11=12

With this proposed correction, as it is illustrated, the estimated EC of UE values in two cases are not significantly different.

6.x.1.1	Considering NF scaling

In virtualized deployment, network functions are realized by VNF. The resource allocated to the VNF and consequently the static/idle energy consumption is not necessarily a fixed value, i.e., the OAM may scale in/out or up/down the VNF. To consider this dynamic behaviour of the VNFs, in this solution is it assumed that OAM is able to provide (an estimation of) static/idle energy consumption per scaling level.

As explained in clause 6.7.3.1 of TS 28.554, the energy consumption of virtualized NFs is estimated based on the allocated resources. It is not directly measure. Hence, this understanding is that, to estimate the static/idle energy consumption it is not needed to create an isolated environment without any load on the NF; and it would be possible for OAM to provide the static/idle energy consumption per scaling level considering the allocated resources.

NOTE: Availability of (an estimation of) static/idle energy consumption of a node (i.e., UPF and gNB) per scaling level needs SA5 confirmation.

6.x.1.2	Energy consumption estimation model

This solution proposal is as follows: In the current EC model/formula documented in Annex T of TS 23.501, exclude (deduct) the (estimation of) node static/idle power consumption from the total energy consumption of the node; i.e., instead of EUPF or EgNB, E'UPF=EUPF-EUPF,0 or E'gNB=EgNB-EgNB,0 are used where EUPF,0 or EgNB,0 are the (estimation) of the static power consumption of the node (i.e., the node level energy consumption provided by OAM when the load on the node is (almost) zero).

EUE,gNB=(EgNB- EgNB,0)DVUE,gNBDVgNB

ES-NSSAI,gNB=(EgNB- EgNB,0)DVS-NSSAI,gNBDVgNB

ESession,gNB=(EgNB- EgNB,0)DVSession,gNBDVgNB

EFlow,gNB=(EgNB- EgNB,0)DVFlow,gNBDVgNB

EUE,UPF=(EUPF- EUPF,0)DVUE,UPFDVUPF

ES-NSSAI,UPF=(EUPF- EUPF,0)DVS-NSSAI,UPFDVUPF

ESession, UPF=(EUPF- EUPF,0)DVSession,UPFDVUPF

EFlow,UPF=(EUPF- EUPF,0)DVFlow,UPFDVUPF

EUE=gNBi∈gNBUETEUE, gNBi+ UPFi∈UPFUETEUE, UPFi

ES-NSSAI=gNBi∈gNBS-NSSAITES-NSSAI, gNBi+ UPFi∈UPFS-NSSAITES-NSSAI, UPFi

ESession=gNBi∈gNBSessionTESession, gNBi+ UPFi∈UPFSessionTESession, UPFi

EFlow=gNBi∈gNBFlowTEFlow, gNBi+ UPFi∈UPFFlowTEFlow, UPFi

### 6.x.2	Procedures

The procedure is the same as clause 4.29.2 of TS 23.502 where EIF also collects the static/idle energy consumption of the node in the current scaling level.

### 6.x.3	Impacts on existing services, entities and interfaces

The solution has the following impacts:

EIF:

-	Collecting the static/idle power consumption of the node

-	Taking into account the static/idle power consumption of the node in the energy consumption estimation

OAM:

-	Providing static/idle power consumption of node at each scaling level

********** End of Changes **********