3GPP TSG-SA2 Meeting #169	S2-2504839

19 - 23 May, 2025, Fukuoka, Japan

Source:	China Telecom

Title:	New Sol: General procedure to enable sensing service

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The contribution proposes general procedure to enable sensing service in the network.

1. Introduction

This solution proposes the general procedure to enable end-to-end sensing service, which is related to all 6 Key Issues. Detailed solutions for specific Key Issues will not be involved in this solution.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X | X | X | X | X | X | X |

# * * * * Second change (All Text New) * * * *

6.X	Solution #X: General procedure to enable sensing service

6.X.0	High-level solution Principles

The high level principles of the solution are:

- A new Sensing Function is introduced for sensing service management, which can be further splitted to the Control Plane Sensing Function (SF-C) and the User Plane Sensing Function (SF-U).

- Only the authorized AF can access and invoke the sensing service.

- The Sensing Service Consumer initiates the sensing service request(s) to obtain the Sensing Result.

- The network can select appropriate Sensing Function(s) and Sensing Entity/Entities to support sensing service.

6.X.1	Description

To enable end-to-end sensing services, the essential functionalities contain sensing service authorization, discovery and selection of Sensing Entity/Entities, sensing data collection, Sensing Result calculation and exposure, etc.

Sensing service can be initiated by a Sensing Service Consumer (either an AF application, an internal NF, or a UE). The main procedures are the followings:

-	When the Sensing Service Consumer is an AF application, the AF sends the sensing service request(s) to the NEF, when received the sensing service request(s), the NEF selects an appropriate SF to manage sensing service.

-	When the Sensing Service Consumer is an internal NF or a UE, the internal NF or the UE sends the sensing service request(s) to the appropriate SF.

NOTE 1: 	The UE that requests the sensing service is not required to be the sensing transmitter/receiver.

-	Based on the received sensing service request(s), the SF performs the following functionalities:

-	Discovers and selects appropriate Sensing Entity/Entities to collect 3GPP Sensing data;

-	When the 3GPP Sensing data is transmitted to the SF, the SF further performs calculation to obtain the Sensing Result, and then exposes the Sensing Result to the AF via the NEF or reports the Sensing Result to the internal NF or reports the Sensing Result to the UE.

Considering the Sensing Result calculation demands significantly more computational resources than sensing signaling handling, while Sensing data transmission consumes greater network resources, the Sensing Function can be splitted into Control Plane Sensing Function (SF-C) and User Plane Sensing Function (SF-U), the functionalities of two separated entities are as follows:

-	SF-C: Authorization and revocation of the sensing service, discovery and selection of Sensing Entity/Entities based on the service requirements, parameter/policy configuration to the Sensing Entity/Entities, as well as exposing the Sensing Result and contextual information to the Sensing Service consumer.

-	SF-U: Sensing Result calculation using the 3GPP Sensing data received from Sensing Entity/Entities, as well as reporting the Sensing Result to the SF-C.

NOTE 2: The detailed interaction procedures between SF-C and SF-U, and the deployment of SF-C entity and SF-U entity are not involved in this solution.

6.X.2	Procedures

Taking the Sensing Service Consumer is an AF as an example, the general procedure for sensing service are as follows:

Figure 6.X.2-1: Procedure of AF initiated sensing service

1.	The AF sends the sensing service request(s) to the NEF, which may include:

-	Sensing service type: dynamic map, speed detection, vehicle tracking, emergency event notification, etc.

-	Sensing service requirement: the target area/the target object, the required report frequency (e.g., one-time, periodic, or event-based reporting), the required Sensing Result accuracy, etc.

2.	When received the sensing service request(s) from the AF, the NEF performs the authorization of AF request(s). The authorization information may be stored locally in the NEF or in the UDM, and the NEF may request authorization verification from the UDM.

3. 	The NEF may discover and select the appropriate Sensing Function(s) for sensing service management.

4.	The NEF sends the sensing service request(s) to the Control Plane Sensing Function (SF-C), which contains the information provided by the AF. Optionally, when the Sensing Service Consumer is an internal NF or a UE, the internal NF sends the sensing service request(s) to the appropriate SF-C.

5. The SF-C may perform further authorization and revocation of the sensing service, and then sends a response to the AF (via NEF) to indicate whether the sensing request has been authorized or rejected. If the SF is not involved in authorization, this step can be executed after step 2.

6. The SF-C may perform Sensing Entity/Entities discovery and selection, and optionally AMF discovery and selection, based on the sensing service requirement and sensing capability of the Sensing Entity/Entities, etc.

7. The SF-C may perform parameter/policy configuration to the Sensing Entity/Entities to help with sensing-related measurement.

8. Based on the received sensing service requirement, the Sensing Entity/Entities perform 3GPP sensing data collection.

9. The collected 3GPP sensing data and associated information are transmitted to the User Plane Sensing Function (SF-U).

NOTE 3: Whether sensing data transmission needs the involvement of the UPF is not addressed in this solution.

10. Based on the received sensing service requirement, the SF-U performs Sensing Result calculation.

11. The SF-U sends the Sensing Result to the SF-C, and the NEF further reports the Sensing Result to the AF.

12. The SF-C sends the Sensing Result and contextual information to the NEF, and the NEF further reports the Sensing Result and contextual information to the AF.

NOTE 4: In the above steps, the detailed solutions for specific Key Issues are not involved.

6.X.3	Impacts on services, entities and interfaces

AF:

-	Initiate the sensing service request(s) to obtain the Sensing Result(s).

NEF:

-	Interact with AF and Sensing Function for sensing authorization and revocation.

-	Select the appropriate Sensing Function(s).

-	Report the Sensing Result and contextual information to the AF.

Sensing Function:

-	Control Plane Sensing Function (SF-C):

-	Authorization and revocation of the sensing service.

-	Discovery and selection of Sensing Entity/Entities.

-	Parameter/policy configuration to the Sensing Entity/Entities.

-	Report the Sensing Result and contextual information to the NEF.

-	User Plane Sensing Function (SF-U):

-	Calculate the Sensing Result.

-	Report the Sensing Result to the SF-C.

# * * * * End of changes * * * *