3GPP TSG-WG SA2 Meeting #169	S2-2504890

Fukuoka, Japan, May 19 - 23, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	Clause 4.5.6: Update on NRF for ADM

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: This pCR updates the NRF description clause to include the new NF, ADM.

1. Introduction

In clause 4.5.6 of TS 23.369, NRF updates have been described. In the clause, the updates of introducing AIOTF have been included.

However, another new NF, ADM, has been introduced as well. It is essential to have the NRF updates, so that the ADM discovery and selection can be performed properly.

[Proposal-1] Update NRF description due to the introduction of ADM.

In clause 5.3.2 of TS 23.369, the factor of discovery or selection of AIoT device profile can be:

-	The AIoT device permanent ID or domain information.

To support the discovery based on domain information, the supported domain information of AIoT device permanent IDs need to be included in the NF profile.

To support the discovery based on AIOT device permanent ID, the supported AIoT device permanent ID range needs to be included in the NF profile, as SUPI range in UDM. It will overload NRF if individual AIoT device permanent IDs are included.

In clause 5.3.2 of TS 23.369, the factor of discovery or selection of AF authorization data can be:

-	The AF ID.

To support the discovery based on AF ID, the supported AF IDs need to be included in the NF profile.

[Proposal-2] Include the above information in the NF profile of ADM.

In the description, the NF profile of AIOTF is described as “The NF profile includes the AIOTF ID or IP address, NF type and the serving area information of the AIOTF.”

Below is the description of clause 6.2.6.2 of TS 23.501:

NF profile of NF instance maintained in an NRF includes the following information:

-	NF instance ID.

-	NF type.

-	PLMN ID in the case of PLMN, PLMN ID + NID in the case of SNPN.

-	Network Slice related Identifier(s) e.g. S-NSSAI, NSI ID.

-	FQDN or IP address of NF.

……

According to our understanding, it is not correct to state AIOTF ID or IP address should be included. AIOTF ID, as well as FQDN or IP address should be included.

[Proposal-2] Fix the description of AIOTF.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369 v0.3.0:

* * * Start of Change (ALL TEXTS ARE NEW) * * * *

### 4.5.6	NRF

In addition to the functions defined in TS 23.501 [3], the NRF performs the following functions:

-	Support of new NF type AIOTF and its corresponding NF profile. The NF profile includes the AIOTF ID, FQDN or IP address, NF type and the serving area information of the AIOTF.

-	Support of AIOTF discovery based on parameters such as target area information.

-	Support of new NF type ADM and its corresponding NF profile. The NF profile includes the ADM ID, FQDN or IP address, NF type, the supported domain information of AIoT device permanent IDs, the supported AIoT device permanent ID ranges, and optionally the supported AF IDs of the ADM.

-	Support of ADM discovery based on parameters such as the domain information of AIoT device permanent ID.

* * * End of Change * * * *