SA WG2 Meeting #169	S2-2505025

19 - 23 May, 2025, Fukuoka, Japan

Source: 	Xiaomi

Title: 	Discussion on signalling feasibility of dataset and parameter sharing

Document for: 	Discussion/Approval

Agenda Item: 	19.50

Work Item / Release:	NR_AIML_air-Core

Abstract of the contribution: Discusses the RANs proposal for signalling feasibility of dataset and parameter sharing.

1	Introduction

In Rel-19, two-sided AI/ML CSI compression for AI/ML Air Interface is under study, during RAN1 discussions, the sharing of model parameters and/or datasets from the network side to the UE or UE-side via standardized signaling was considered. RAN1 requested feedback from RAN2 on the feasibility of standardized signaling (over-the-air and/or other approaches) for the network side to share model parameters and/or datasets with the UE or UE-side.

In [2], RAN2 provided some high-level principles and potential solutions to support the network side's sharing of model parameters and/or datasets with the UE or UE-side. These solutions include OTA (over-the-air) solutions and non-OTA solutions. Before provide the final answer to RAN1, RAN2 needs the confirmation from RAN3/SA2/SA3/SA5 regarding the assumptions on non-OTA solutions.

In this paper, we aim to discuss the feasibility of non-OTA solutions from the SA2 perspective.

2	Discussion

## 2.1	SA2 discussion scope

Based on RAN2’s analysis in [2] below, SA2 should only focus on the following two options in the non-OTA approaches:

- Option 2: CN -> UE-side training entity,

- Option 3: gNB -> OAM/CN -> UE-side training entity

to confirm RAN2’s assumption on whether this option can be supported within Rel-19 existing architecture framework.

For non-OTA approaches, i.e., ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidate solutions (see below Table 1). From RAN2 point of view, it is assumed they can be supported within Rel-19 existing architecture framework. RAN3, SA2, and SA5 can further confirm the above RAN2 assumption. Furthermore, it does not preclude RAN3/SA2/SA5 to identify other candidate solutions beyond options listed below. Table 1. non-OTA candidate solutions Option; Impacted WG; Specification impact/Implementation impact. OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity; SA5, SA3; Up to SA5 (any intermediate node between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion). CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity; SA2, SA3; Up to SA2 (any intermediate node between CN and UE-side OTT server is up to SA2). gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity; RAN3, SA2, SA5, SA3; Up to RAN3, SA2, SA5 (any intermediate node between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5).

It should also be noted how the gNB/CN collects related data is not in the LS discussion scope. SA2 should focus on the feasibility of the corresponding dataset/model parameter transfer from architecture framework point of view.

Proposal 1:	SA2 only focus on the feasibility of the data transfer from CN -> UE-side training entity or gNB -> CN -> UE-side training entity, and confirm RAN2’s assumption on whether it can be supported within Rel-19 existing architecture framework.

Considerations for transfer of the dataset/model parameters

Based on the background information in the LS and RAN1/2 discussions, the dataset and model parameters are provided to the UE or UE-side training entity, and collected by the gNB side/CN.

AN2 discussed the candidate solutions and feasibility of standardized signaling for NW-side sharing dataset/model parameter to UE or UE-side training entity for below options: - Dataset sharing consisting of {(Target CSI, CSI feedback)} - Encoder parameter sharing - Encoder parameter sharing + dataset sharing consisting of {target CSI}

According to the principles outlined by RAN2, the dataset and model parameters should be readable and controllable by the network, with relaxed latency requirements. The size may typically be around hundreds of megabytes, all of which can affect how data is transferred.

Dataset/model parameter transfer solution from NW to UE training entity shall follow below principles: - A1 - Size: From RAN2 point of view, RAN2 aims to define a unified solution (e.g. OTA, non-OTA, or a combined) to support various sizes of dataset/model parameter transfer (dataset and/or parameter sharing size could range from tens of KBs to hundreds of MBs, but in average around hundreds of MBs); - A2 - Continuity: Service continuity of dataset and/or parameter transfer/delivery during UE mobility needs to be supported; - A3 - Controllability: NW decides on if and when to transfer/delivery the dataset and/or model parameter from NW to UE or UE training entity (a server inside MNO or an OTT server); - A4 - Latency: Relaxed latency requirement and infrequent update; - A5 - Visibility: dataset and model parameter to be understandable by UE/UE-side training entity (a server inside MNO or an OTT server). - A6: Proprietary information of the network and UE should be respected and not disclosed.

Observation 1:	The following characteristics for data transfer should be considered within the scope of SA2 discussions: Average data size in the hundreds of MBs, Network-controlled data transfer, Continuity, Relaxed latency requirements and MNO Visibility.

## 2.2	Feasibility analysis on data transfer from CN -> UE-side training entity

In general, from the SA2 perspective, the requirement for data transfer from CN -> UE-side training entity is to transfer the AI data from CN to a server inside MNO or an OTT server. Within Rel-19 existing architecture framework, there is trusted AF and untrusted AF, which can cover the case of a server inside MNO or an OTT server.

- For option 2a, where data transfer from CN to a trusted AF which can be understood as server inside MNO or and trusted OTT server, the CN NF can exchange data with trusted AF directly. There are existing examples for data exchange between NWDAF and AF, or Data Collection Coordination is supported by a DCCF or an NWDAF.

- For option 2b, where data transfer from CN to an untrusted AF which can cover untrusted OTT server, the CN NF can exchange data with with untrusted AF via NEF. The other logic is similar as option 2a.

| Non-OTA approaches | Feasibility | SA2 impacts |
| --- | --- | --- |
| 2a: CN -> trusted AF(server inside MNO or trusted OTT server) | It is architectural feasible to support the transfer of dataset/model parameters from the CN to an OTT server. The CN NF can exchange data with trusted AF directly. | No architecture impact For data transfer: Stage 2 impact with the details pending to further discussion For data collection, it can be further discussed in cooperate with RAN groups if any |
| 2b: CN -> untrusted AF(untrusted OTT server) | It is architectural feasible to support the transfer of dataset/model parameters from the CN to an OTT server. The CN NF can exchange data with untrusted AF via NEF. | Same as option 2a with additionally NEF involved |

Observation 2:	it is architectural feasible to support data transfer from CN -> UE-side training entity within Rel-19 existing architecture framework from the SA2 perspective, but the details need to be studied.

## 2.3	Feasibility analysis on data transfer from gNB -> CN -> UE-side training entity

In general, from the SA2 perspective, the requirement for data transfer from gNB -> CN -> UE-side training entity is to transfer the AI data to from gNB to CN, and then from CN to UE-side training entity.

- For transfer the AI data to from gNB to CN, there are existing examples to collect data from RAN, e.g. the NG-RAN location reporting, LMF obtaining Network Assistance Data from NG-RAN, similar mechanisms can be reused. This also requires cooperation with RAN3, please refer to [3].

- For the AI data to from CN to UE-side training entity, it is covered in clause 2.2.

| Non-OTA approaches | Feasibility | SA2 impacts |
| --- | --- | --- |
| 3a: gNB -> CN -> trusted AF(server inside MNO or trusted OTT server) | It is architectural feasible to transfer the dataset/model parameters from the gNB -> CN -> a server inside MNO, as the gNB can transfer the data to CN with signaling enhancement on NG interface, and the CN can then transfer the data to trusted AF. | No architecture impact For data transfer: Stage 2 impact with the details pending to further discussion For data collection, it can be further discussed in cooperate with RAN groups if any |
| 3b: gNB -> CN -> untrusted AF(untrusted OTT server) | It is architectural feasible to transfer the dataset/model parameters from the gNB -> CN -> an OTT server, as the gNB can transfer the data to CN with signaling enhancement on NG interface, and the CN can then transfer the data to untrusted AF. | Same as option 3a with additionally NEF involved |

Observation 3:	it is architectural feasible to support data transfer from gNB -> CN -> UE-side training entity within Rel-19 existing architecture framework from the SA2 perspective, but the details need to be studied.

Proposal 2:	SA2 reply to RAN1 and RAN2, cc RAN3, SA3, SA5 confirm RAN2’s that it is architectural feasible to support data transfer from CN -> UE-side training entity or gNB -> CN -> UE-side training entity within Rel-19 existing architecture framework.

3	Conclusion

Proposal 1:	SA2 only focus on the feasibility of the data transfer from CN -> UE-side training entity or gNB -> CN -> UE-side training entity, and confirm RAN2’s assumption on whether it can be supported within Rel-19 existing architecture framework.

Observation 1:	The following characteristics for data transfer should be considered within the scope of SA2 discussions: Average data size in the hundreds of MBs, Network-controlled data transfer, Continuity, Relaxed latency requirements and MNO Visibility.

Observation 2:	it is architectural feasible to support data transfer from CN -> UE-side training entity within Rel-19 existing architecture framework from the SA2 perspective, but the details need to be studied.

Observation 3:	it is architectural feasible to support data transfer from gNB -> CN -> UE-side training entity within Rel-19 existing architecture framework from the SA2 perspective, but the details need to be studied.

Proposal 2:	SA2 reply to RAN1 and RAN2, cc RAN3, SA3, SA5 confirm RAN2’s that it is architectural feasible to support data transfer from CN -> UE-side training entity or gNB -> CN -> UE-side training entity within Rel-19 existing architecture framework.

4	Reference

[1]  R2-2500015 (R1-2410922), LS on signalling feasibility of dataset and parameter sharing, RAN1

[2]  S2-2504512(R2-2503169), Reply LS on signalling feasibility of dataset and parameter sharing, RAN2

[3]  R3-253380, (Draft LS out) Reply LS on signalling feasibility of dataset and parameter sharing, RAN3