3GPP TSG-SA WG2 Meeting #169	S2-2505187

Fukuoka, Japan, May 19–23, 2025

Source:	NEC

Title:	KI3 New solution: UP path adjustment based on energy related information

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes new solution for UP path adjustment based on energy related information for KI#3

1. Introduction

This contribution proposes a new solution for KI#3 for the Rel-20 FS_EnergySys_Ph2 study

-	Whether and how to perform UP path adjustment based on energy related information, including:

-	Whether and how to trigger UP path adjustment.

-	What information is to be considered for UP path adjustment.

2. Proposal

It is proposed to agree the following changes to 3GPP TR23.700-67.

* * * Start of change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |
| --- | --- | --- | --- |
|  | 1 | 2 | 3 |
| X |  |  | X |

* * * End of change * * * *

* * * Start of change * * * *

6.X	Solution #X: UP path adjustment based on energy related information

6.X.1	Description

This solution applies to Key Issue #3.

As noted in clause 4.2.18.1, TS23.501, the EIF requests energy related information of the targeted UPF(s) from OAM by creating management objects defined in TS 28.622 [x] using procedures defined in TS 28.532 [x]. It is expected to have the report as stream data, file-based or threshold crossing etc. Based on the service, this solution addresses the kye issue #3 for the UP path adjustments for the UE based on the energy attributes and energy related information from EIF.

6.X.2	Procedures

- As noted in clause 4.2.18.1, TS23.501, the EIF requests energy related information of the targeted UPF(s) from OAM by creating management objects defined in TS 28.622 [x] using procedures defined in TS 28.532 [x]. It is expected to have the report as stream data, file-based or threshold crossing etc.

- In the PDU establishment, SMF retrieves the subscription data on UDM in Clause 4.3.2, TS23.502 [3]. The subscription may include information that indicates if the subscriber allows the operator to use energy efficient resource or not. If allowed, the network operator is allowed to use energy efficient network resources. The information may, for example, indicate that the subscriber allows the network operator to use energy efficient network resource when the network is in a condition of off-peak (e.g. workload become less such as in night-time.).

- The SMF invokes Neif_EventExposure_Subscribe to be notified with energy related information from the discovered UPF(s).

- The EIF computes energy related information based on the reported energy consumption data.

- The EIF invokes Neif_EventExposure_Notify to report the outcome of the computation. As described in clause 5.51.2.2.3, TS23.501[2], it is expected to report UPF energy consumption and UPF data volume. The reported UPF energy consumption and UPF data volume may optionally be stored by SMF.

- In order to optimize the energy saving and efficiency whilst maintaining a high service quality along with the time-of-the day, SMF determines to initiate UPF relocation.

- Based on the reported or stored UPF energy consumption and UPF data volume, SMF selects targeted UPF(s) for UPF relocation.

- The same procedures in clause 4.3.5.1, TS23.502[3], can be reused to relocation UPF.

Figure 6.x.2-1: UP path adjustments for the UE based on energy related information

6.X.3	Impacts on services, entities and interfaces

SMF:

- Determine when UPF relocation should be initiated

- Selects the UPF based on energy related information

UDM:

- Support information that indicates if the subscriber allows the operator to use energy efficient resource or not

* * * End of change * * * *