3GPP TSG-SA2 Meeting #169	S2-2505327

Fukuoka, Japan, May 19th – 23rd 2025

Source:	Interdigital

Title:	KI#2, New Sol: UE initiated sensing service request

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The UE initiated sensing service request procedure is proposed.

# 1. Introduction

This pCR proposes the solution for Key issue#2 service authorisation and revocation.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  | X |  |  |  |  |

# * * * * Second change (All text new) * * * *

## 6.X	Solution #X: UE initiated sensing operation

### 6.X.1	High-level solution Principles

The UE may initiate a sensing service request as a sensing service consumer. Whether the UE is allowed to act as a sensing service consumer may be based on UE’s subscription data. When UE is allowed to act as a sensing service consumer, the sensing result may be consumed by UE itself or by another sensing service consumer.

### 6.X.2	Description

This solution addresses key issue #2 for the case where the UE requests a sensing service as a sensing service consumer.

When UE sends a sensing service request, it may be delivered to the AMF via UL NAS Transport. When the AMF receives a sensing service request from a UE, it is forwarded to a (authorizing) sensing NF. The Sensing NF may authorize the sensing service request from a UE based on UE’s subscription data and availability of sensing operation at a target sensing service area which is determined based on UE’s location or UE’s requested sensing service area.

As a result of a periodic sensing report or event triggered sensing report, UE may initiate a sensing service request, and in this case the associated identifier for the pending service request may be included in the request.

The architecture and functional split are based on the solution in S2-2505326.

### 6.X.3	Procedure for sensing service

Figure 6.X.3-1 Procedure of UE initiated sensing service request

1.	The UE sends a Service Request for Sensing in a UL NAS Transport message. The Service Request for Sensing may indicate a specific type of sensing service request (e.g., object detection and tracking, environmental detection, motion detection, etc), time information for requested sensing operation, UE’s ID, and target region information in which the sensing is to be executed or UE’s location information. The service request may include specific requirements for the sensing service, e.g. sensing accuracy, latency, sensing frequency, resolution, etc.

If the service request is related to a pending sensing service (e.g., periodic sensing service, event triggered sensing service), it may include a transaction ID associated with the pending service.

The UE may include serving sensing NF information, if it was received before for the pending sensing service.

Additionally, the UE may include sensing service consumer information as a recipient of sensing result if UE is not the sensing service consumer.

2.	If needed, the AMF may acquire UE’s location information by other procedures.

Step 2 may be performed by a sensing NF after the service request for sensing is received from AMF.

3.	The AMF may send a sensing service request to an (authorizing) sensing NF.

If there is no dedicated sensing NF for authorization, AMF may discover a sensing NF to serve the sensing service area including UE’s location and send the sensing request to the sensing NF.

If a sensing NF’s information is received in step 1, AMF may send a sensing request to the sensing NF.

4.	Based on UE’s subscription data, the requested sensing service may be authorized, and the sensing NF may determine the sensing mode for the requested sensing service and verify whether the sensing mode may satisfy the service request for sensing with QoS requirements.

After authorization of the sensing request, the sensing operation may be coordinated by a (distributed) sensing NF which serve the target sensing service area. In this case, a (authorizing) sensing NF may discover and select a (distributed) sensing NF and send a sensing request to the (distributed) sensing NF.

5.	After the sensing service is authorized, a (distributed) sensing NF may select sensing entities for sensing operation at the target sensing service area and target sensing mode. Sensing entities may be selected based on their registered capability. The distributed sensing NF may provide the candidate sensing entities to the (distributed) sensing NF.

6.	A (distributed) sensing NF sends a sensing request to the selected sensing entities

A sensing request message may include the requested sensing mode, time for sensing operation, list of sensing entities, and any configuration information for coordinating sensing operation. QoS

Editor’s Note: the content of the configuration information will be determined in collaboration with RAN WGs.

7.	Based on the coordination of the (distributed) sensing NF, the sensing entities collect sensing measurement data.

8.	The collected sensing measurement data is sent to the (distributed) sensing NF (via CP or via UP)

9.	The (distributed) sensing NF may calculate the sensing result using collected sensing measurement data.

10-11.	The sensing NF may send the result to the sensing service consumer (to UE, or to an indicated sensing service consumer received in UE’s service request for sensing) or to a sensing NF based on the indicated transaction ID in the UE’s service request.

### 6.X.4	Impacts on services, entities and interfaces

UE, AF (as a sensing service consumer):

-	Support service request for sensing procedure.

Sensing NF:

-	Support sensing service authorization and coordination of sensing operation with sensing entities

Sensing entities:

-	Support a sensing operation by coordination of sensing NF.

# * * * * End of changes * * * *