SA WG2 Meeting #169	S2-2505398

Fukuoka, Japan, May 19 – May 23, 2025	(revision of S2-25xxxxx)

Source:	CATT

Title:	KI #3, New Sol: Discovery and (Re-)selection of SF and Sensing Entity

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: Introduce a new solution to key issue#3 in TR 23.700-14.

1. Introduction

This pCR is proposed to add a new solution to key issue#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection.

2. Proposal

It is proposed to add the following changes in TR 23.700-14.

* First change *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | KI#3 |  |  |  |
| #X | x |  |  |  |

* Second change (all text new)*

6.X	Solution #X: Discovery and (Re-)selection of SF and Sensing Entity

6.X.1	High level principles

The high level principles of the solution include:

-	SF discovery and selection: NEF or AF discovers SF via NRF.

-	Sensing Entity Discovery and (Re-)Selection:

-	SF obtains the Sensing Entity information from OAM and/or Sensing Entity.

-	SF is responsible to (re-)select Sensing Entity.

6.X.2	Description

6.X.2.1	SF Discovery and Selection

SF selection functionality is supported by the NEF and AF to determine a SF for sensing result.

The NEF or AF utilizes the NRF to discover SF instance(s) by providing to the NRF the target sensing service area. The NRF provides SF instance(s) to the NEF or AF. The NEF or the AF selects the SF instance(s) based on the available SF instance(s) obtained from the NRF.

6.X.2.2	Sensing Entity Discovery and (Re-)Selection

The SF is responsible to select the Sensing Entity(ies).

The SF obtains the following Sensing Entity information from OAM and/or Sensing Entity.

-	Sensing Entity identifier.

-	Supported sensing service area.

-	Location of Sensing Entity.

-	Supported sensing operation(s).

When the SF receives sensing request including target sensing service area and sensing QoS from NEF or trusted AF. Based on the received target sensing service area, sensing QoS and the Sensing Entity information, the SF selects Sensing Entity(ies).

When the SF detects the sensing target moves out of the sensing service area of a Sensing Entity or the sensing measurement fails, the SF re-selects the Sensing Entity(ies).

6.X.3	Procedures

6.X.3.1	SF profile maintenance in NRF

SF invokes the NF service Registration procedure specified in clause 4.17.1 in TS 23.502[] to inform the NRF of its NF profile. The SF provides the sensing service area to NRF.

SF invokes the NF service update procedure specified in clause 4.17.2 in TS 23.502[] to inform the NRF of its updated NF profile (e.g. with updated sensing service area).

SF invokes the NF service deregistration procedure specified in clause 4.17.3 in TS 23.502[] to inform the NRF of its unavailability when e.g. it’s about to gracefully shut down.

6.X.3.2	SF discovery via NRF

The NEF or AF invokes the NF/NF service discovery by NF service consumer in the same PLMN in clause 4.17.4 in TS 23.502 to discover SF.

6.X.3.3	Sensing Entity Registration in SF

If Sensing Entity is not configured with SF information or the direct communication between Sensing Entity and SF is not support, the procedure in Figure 6.X.3.3-1 is performed to register the Sensing Entity information in SF.

Figure 6.X.3.3-1: Sensing Entity Registration Procedure

1.	The Sensing Entity sends the Sensing Entity Registration Request (Sensing Entity identifier, Supported sensing service area, Location of Sensing Entity, Supported sensing operation(s)) to AMF. This message can be encapsulated in a UL NAS TRANSPORT or N2 message.

2.	The AMF selects SF via NRF.

3.	The AMF invokes Namf_Sensing_MessageDelivery Request to SF. The SF stores the information of Sensing Entity.

4.	The SF invokes Namf_Sensing_MessageDelivery Response to AMF. The service operation includes the Sensing Entity Registration Response.

5.	The AMF sends the Sensing Entity Registration Response to Sensing Entity. This message can be encapsulated in a DL NAS TRANSPORT or N2 message.

If the direct communication between Sensing Entity and SF is supported and the Sensing Entity is configured with SF information, to register Sensing Entity in the SF, only the step 1 and step 5 in Figure 6.X.3.3-1 are performed between Sensing Entity and SF directly.

6.X.4	Impacts to Services, Entities and Interfaces

NRF:

-	Support SF service registration, update and deregistration.

-	Support SF discovery and selection.

NEF: Support SF discovery and selection.

AF (trusted): Support SF discovery and selection.

SF:

-	Support Sensing Entity registration.

-	Support Sensing Entity discovery and selection.

AF:

-	Support SF discovery and selection.

-	Support sensing message delivery between Sensing Entity and SF.

Sensing Entity: Support Sensing Entity registration to SF.

* End of changes *