3GPP TSG SA WG2 Meeting #169	S2-2505413

Fukuoka, Japan, 19-23 May, 2025	(revision of S2-220xxxx)

Source:	Xiaomi

Title:	New solution for KI#2 Authorization and revocation based on sensing service conditions

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing-ARC / Rel-20

Abstract: This paper proposes a new solution for KI#2.

1. Discussion

A new key issue on Authorization and Revocation to Support Sensing Service was added as key issue #2 in TR 23.700-14. This pCR proposes a new solution to address this key issue.

2. Proposal

The following changes are proposed for TR 23.700-14.

* * * Start of Change (ALL NEW TEXT) * * *

6.X	Solution #X: Authorization and revocation based on sensing service conditions

6.X.1	Description

This solution addresses Key Issue #2 "Authorization and Revocation to Support Sensing Service".

Unlike the legacy mobile communication services which the operators strive to provide to its subscribers anytime anywhere, most of the use cases of sensing services described in TR 22.837 [x] are provided with some case specific conditions, e.g. specific flight route for UAV sensing, specific sensing geographical area/zone, specific time or duration for sensing, specific owner for in-building sensing, etc. These are also captured in a note (in terms of time and space) for configuration and authorization requirements in TS 22.137 [2] clause 5.2.2 and the first requirement for network exposure in TS 22.137 [2] clause 5.2.3 to meet specific requested conditions.

To ensure that specific service conditions are met, this solution proposes that authorization is performed at two stages:

- The first stage is after receiving sensing service request and before the sensing service is triggered in the network;

- The second stage is during the sensing service operation. If the authorization at this stage fails, the on-going sensing service needs to be revoked.

The criteria for sensing service request authorization and sensing operation revocation are based on the conditions of specific sensing services.

6.X.2	Procedures

6.X.2.1	Before triggering sensing service operation

Sensing service request is sent from a sensing service consumer (e.g. a 3rd UAV operator or UTM) to an operator network. Once the NF responsible for service exposure (e.g. NEF) receives a sensing service request, it needs to authorize the service request by checking whether the sensing service consumer is allowed to access the requested sensing service. For this check, it is proposed that the NEF needs to do the following, in order to meet different conditions of specific sensing services:

- The check on condition compliance needs to be performed by the NEF, as long as the received service request contains the information implying specific conditions (e.g. a flight route, an area, a duration, a houseowner, etc.).

- A service profile (i.e. the service-level information containing specific conditions) is configured in a NF (e.g. UDR or CCF), from which the NEF can retrieve for the check. The service profile can also be locally configured in the NEF.

Editor’s Note: If the NEF supports CAPIF framework for authorization, the details of CAPIF-based authorization is to be addressed by SA WG3.

Even if the service profile contains sufficient information for the condition check, in order to protect the interests or privacy of the owner of the sensing objects (e.g. house for intruder detection, body for sleep monitoring, or factory of AMR collision avoidance), check with the owner may still be needed to ensure the service request sent by the sensing service consumer is initiated by the owner or the user authorized by the owner. Hence, it is proposed that:

- The NEF may need to obtain the permission/consent from the owner of the sensing object, as long as the information of user or owner is contained in the service request explicitly or implicitly.

- If the owner is a subscriber of the network receiving the service request, the NEF may trigger the network to obtain the permission/consent from the subscriber via the UE. For example:

- The NEF obtains the subscription information or consent information of the subscriber/owner from the UDM, or

- The NEF obtains the permission/consent via the UE of the subscriber/owner by establishing direct connection with the UE, or

- The NEF requests another NF (e.g. AMF) to establish connection with the UE to obtain the owner’s permission/consent.

- If the owner cannot be mapped to a subscriber of any mobile network, an owner list or allowed user list associated with the sensing object needs to be configured in the service profile for the NEF to retrieve.

Editor’s Note: The details of user consent mechanism is to be addressed by SA WG3.

Editor’s Note: The details of authorization of owner or user is to be addressed by SA WG3.

6.X.2.2	During sensing service operation

After the sensing service is triggered by the network, the condition compliance still needs to be checked by the NF (e.g. SF) responsible for selecting the sensing entities or controlling sensing activation/deactivation during the sensing service operations. Hence, it is proposed that:

- The SF needs to store the information implying service conditions (e.g. a flight route, an area, a duration, a houseowner, etc.) once received from the service request.

During the initial sensing entity selection phase, it is proposed that:

- The SF selects sensing entities based on the condition information, e.g. base stations within the requested area, etc.

As the sensing object can be moving (e.g. the flying UAV), sensing entities may need to be reselected due to the object’s movement. It is proposed that:

- The SF needs to be aware of the trajectory of the moving object, e.g. notified by the sensing entities or requested from the sensing entities.

- The SF needs to check the condition information when reselecting the sensing entities while the sensing object is moving.

NOTE x:	For example, during the flying course, if a UAV is detected to have left the coverage of an old base station (as the old sensing transmitter/receiver) and entered into the coverage of a new base station, the old base station needs to stop radio sensing and the new base station needs to be selected for radio sensing. If the flight route is received and stored by the SF, the SF needs to check the flight route to reselect the base station one after another along the flight route.

- The SF needs to revoke the ongoing sensing operation when the check of condition compliance fails.

NOTE y:	For example, if the SF detects that a flying UAV enters into the coverage of a new base station residing away from the flight route, the SF needs to deactivate the sensing operation to avoid serving the UAVs flying in an unpermitted route.

6.X.3	Impacts on Services, Entities and Interfaces

NEF:

-	Need to retrieve service profile containing service specific conditions to authorize the service request

-	Need to trigger the network to obtain the permission/consent from the subscriber via the UE

SF:

-	Need to store the information implying service conditions

-	Need to select or reselect sensing entities based on service conditions

-	Need to revoke ongoing sensing operation when the check of condition compliance fails

NG-RAN:

-	No impact.

* * * End of Change * * *