3GPP TSG-WG2 Meeting #170	S2-2506344

25 - 29 August, 2025, Göteborg, Sweden	(revision of S2-25xxxxx)

Source:	Ericsson

Title:	KI#4, new solution: Direct Sensing Association between gNB and SnCF

Document for:	Discussion and approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution:

This contribution proposes a solution to establish a direct association (sensing association) between gNB and SnCF.

# 1. Discussion

This document proposes a new solution for KI#4 to establish a direct association (sensing association) between gNB and SnCF used for sending controlling and data messages without AMF involvement. The solution also has impact on KI#1 as defining a new NF for sensing and on KI#3 since sensing association establishment between gNB and SnCF may initiate NF profiles update of SnCF in NRF.

The proposal lists method on how gNB can initiate, update and if necessary, remove the sensing association.

# 2. Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-14.

***** Start of Changes *****

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | X |  | X | X | X |  |
| #2 |  | X |  |  | X |  |
| #3 | X | X | X | X | X | X |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  | X |  |  |
| #6 | X | X | X |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |
| #10 |  | X |  |  |  |  |
| #11 |  | X |  |  |  |  |
| #12 |  | X |  |  |  |  |
| #13 |  |  | X |  |  |  |
| #14 |  |  | X |  |  |  |
| #15 |  |  | X |  |  | X |
| #16 |  |  | X |  |  |  |
| #17 |  |  | X |  |  |  |
| #18 |  |  |  | X |  |  |
| #19 | X | X |  |  | X |  |
| #20 |  |  |  |  |  | X |
| #X | X |  | X | X |  |  |

***** Next Change (all new text) *****

## 6.X	Solution #X: Direct Sensing Association Establishment between gNB and SnCF

### 6.X.0	High-level solution Principles

The solution is based on following principles:

- Direct association (sensing association) is established between gNB and SnCF without AMF involvement.

- Establishment is initiated by gNB.

- Direct sensing association is used to send controlling messages and to receive sensing-related result/data

- This solution is agnostic to the presence of a transport connection.

### 6.X.1	Functional Description

gNB initiates establishment of sensing association between gNB and SnCF based on provisioned OAM information (e.g.: direct or indirect identity of SnCF). Sensing association is used to exchange controlling messages between SnCF and gNB, as well as sensing-related measurements (result) from gNB to SnCF, without involving AMF.

### 6.X.2	Procedures

#### 6.X.2.1	gNB initiated establishment of sensing association

gNB is provisioned with SnCF related information and initiates the sensing association establishment to SnCF. This procedure can be done by pre-defined serving sensing area or dynamic serving sensing area.

##### 6.X.2.1.1	gNB initiated establishment of sensing association with pre-defined serving sensing area.

In this setup, the SnCF is provisioned with serving sensing area that it serves, this information is used to update the SnCF’s profile in NRF.

Figure 6.X.2.1.1-1 describes the procedure for serving sensing area registration in NRF.

Figure 6.X.2.1.1-1: Serving sensing area registration in NRF

- SnCF is configured with the sensing area(s) that it serves.

- SnCF sends Nnrf_NFManagement_NFUpdate Request message to NRF to update its NF Profile related to serving sensing area.

The SnCF NF profile in NRF should include (among other parameters):

  - Serving Sensing Area(s)

NOTE: Exact composition of Serving Sensing Area is FFS

- The NRF acknowledges that NF Update is accepted via Nnrf_NFManagement_NFUpdate response.

Figure 6.X.2.1.1-2 describes sensing association establishment between gNB and SnCF.

Figure 6.X.2.1.1-2: gNB initiates the sensing association towards SnCF

- gNB is configured with the covered sensing area(s). Furthermore, the gNB is either configured with the SnCF’s address or it obtains the SnCF’s address via DNS interaction.

- gNB sends Sensing Association Setup Request to the SnCF for the sensing service. It may include (among other parameters) the covered sensing area(s) and gNB address, which is stored by SnCF.

- SnCF sends an acknowledgement to the gNB to indicate the successful establishment of the sensing association.

- The sensing association is used to exchange sensing controlling messages between the gNB and the SnCF, as well as to provide the sensing-related data/result from the gNB to the SnC

NOTE: Sensing measurement reporting is outside of the proposal.

##### 6.X.2.1.2	gNB initiated establishment of sensing association with dynamic serving sensing area

In this setup, the SnCF updates the serving sensing area in NRF profile, dynamically based on the sensing associations to gNBs.

Figure 6.X.2.1.2-1: gNB initiating the sensing association towards SnCF

0-2: Same as steps 0-2 in 6.X.2.1.1-2.

3: [Conditional] If after the new sensing association establishment NF profile update is required due to the change of serving sensing area, SnCF sends Nnrf_NFManagement_NFUpdate Request message to NRF to update its NF Profile related to serving sensing area.

4: [Conditional] The NRF acknowledges NF Update is accepted via Nnrf_NFManagement_NFUpdate response.

5: Same as steps 3 in 6.X.2.1.1-2.

#### 6.X.2.2	 Sensing Configuration Update

##### 6.X.2.2.1	Configuration Update by gNB

In this procedure gNB updates the configuration of the sensing association.

Figure 6.X.2.2.1: gNB updates the configuration of the sensing association

- gNB sends Sensing Configuration Update Request to the SnCF for the sensing service if e.g., the covered sensing area(s) of gNB has been changed. SnCF updates its stored sensing area(s) if it is required.

- SnCF sends an acknowledgement to the gNB to indicate the successful configuration update.

- [Conditional] If the serving sensing area of the SnCF has changed due to the update from gNB, the SnCF sends the Nnrf_NFManagement_NFUpdate Request message to NRF to update its NF Profile related to serving sensing area.

- [Conditional] The NRF acknowledges NF Update is accepted via Nnrf_NFManagement_NFUpdate response.

NOTE. Step 3 and 4 are not required for that option when sensing area is pre-defined in SnCF.

##### 6.X.2.2.2	Configuration Update by SnCF

In this procedure SnCF updates the configuration of sensing association.

Figure 6.X.2.2.2: SnCF updates the configuration of the sensing association

- SnCF sends Sensing Configuration Update Request to the gNB for the sensing service.

- gNB sends an acknowledgement to the SnCF to indicate the successful configuration update.

Editor’s Note: Weather this procedure is needed or not will be evaluated together with RAN3 WG.

#### 6.X.2.3	 Sensing Association Removal

gNB may decide to remove the sensing association.

Figure 6.X.2.3-1: gNB removes the sensing association (dynamic NRF update)

- gNB sends a Sensing Association Removal request to the SnCF. SnCF updates its stored serving sensing area if it is required.

- SnCF sends an acknowledgement to the gNB about the removal.

- [Conditional] If the removal changes the serving sensing area of the SnCF and an NF profile update is required, SnCF sends Nnrf_NFManagement_NFUpdate Request message to NRF to update its NF Profile related to serving sensing area.

- [Conditional] The NRF acknowledges NF Update is accepted via Nnrf_NFManagement_NFUpdate response.

NOTE. Step 3 and 4 are not required for in case when serving sensing area is pre-defined in SnCF.

Editor’s Note: this can be further evaluated by RAN WGs.

### 6.X.3	Impacts on services, entities and interfaces

The solution has impacts on the following entities:

gNB:

- Establishing direct sensing association with SnCF.

- Sensing configuration updated message to SnCF.

- Remove direct sensing association with SnCF.

SnCF:

- Establishing direct sensing association with gNB.

- Sending configuration updated message to gNB.

NRF:

- Storing SnCF NF profile including serving sensing area.

***** End of CHANGE *****