SA WG2 Meeting #170	S2-2506443

25 – 29 Aug, 2025, Goteborg , SE

Source:	MediaTek Inc.

Title:	[WT#1.4, Voice Support] Support of voice service

Document for:	Agreement

Agenda Item:	20.6.1.4

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution covers how to support and/or enhance the voice service, based on which a scope for the related (sub-)Work Task (WT) and related Key Issue (KI) are proposed.

1. Justifications

History shows that, for the first wave of new “G” deployments, data service (e.g., internet) is ready, but native voice service over the new “G” is typically not ready on Day 1, lesson learnt including:

A)	voice call initiation aspects:

1.	when LTE was commercially launched, CS Fallback was used for long period of time, before VoLTE become commercially viable;

2.	when 5G SA is commercial ready, often EPS Fallback was used to support voice until VoNR was ready;

3.	for 5G SA ready networks, it is not VoNR everywhere: some networks still trigger EPS Fallback when the NR signal strength is below a certain threshold;

4.	there are still some 5G networks which are not 5G SA ready (i.e., still using 5G NSA and VoLTE);

B)	voice call service continuity aspects:

5.	for a network with bigger LTE coverage than NR coverage, VoNR HO to VoLTE is required to provide voice call service continuity.

Based on these history lesson learnt, we can infer that relying on 6G native voice (Vo6G) from Day 1 is unlikely to be feasible, thus:

-	A transitional phase for 6G voice relying on 5G is needed; and

it is hard to predict whether all networks will:

a)	be upgraded to 5G SA;

b)	become VoNR ready; and

c)	support VoNR everywhere (see bullet A) 3.);

when 6G is commercially ready, thus:

-	A transitional phase for 6G voice relying on 4G is also "likely" needed (i.e., fallback/IWK to VoLTE cannot be pre-excluded at early stage).

1.2 Summary

Architectural Motivation:

Be able to provide MO/MT voice call services, initiated by 6G camped UE and being able to smoothly switch to legacy voice e.g., VoNR and VoLTE, when needed.

Gap Analysis:

-	Native 6G voice (Vo6G);

-	MO/MT voice call initiation for 6G camped UE by 5GS fallback or, optionally EPS fallback; and

-	Handover of ongoing Vo6G session to VoNR or, optionally VoLTE.

Technical Impact: Following impacts are expected to be covered to resolve this gap:

- Procedure flows for Native 6G voice (Vo6G);

- MO/MT voice call initiation procedure flows for 6G camped UE, by 5GS fallback or, optionally EPS fallback; and

- Procedure flows for ongoing Vo6G session handover to VoNR or, optionally VoLTE;

**** First Change ****

Annex A.1.1.X WT#1.4 Scope for 6G voice service

Extracted below from SP-250806:

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

NOTE  1:	The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split.

…

1.4.	Study whether and how to support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS) in 6G.

Note the below update is just intended as information in the TR and no change is expected in the WT wording in the SID.

WT 1.4 focuses on studying whether and how to support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS) in 6G, including at least the following:

How to support IMS voice over PS session, including:

A)	How to support UE domain selection.

B)	How to enable:

1)	Native IMS voice over PS session in 6GS (Vo6G);

2)	IMS voice over PS session via 5GS fallback; and

3)	MS voice over PS session service continuity to 5GS;

C)	Whether and how to enable:

4)	IMS voice over PS session via EPS fallback; and

5)	IMS voice over PS session service continuity to EPS.

Note 1:	Potential impact on Interworking, due to support of some of the services (e.g., voice), may need to be coordinated with WT#2.

Note 2:	The existing IMS reference architecture, as specified in TS 23.228 [XXX] is used as starting point for this WT.

**** Second Change ****

5.X. Key Issue #X: Support of 6G voice service

To provide voice service in 6G, this key issue studies:

A)	How to support UE domain selection.

B)	How to enable:

1)	Native IMS voice over PS session in 6GS (Vo6G);

2)	IMS voice over PS session via 5GS fallback; and

3)	MS voice over PS session service continuity to 5GS;

C)	Whether and how to enable:

4)	IMS voice over PS session via EPS fallback; and

5)	IMS voice over PS session service continuity to EPS.

Note 1:	Potential impact on Interworking, due to support of some of the services (e.g., voice), may need to be coordinated with Key Issue XXXXX.

Note 2:	The existing IMS reference architecture, as specified in TS 23.228 [XXX] is used as starting point for this key issue.

**** End of Changes ****