3GPP SA WG2 Meeting #170	S2-2506826

25th – 29th August 2025, Goteborg, SE

Source:	China Telecom

Title:	[WT#4, Sensing] Scope and key issue on 6G sensing

Document for:	Approval

Agenda Item:	20.6.4

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution intends to give the justifications on the study of integrated sensing and communication in 6G, and proposes the scope and the key issue description.

1. Justifications

This contribution discusses the scope and proposes a corresponding key issue to facilitate work on Work Task #4 of the approved 6G SID.

WT#4: Study the integration of Sensing and Communication over 3GPP access, considering the sensing modes to be supported and other sources of sensing data.

NOTE 5:	The detailed scope of WT#4 will be determined considering the scope and work of R20 FS_Sensing_ARC.

1.1 Architecture Motivation

Integrated Sensing and Communication (ISAC) has been recognized as one of the most promising 6G technologies to extend the network capabilities and provide services beyond communications.

According to ‘Recommendation ITU-R M.2160-0: Framework and overall objectives of the future development of IMT for 2030 and beyond’, the following texts describes the necessity of conducting a comprehensive study on 6G sensing services:

- In IMT-2030, the integration of sensing and communication is expected to become a key enabler for a wide range of use cases. Moreover, sensing the physical surroundings together with appropriate AI could further enhance situational awareness.

- This usage scenario facilitates new applications and services that require sensing capabilities. It makes use of IMT-2030 to offer wide area multi-dimensional sensing that provides spatial information about unconnected objects as well as connected devices and their movements and surroundings.

In addition, the 6G sensing use cases and scenarios and have been comprehensively studied in 3GPP SA1 (see TR 22.870 section 7). The defined 20 use cases can be in general categorized into 3 groups, see the following table with more details:

Table 1: Categorized 6G sensing use cases

| Usage | Examples of use cases from TR 22.870 |
| --- | --- |
| Set#1: multiple sensing targets in wide area | Coordination of search and rescue missions (7.1), Low-altitude UAV supervision (7.4, 7.17), Environment Object Reconstruction (7.5), etc. |
| Set#2: high accuracy sensing | Safety assistance for vulnerable pedestrians (7.2), High-resolution topographical maps (7.3), Enhanced XR User Navigation (7.12), etc. |
| Set#3: sensing capability extension | Sensing data related (7.10 stored, 7.11 credential), non-3GPP sensing data fusion (7.15), 6G wireless sensing via Non-3GPP RAT (7.16), etc. |

Therefore, the 6G sensing architecture shall support all potential usage scenarios arising from sensing-related capabilities, along with the provided communication capabilities.

1.2 Gap Analysis

Rel-20 ISAC SI (see RP-251861) was approved in RAN#108 meeting, it will focus on gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case. Therefore, the final scope of SA2 ongoing Rel-20 study FS_Sensing_ARC (see SP-250833) is gNB-based sensing, aligned with Rel-20 5G-A RAN ISAC study scope.

It makes sense for RAN/SA WGs to consider real marketing requirements to complete the 6G sensing architecture, interface, procedure, protocol design from Day 1. The gaps are mainly reflected in the following aspects:

- Sensing mode: gNB-based monostatic sensing has its limitation on accuracy, therefore cannot meet requirements of 6G sensing use cases. Therefore, 6G sensing architecture should support all potential sensing modes (including gNB-based/UE-based mono-static/bi-static sensing mode, and UE-gNB coordinated sensing mode).

- Sensing signaling control: In Rel-20 FS_Sensing_ARC study, the discussion on the split of sensing control and data processing capabilities into different entities is ongoing, to align with the 6G overall architecture design, the 6G sensing signalling control, especially for the UE involved sensing scenarios, requires further study.

- Sensing data fusion: In addition to 3GPP sensing data, non-3GPP sensing data (e.g. mmWave data and camera data), can be used to help improve the sensing accuracy. Therefore, the fusion and processing of both 3GPP sensing data and non-3GPP sensing data to obtain accurate sensing result shall be studied in 6G.

- Sensing data transfer mechanism: In 5G, the data (data for AI, data for positioning, etc) transfer mechanisms are defined case by case. With the introduction of wide area sensing capabilities in 6G, it is expected to define a unified sensing data transfer mechanisms and the corresponding RAN-CN interface/protocol to reduce the sensing data transmission latency.

1.3 Technical Impact

The potential technical impacts may include the following:

- Overall 6G Core network architecture:

- 6G sensing architecture needs to split sensing signalling control and sensing data processing into different entities with standardized interface between them.

- 6G sensing architecture needs to coordinate with the design of 6G new NAS mechanism for sensing signalling control and the Common Data Framework for sensing data handling.

- Interaction between network services:

- 6G sensing service may integrate with AI service to utilize AI capabilities to improve sensing entity selection, sensing data processing, and sensing-related policy generation.

- Interworking or migration from 5G to 6G:

- The common aspects defined in 5G-A sensing service procedures (e.g. the authorization and revocation of sensing service request, the exposure of sensing result, etc.) can be referred when defining similar procedures for 6G sensing.

**** First Change ****

Annex A.X. WT#X Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

The following aspects for WT#4 will be studied:

- Study how to enhance the architecture and function to support all potential sensing modes, and design the 6G sensing function, protocol, procedure, signalling and data transfer mechanism.

**** Second Change ****

5.X. Key Issue #X: Architecture support for integrated sensing and communication

The following key issues should be studied to support sensing in 6G:

- How to design the architecture and function to support all potential sensing modes, including the mobility of both sensing entities and sensing object.

- How to investigate mechanisms for the authorization and revocation of sensing service requests received from the sensing service consumer (e.g. AF, UE, or both).

- How to support the discovery and selection of sensing entities based on the service requirements triggered by the service request and capability of the sensing entities.

- How to support the sensing signalling control and the sensing parameters configuration mechanisms over 6G new interface and function.

- How to support the sensing data and the associated information collection and transfer mechanisms to support high efficiency sensing data handling.

- How to support the sensing data fusion to obtain accurate sensing result and how to expose the sensing result to the sensing service consumer (e.g. AF, UE, or both).

NOTE 1: 	Privacy protection and other security aspects will be coordinated with SA WG3, and the related impact to architecture enhancement will be based on SA WG3 conclusion.

NOTE 2:	Charging and OAM aspects will be coordinated with SA WG5, and the related impact to architecture enhancement will be based on SA WG5 conclusion.

**** End of Changes ****