3GPP TSG SA WG 2 Meeting SA2#170                                                                  TDoc S2-2506857

Gothenburg, Sweden, 2025-08-25 – 2025-08-29

Title:	DRAFT LS reply on AIML_CN issues

Response to:	LS C3-251653 on LS on AIML_CN issue from CT3

Release:	Rel-19

Work Item:	AIML_CN

Source:	Ericsson (will be SA2)

To:	CT3

Cc:	CT4

Contact person:	Magnus Hallenstål

magnus.l.hallenstal@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-250nnnn, CR#0752 on TS23.273

S2-250nnnn, CR#1506 on TS23.288

S2-250nnnn, CR#5549 on TS23.502

# 1	Overall description

SA2 has discussed the questions in received LS C3-251653 and have arrived at the following conclusions/answers:

Question 1: Can Signalling Storm analytics be subscribed (by the SCP, the NRF, the UDM) via the DCCF (potentially using the MFAF)?

Answer: SA2 confirms that Signalling Storm analytics can be subscribed by the above consumers via DCCF., and thus agreed to add NRF, SCP, UDM as service consumers of the Ndccf_DataManagement and Nmfaf_3caDataManagemen services.

Question 2: Can the DCCF be used to collect data using the Nlmf_DataExposure service?

Answer: SA2 confirms that the DCCF can be used to collect data using the Nlmf_DataExposure service. DCCF has thus been added as service consumer of the Nlmf_DataExposure service.

Question 3: Can information about the used QoS Profile be exposed by the QFI allocation, change, and deallocation events of the Nsmf_EventExposure service?

Answer: Yes, the QoS profilecan be exposed, except information related to notification control.

Question 4: If the answer to Question 3 is "yes", can SA2 clarify which QoS Profile parameters are relevant to be exposed for QoS and Policy Assistance analytics?

Answer: The QoS profile as defined in TS 23.501 in clauses 5.7.1.2 and 5.7.2, except information related to notification control. SA2 has updated the related text in TS 23.502 to clarify that the QoS information exposed by the Nsmf_EventExposure Service of the QFI allocation event is the QoS profile defined in TS 23.501, except information related to notification control.

# 2	Actions

To CT3

ACTION: Based on the answers above, SA2 asks CT3 to take any needed action.

# 3	Dates of next TSG SA WG 2 meetings

SA2#171		2025-10-13 – 2025-10-17		Wuhan, China

SA2#172		2025-11-17 – 2025-11-21		Dallas, USA