3GPP TSG-SA2 Meeting #170	S2-2506314

# Goteborg, Sweden, 25 – 29 August 2025

Source: 	Transsion Holdings

Title: 	KI#1&3&4&5: NewSol: Sensing procedure for the sensing service by request with UE Identity

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This paper proposes a new solution for KI#1&KI#3&KI#4&KI#5 in TR 23.700-14.

.

1	Discussion

This paper proposes a new solution for KI#1&KI#3&KI#4&KI#5 in TR 23.700-14. The solution focuses on enhancement to 5G system to provide sensing service by request with UE Identity.

2. Proposal

It is proposed to add the following contents to TR 23.700-14.

Start of Change

Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | x |  | x | x | x |  |

Next Change (all new text)

6.X	Solution #X: Sensing procedure for the sensing service with UE identity by data transferring via CP (option 1) and UP (option 2)

6.X.1	High level principles

High-level principles of this solution are as below:

a)	Sensing Consumer requests sensing result to the core network by request with UE identity.

b)	GMSENC is introduced as the gateway for AMF discovery.

c)	AMF decides the sensing entiti(es) (i.e. gNB and/or UE) to perform sensing activiti(es).

d)	SENF is introduced for providing commands to AMF and for sensing data processing.

e)	Control plane connection (Option 1) and User plane connection (Option 2) are used for transferring sensing data between the UE/gNB and the core network.

6.X.1	Description

This solution is for Key Issue#1&KI#3&KI#4&KI#5 for sensing procedure for the sensing service with UE identity by sensing data transferring via CP (option 1) and UP (option 2).

Figure 6.X.1-1 illustrates Non-roaming reference architecture for Sensing Services in SBI representation. New NFs, i.e. GMSENC and SENF, are introduced as new network functions to 5G enhanced SBI architecture for providing sensing services.

Figure 6.X.1-1: Non-roaming reference architecture for Sensing Services in SBI representation

NOTE 1: GMSENC, i.e. gateway mobile sensing center, is introduced as sensing gateway for sensing service.

NOTE 2: SENF, i.e. sensing function, is introduced as sensing function.

The Sensing Consumer requests sensing service related to UE (e.g. sensing results of a certain area around a UE) by providing UE identity to the core network via new introduced network function GMSENC. SENF is another newly introduced network function for providing commands to AMF and for sensing data processing. Furthermore, Control plane connection (Option 1) and User plane connection (Option 2) are both described in the solution for transferring sensing data between the UE/gNB and the core network.

6.X.2	Procedures

6.x.2.1 Sensing procedure for the sensing service by data transferring via Control Plane (option 1) and User Plane (option 2)

Figure 6.X.2-1 illustrates the general network sensing for the Sensing Consumer to the PLMN for the sensing service for non-roaming scenario service where the sensing data is transferred via Control Plane. And Figure 6.X.2-2 illustrates the general network sensing for the Sensing Consumer to the PLMN for the sensing service for non-roaming scenario service where the sensing data is transferred via User Plane. The difference for the two procedures is the method how the sensing data is transferred which is in step 8. Other procedures except step 8 are the same.

In both scenarios, it is assumed that the target UE is identified by using an SUPI.

Both procedures are applicable to a request from an Sensing Consumer for a current sensing of the target UE, and it is assumed that the Sensing Consumer is authorized to use the sensing service and no privacy verification is required.

Figure 6.X.2-1: Sensing procedure for the sensing service by data transferring via Control Plane

Figure 6.X.2-2: Sensing procedure for the sensing service by data transferring via User Plane

1.	The Sensing Consumer (e.g. external sensing services client) sends a request to the GMSENC for sensing area around the target UE identified by an SUPI. The request may include the required QoS, UE unaware indication and Supported GAD shapes. If sensing area is required for more than one UE, the steps following below may be repeated and in that case the GMSENC shall verify whether the number of Target UEs in the Sensing request is equal to or less than the Maximum Target UE Number of the Sensing Consumer. If Maximum Target UE Number is exceeded, the GMSENC shall reject the Sensing request, the step 2-10 are skipped, and then GMSENC respond to the client with proper error cause in the step 11. If the Sensing Consumer is external network function, then the Sensing Consumer initiate the sensing service towards GMSENC via NEF.

2.	The GMSENC invokes a Nudm_UECM_Get service operation towards the home UDM of the target UE to be sensed with the SUPI of this UE.

3.	The UDM returns the network addresses of the current serving AMF.

4.	The GMSENC invokes the Namf_Sensing_ProvideInfo service operation towards the AMF to request the current sensing area around the UE. The service operation includes the SUPI, and client type and may include the required QoS, UE unaware indication and Supported GAD shapes.

5.	If the UE is selected as sensing entity but is in CM IDLE state and UE unaware indication is not included, the AMF may initiate a network triggered Sensing Service Request procedure.

However, the AMF may decide to skip steps 5 - 9 in step 5 based on the UE unaware indication and the UE state or the NG-RAN rejects the Network Sensing message with appropriate rejection cause (e.g. UE cannot be paged).

6.	The AMF selects an SENF based on the available information or based on AMF local configuration. The SENF selection takes the 5G-AN currently serving the UE into account. The selection may use a NRF query.

7.	The AMF invokes the Nsenf_Sening_Determine Request service operation towards the SENF to request the current sensing operation around the UE. The service operation may include a Sensing Correlation identifier, the serving cell identity of the Primary Cell in the Master RAN node and the Primary Cell in the Secondary RAN node when available based on Dual Connectivity scenarios, and the client type, the required QoS, UE unaware indication and Supported GAD shapes.

8.	The SENF performs one or more of the sensing procedures. During this step the SENF may use the Namf_Communication_N1N2MessageTransfer service operation to request the transfer of a Sensing related N1 message to the UE or the transfer of a Network Sensing message to the serving NG-RAN node (e.g. gNB) for the UE. The SENF shall determine a geographical sensing and optionally a sensing in local coordinates. Sensing data could be transferred via Control Plane (Option 1 Figure 6.X.2-1) or User Plane (Option 2 Figure 6.X.2-2)

9.	The SENF returns the Nsenf_Sensing_Determine Response towards the AMF to return the current sensing around the UE. The sensing service operation includes the Sensing Correlation identifier, sensing area, its age and accuracy and may include information about the sensing method and the timestamp of the sensing estimate.

10.	The AMF returns the Namf_Sensing_ProvideInfo Response towards the GMSENC to return the current sensing of the UE. The service operation may include the sensing area, its age and accuracy and may include information about the sensing method and the timestamp of the sensing estimate, etc.

11.	The GMSENC sends the sensing service response to the Sensing Consumer.

6.X.3	Impacts on services, entities and interfaces

GMSENF:

-	Receives sensing service request from Sensing service consumer such as AF, external client and sends the request to AMF;

-	Discoveries the AMF via UDM with UE identity;

- -  Receives response on sensing service request from AMF and send the response to service consumer.

.

SENF:

-	Sends commands to AMF to perform sensing operation;

-	Receives sensing data collected by the gNB and UEs via CP or UP connection;

- -  Derives sensing results by processing sensing data.

.

AMF:

- -  Receives commands and sends the commands to gNB and/or UE to perform sensing operation;

- -  Collects sensing data if via CP connection;

- -  Receives sensing results from SENF and expose them to sensing consumer via GMSENF.

- .

NG-RAN:

- -  Receive sensing request(s) from AMF and send the requests to one UE or a group of UEs or all UEs in the cell if needed;

- -  Performs sensing activities according to commands received from AMF;

- -  Sends the sensing data to SENF via AMF (via CP connection) or via UPF (via UP connection).

- .

UE:

-	Receives sensing request(s) from AMF via NG-RAN;

-	Performs sensing activities according to commands received from AMF;

- 	Sends the sensing data to SENF via AMF (via CP connection) or via UPF (via UP connection).

End of Changes