SA WG2 Meeting #170	S2-2506424

25 – 29 Aug, 2025, Goteborg , SE

Source:	NEC

Title:	[WT#04] Scope and key issue on Sensing

Document for:	Agreement

Agenda Item:	20.6.4

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract: This contribution proposes scope and key issue to Work task 4 of the 6G study on Sensing.

1. Justifications

This contribution discusses the scope and proposes a corresponding key issue to facilitate work on Work Task #4 of the approved 6G SID SP-250806:

WT#4: Study the integration of Sensing and Communication over 3GPP access, considering the sensing modes to be supported and other sources of sensing data.

NOTE 5:	The detailed scope of WT#4 will be determined considering the scope and work of R20 FS_Sensing_ARC.

Architectural Motivation:

3GPP SA1 6G TR22.870 section 7 Integrated Sensing and Communication introduces multiple use cases where the service includes offering wide area multi-dimensional sensing that provides spatial information about unconnected objects as well as connected devices and their movements and surroundings.

The 6G network will support all kinds of Sensing modes, 3GPP and Non-3GPP sensing data that can be combined with communication services.

Gap Analysis:

Whereas 5G Advanced (Rel-20) comprise of the feature to support gNB based mono-static Sensing, the limitation is expected to be solved in 6G with supporting all kinds of sensing modes including UE based sensing, bi-static and multi-static sensing, and Non-3GPP sensing data.

Technical Impact:

- How to support all kinds of sensing modes with Sensing control, Sensing data collection, storage and processing.

- When the sensing target moves, how to support the sensing service continuity.

- When the sensing transmitter/receiver e.g. UE moves, how to support the sensing service continuity.

- How to control, collect, store, processing Non-3GPP sensing data and fuse Non-3GPP sensing data with 3GPP sensing data.

- How to support sensing capability or result exposure to the 3rd party/AF.

Based on the justification, it is proposed to update TS23.801-1 to reflect the scope and key issue of WT#4.

**** First Change (ALL NEW TEXT) ****

Annex A.X. WT#4 Scope

WT#4: Study the integration of Sensing and Communication over 3GPP access, considering all kinds of Sensing modes, 3GPP and Non-3GPP sensing data processing and fusion.

**** Second Change (ALL NEW TEXT)****

5.X. Key Issue #X: Architecture support for integrated compute

This key issue will address the system architecture to support integration of Sensing and Communication, especially considering the following aspects:

- How to support all kinds of sensing modes with Sensing control, Sensing data collection, storage and processing.

- When the sensing target moves, how to support the sensing service continuity.

- When the sensing transmitter/receiver e.g. UE moves, how to support the sensing service continuity.

- How to control, collect, store, processing Non-3GPP sensing data and fuse Non-3GPP sensing data with 3GPP sensing data.

- How to support sensing capability or result exposure to the 3rd party/AF.

**** End of Changes ****