SA WG2 Meeting #170	S2-2506512

25 – 29 Aug, 2025, Goteborg , SE

Source:	China Mobile

Title:	[WT#4, ISAC] Key issues and justifications for 6G ISAC

Document for:	Agreement

Agenda Item:	20.6.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper discusses some justifications and motivations of integration of Sensing and Communication in 6G. Based on the discussion, a detailed key issue is proposed.

1. Justifications

1.1 Existing Progress/Architectural Motivation/ Gap Analysis

Existing progress in 3GPP:

The approved SA2 6G SID includes the following work task:

WT#4: Study the integration of Sensing and Communication over 3GPP access, considering the sensing modes to be supported and other sources of sensing data.

NOTE 5:	The detailed scope of WT#4 will be determined considering the scope and work of R20 FS_Sensing_ARC.

Due to the limitations of TU,  the scope in R20 FS_Sensing_ARC is focused on gNB-based sensing, and the following aspects is being studied:

- WT-1: Architecture and function enhancements to support sensing.

- WT-2: Service authorization and revocation. This includes investigating mechanisms for authorization and revocation sensing service requests from the service consumer.

- WT-3: Discovery and selection of sensing entities based on service requirements triggered by the service request and capability of the sensing entities.

- WT-4: Sensing data and the associated information collection and transport mechanisms for result calculation.

- WT-5: Mechanisms for providing sensing associated information and result (including sensing result and contextual information) exposure, for one time, periodic or event based reporting.

- WT-6: Configuration parameters for the support of ISAC services.

Just as described in the NOTE 5 in the approved SA2 6G SID, some leftover feature from R20 FS_Sensing_ARC should be further studied.

Motivation for Integration of Sensing and Communication in 6G

In 5G-A, the six work tasks have covered the end-to-end sensing service. Since 6G may introduce a new network architecture. Based on the 6G core network, the related work tasks should be studied.

Besides, the scope of TR 23700-14 is focused on gNB-based sensing. However, other WGs, e.g., RAN, SA1, have discussed UE-based sensing and six sensing modes. Especially in  small-scale sensing scenarios, for example, smart home, health monitoring and so on, UE-based sensing and diverse sensing modes would be an important supplement.

On the other hand, in 5G-A, the sensing continuity is excluded  in the existing work task. But in the wide-scale sensing scenarios, for example, UAV flight trajectory tracing, the sensing target would span multiple base stations, therefore, multi-station collaborative sensing is necessary to support the continuous sensing. Besides, for UE-based sensing, as the sensing entity, the coverage range and mobility of UEs also impose higher requirements on continuous sensing.

Therefore, to enhance operators' sensing service capabilities, further study on UE-based sensing and sensing continuity must be important in 6G.

1.2 Technical Impact

Following aspects need to be taken account when studying Integration of Sensing and Communication:

- Basic sensing service enhancement in 6G based on the study in 5G-A.

The basic end-to-end sensing service has been studied in 5G-A, but the related feature should be further studied based on 6G network architecture. In order to avoid redundant study, the existing sensing service logic of 5G-A should be reused as much as possible.

2. Sensing entities management.

- As a sensing entities, UE/RAN should register its sensing capability with the network. Specially, due to UE mobility, the network requires a mechanism to update UE sensing coverage information in order to support UE discovery and selection.

- For scenarios where the UE acts as the transmitter and the RAN as the receiver, as well as the reverse case, coordination management (including discovery and selection of UE and RAN) is needed between RAN and UE devices.

- As a sensing entities, the UE's subscription management should be enhanced to support user privacy consent.

3. Sensing continuity

For wide-scale sensing scenarios involving multi-station coordination as well as UE-RAN collaboration, the network needs to efficiently support sensing entities re-selection, sensing context synchronization, and sensing transmission path reestablishment.

**** First Change ****

Annex A.X. WT#4 Scope

The scope of WT #4 is presented in SP-250806. No update to the Working task description.

WT#4: Study the integration of Sensing and Communication over 3GPP access, considering the sensing modes to be supported and other sources of sensing data.

NOTE 5:	The detailed scope of WT#4 will be determined considering the scope and work of R20 FS_Sensing_ARC.

**** Second Change ****

5.X. Key Issue #X: Integration of Sensing and Communication

It is proposed to study the following key issues:

KI#X：Study the integration of Sensing and Communication over 3GPP access, considering RAN-based sensing and UE-based sensing

- Whether and how to achieve the sensing functionality in 6G based on the study in 5G-A.

- Study the sensing entities management, including whether and how to support sensing entities registering and updating sensing capability with the network,  discovery and selection of sensing entities.

- Study the sensing continuity, including whether and how to support sensing entities re-selection, sensing context synchronization, sensing transmission path reestablishment, and so on.

**** End of Changes ****