3GPP TSG-WG SA2 Meeting #170	S2-2506808

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon, ZTE, SK Telecom

Title:	Interim agreements of KI#2

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: It is proposed to capture interim agreements for KI#2.

1. Introduction/Discussion

1.1	General

For KI#2, SA2 has discussed several solutions for both Use case 1 and Use case 2 as described in clause 5.1 of TS 23.700-04.

| #22 |  | X | X | X |
| --- | --- | --- | --- | --- |
| #23 |  | X | X | X |
| #24 |  | X | X |  |
| #25 |  | X |  | X |
| #26 |  | X |  |  |
| #27 |  | X | X | X |
| #28 |  | X | X |  |
| #29 |  | X | X | X |
| #30 |  | X | X |  |
| #31 |  | X | X | X |

Solution#22 proposes a solution for both UC1 and UC2. It lets NWDAF to collect UPF events which can be presented as packet event ID. The details of packet event ID are FFS.

Solution#23 proposes a solution for abnormal traffic UC to detect UPF related status and user plane status to determine whether it is abnormal traffic or not.

Solution#24 proposes AnLF to be collocated with UPF. The training data is collected offline. The details for the input and output are FFS.

Solution#25 focus on normal traffic (UC2) and let NWDAF predict application requirements based on network status. The PCF can generate PCC decisions based on such prediction.

Solution#26 is focusing on how to minimize the size of the report info from UPF. It introduces processing handling (which is in DCCF) into UPF.

Solution#27 proposes a solution for both UC1 and UC2. It lets NWDAF to collect UPF related status via UPF directly or via OAM. Consumers of related NWDAF analytics could then block abnormal traffic or to increase capacity for legitimate traffic peaks. An IoT Server AF as analytics consumer can also use the information to control abnormal traffic to/from (IoT) devices or to reconfigure some mis-configured devices.

Solution#28 proposes UPF to get model from MTLF directly. The model is just like filtering. It seems no need to introduce AI in this case. Also, it is similar as PFD related analytic. The details of the model are FFS.

Solution#29 introduces the analysis of user plane traffic patterns, primarily enabling dynamic QoS adaptation, while also allowing detection of anomalies if needed. One of the differences is to focus on encrypted traffic. However, how to detect encrypted traffic is FFS.

Solution#30 assumes the UPF is the analytics consumer and forwards related information to the SMF (that in turn forwards it to PCF), rather than SMF and/or PCF being analytics consumers.

Solution#31 introduces the analysis of user plane traffic patterns, considering also the UPF status.

It can be seen that Solution#24, 26, 28 and 30 currently are mainly focusing on how to do the data collection/training/inferencing. All other solutions have shown the views on the detailed input and output of the analysis with the action of the network. It is proposed to discuss input and output of the analysis with the action of the network first.

1.2	Discussion for UC1

The input and output of the analysis with the action of the network for UC 1 can be categorized as following:

For UC 1:

Input

| Type of Parameters | Example of parameters (not full list) | Reference |
| --- | --- | --- |
| Traffic description | packet filter set/traffic flow filter/PDR/IP 5-tuple/APP ID | Solution 22, 23, 27, 29, 31 |
| UE ID | SUPI, UE address | Solution 23, 27, 29 |
| Traffic QoS status/Traffic pattern | Traffic life-time/duration packet size/per flow status info packet delay/delay info (N6, N3/N9, RTT) | Solution 22, 23, 27, 29, 31 |
| UPF status | Capacity GTP Data transmission over N3 interface Data loss over N3 interface Throughput of N3 interface Data transmission over N6 interface Data transmission over N9 interface | Solution 23, 27, 29, 31 |
| N4 status | N4 interface information Average number of N4 Sessions Maximum number of N4 Sessions Minimum number of N4 Sessions | Solution 23, 27, 31 |
| Application specific traffic characteristics | UE address distribution Normal SSL/TLS fingerprint Average and variance of the number of UE communicated with the AS Transport protocol information | Solution 23, 27 |

UC1 is to improve efficient performance of user plane as well as UPF's robustness potential by avoiding abnormal or malicious traffic pattern in the user plane. Thus, the information to recognize abnormal or malicious traffic would be very useful for the input of the Analytics, such as Traffic description, UE ID, Traffic QoS status/Traffic pattern and Application specific traffic characteristics. However, the network cannot guess whether the traffic is abnormal or malicious traffic based on the UPF and N4 status. There can be some issues in N4 or N3/N9 as long as there are too many service data flows, even if the service data flow does not belong to any abnormal or malicious traffic.

Observation 1: Traffic description, UE ID, Traffic QoS status and Application specific traffic characteristics are useful to recognize abnormal or malicious traffic while UPF and N4 status are not.

Proposal 1:  Traffic description, UE ID, Traffic QoS status and Application specific traffic characteristics can be input of the Analytic for Use Case 1.

Output

Almost all the related solutions mentioned whether the traffic is abnormal & the abnormal type of the traffic as an output of the Analytic while some solutions (e.g., Solution#27) also mentions the UPF performance.

The purpose of the use case is to avoid abnormal or malicious traffic. Thus, whether the traffic is abnormal & the abnormal type/ traffic pattern of the traffic is a “must to have” in Output. Deriving the UPF performance is very difficult and not useful when determining whether the traffic is abnormal.  Because the specifics of the malicious traffic is not the data volume but the huge amount of IP flows. If it is for the Core Network action, e.g., whether to reselect the UPF, the current Analytic DN Performance can be reused.

Proposal 2:  The information on whether the traffic is abnormal & the abnormal type of the traffic can be the Output of the Analytic for Use Case 1.

Core Network action:

Almost all the related solutions mentioned that PCF can make Policy decision to update PCC rules or PDU Session related policy information and SMF can do UPF management, e.g. UPF reselection; N4 rule update. This is normal behaviour to reject a service data flow.

Proposal 3:  The Core Network action can be Policy decision making and UPF management for Use Case 1.

1.2	Discussion for UC2

The input and output of the analysis with the action of the network for UC 2 can be categorized as following:

Input:

| Type of Parameters | Example of parameters | Reference |
| --- | --- | --- |
| Traffic description | packet filter set/traffic flow filter/PDR/IP 5-tuple/APP ID | Solution 22, 23, 25, 27, 29, 31 |
| UE ID | SUPI, UE address | Solution 23, 27, 29 |
| Traffic QoS status/Traffic pattern | Traffic life-time/duration packet size/per flow status info packet delay/delay info (N6, N3/N9, RTT) | Solution 22, 23, 25, 27, 29, 31 |

Use Case 2 is to predict the traffic pattern for 5GC to do user plane performance optimization with assistance to QoS control management in advance. Thus, the information to recognize the traffic pattern would be very useful for the input of the Analytics, such as Traffic description, UE ID, and Traffic QoS status/Traffic pattern itself.

Proposal 4:  Traffic description, UE ID, Traffic QoS status and Application specific traffic characteristics can be input of the Analytic for Use Case 2.

Output

All the related solutions mentioned traffic pattern of the traffic. That is the purpose of UC2. This is quite different from UC1.

Proposal 5:  Traffic pattern of the traffic can be the Output of the Analytic for Use Case 1.

Observation 2: The output and purpose of Use Case 1 and Use Case 2 are quite different.

Core Network action:

All the related solutions mentioned the action is that PCF can make Policy decision to update PCC rules or PDU Session related policy information. And the SMF can only follow the instruction of PCF on the QoS parameters. So there is no need for SMF to take any action.

Proposal 6:  The Core Network action can be Policy decision making and UPF management for Use Case 2.

Observation 3: The Core Network action of Use Case 1 and Use Case 2 are different.

Proposal 7: Based on Observation 2 and 3, it is proposed Use Case 1 and Use Case 2 should have different Analytics.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-04.

# * * * * First change * * * * all new text

7.1	Agreed Principles

7.1.Y	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

There should be separate new analytics for both use case 1 and use case 2

For new analytics of Use case 1:

The input of the analytics can be:

- Traffic description

- UE ID

- Traffic QoS status/Traffic pattern

- Application specific traffic characteristics

The output of the analytics can be:

- Traffic description

- Whether the traffic is abnormal & the abnormal type of the traffic

The core network action can be:

- PCF: make Policy decision to update PCC rules or PDU Session related policy information.

- SMF: UPF management, e.g. UPF reselection; N4 rule update.

For new analytics of Use case 2:

The input of the analytics can be:

- Traffic description

- UE ID

- Traffic QoS status/Traffic pattern

The output of the analytics can be:

- Traffic description

- Traffic pattern of the traffic

The core network action can be:

- PCF: make Policy decision to update PCC rules or PDU Session related policy information.

# * * * * End of changes * * * *