SA WG2 Meeting #S2-170	S2-2507510

25 - 29 August, 2025, Goteborg, Sweden

Source:	Samsung

Title:	KI#1 Sol#16 update to remove ENs

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 /Rel-20

Abstract of the contribution: This contribution proposes updates to Sol#16 to remove ENs.

# Discussion

This contribution provides clarification and updates to Sol#16 to remove ENs.

Editor's note:	it is FFS how DCF sends the data collection request to target UEs.

If UE data collection could be triggered by DCF, the DCF may send the request to target UE through AMF and RAN node. For example, the DCF may invoke AMF service operation towards AMF that includes the UE data collection indication. Then the AMF may forward the UE data collection indication to RAN via NG interface or directly to UE via NAS. The interaction between RAN node and UE for data collection configuration and action will be based on RAN discussion and decision.

Editor's note:	How the UE establishes dedicated PDU session for AIML data transfer based on URSP is FFS.

The UE may establish dedicated PDU session for AIML data transfer based on existing mechanism of URSP. However, to have better support of UE data transfer and data differentiation, specific new Traffic Descriptor and Route Selection Descriptor might be introduced for UE data transfer. Therefore, based on the specific Traffic Descriptor and Route Selection Descriptor in the request, dedicated PDU session could be applied for AIML data transfer.

Furthermore, to support the AIML traffic transfer via specific QoS flow, certain packet Filter, 5QI and other packet filter information might be reserved for AIML traffic.

Editor's note:	How the gNB triggers UE data transfer is FFS.

It is assumed that the data transfer is controlled by DCF. However, the requirements of data transfer for different AIML services might be different, e.g. some service may require the data transfer within in certain time window, transfer certain data volume, etc. Those requirements might be indicated to gNB from DCF to trigger UE data transfer. Once the data requirements are met, the gNB may trigger the UE data transfer towards UE. The interaction between gNB and UE should be in RAN scope, not SA2.

Editor's note:	Whether and how the DCF will handle the AIML data (excluding transferring the data to server) is FFS.

The one of the fundamental purposes of supporting AIML data transfer is to enable the operator’s controllability and visibility of AIML data. To protect 5GS security and user privacy, based on operator determination, the DCF might be capable of handling the AIML data, e.g. filtering out sensitive UE data before exposing the collected to data to the server.

# Proposal

It is proposed to adopt the following changes into TR 23.700-04.

*** Start of the change ***

### 6.16.3	Procedures

#### 6.16.3.1	Procedures of DCF initiated UP connection for data transfer

Figure 6.16.3.1-1: UE side AIML date transfer via a User Plane Connection between UE and DCF initiated by DCF

The DCF (data collection function) may trigger the user plane connection establishment for UE side AIML data transfer, e.g. after receiving AIML data collection request from AIML (training) server or based its internal logic, if target the UE does not have user plane connection with the corresponding DCF. The DCF may subscribe to the AMF for the status of the user plane connection for the target UE.

Figure 6.16.2.1-1 shows the procedures of DCF initiated UP connection for AIML UE side data transfer for AIML model training.

0.	[Optional] The UE model training entity server may determine to request UE side data for AIML model training by sending request to or subscribe to DCF, optionally with a list of target UEs.

1.	The DCF determines to request the UE side data for AIML model training purpose. This step might be triggered by the request from AIML server or DCF internal logic. The DCF checks the user consent to determine whether the data collection request can be complied or not.

2.	DCF triggers data collection request towards the target UEs, e.g. the DCF sends data collection request via AMF, the AMF forwards the request to UE and/or NG-RAN.

NOTE 1:	The interaction between RAN node and UE for radio measurement configuration and action is in RAN scope.

3.	UE performs data collection for AIML model training based on the request and configuration from network.

4.	[Conditional] If there is no established secure user plane connection between the UE and DCF, the DCF invokes Namf_communication_N1N2MessageTransfer service operation to send its user plane information to AMF in a NAS container to indicate UE to utilize user plane for AIML data transfer.

The user plane information includes the user plane address of DCF (e.g. IP address or FQDN).

DCF may also may allocate an AIML data collection binding/correlation ID to associate the user plane connection to be established with the target UE identify (SUPI and/or GPSI). The DCF may include this ID in the user plane information.

The target UE identify (SUPI and/or GPSI) might be indicated to the AMF by DCF in step 2.

5.	[Conditional] When AMF receives the user plane information, AMF sends it to the target UE via a DL NAS TRANSPORT message by including the information in step 5.

6.	[Conditional] If there is no established applicable PDU session for the user plane AIML data transfer, the UE uses the URSP as defined in TS 23.503 [4] which includes user plane AIML data transfer related PDU session parameters, e.g. a dedicated DNN, time window, S-NSSAI, location, etc. to establish the PDU session for user plane AIML data transfer. Existing URSP is reused to establish dedicated PDU session, e.g. by using specific Traffic Descriptor for the application and Route Selection Descriptor for UE AIML data transfer.

7-8.	[Conditional] UE sends an acknowledgement to the DCF through AMF to indicate a success of user plane connection establishment for AIML data transfer via UP or a failure to utilize the user plane connection.

9.	[Optional] Data transfer request to target UE(s). If the UE data transfer is triggered towards gNB, the gNB may initiated UE data transfer request to UE based on the information indicated by DCF, e.g. the time window for AIML data transfer.

NOTE 2:	how the gNB initiates data transfer towards UEs is in RAN scope.

Alternatively, the DCF may trigger the data transfer towards UE via the UP connection. Then the UE may interact with gNB for radio configuration.

NOTE 3:	the interaction between RAN node and UE is in RAN scope.

10.	AIML UE side data transfer for AIML model training from UE to DCF via the established UP connection with the assigned AIML data collection binding/correlation ID.

The UPF detects or differentiates the AIML data from other regular UE data based on the source/destination IP address of the data packets, by reusing the traffic detection mechanism defined in TS 23.501 [2]. The destination IP address of the AIML data packets is the IP address of the DCF. Different charging policies might be applied to the AIML data.

11.	DCF notifies the AIML UE side data for AIML model training to the AIML server.

The DCF may filtered the data to be transferred to the AIML server based on user consent, user privacy and subject to operators' policies.

The DCF may determine to store the UE data in ADRF.

12.	Data collection and/or transfer termination. The termination might be decided by the AIML server, the target UE or DCF.

*** End of the change ***