SA WG2 Meeting #170	S2-2507613

25 – 29 Aug 2025, Goteborg, SE 	was S2-2507552

Source:	Nokia

Title:	WT#1: Solution for ADC establishment in alerting phase

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for FS_NG_RTC_Ph3.

# 1. Discussion

This pCR proposes a solution to the working task about the establishment of ADC in alerting phase in Next Generation Real Time Communication Phase 3.

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-56.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[yy]	3GPP TS 23.203: "Policy and charging control architecture".

[zz]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

# * * * * Next change * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #3> |
| #X: ADC establishment in alerting phase initiated by the UE | X |  |

## 6.X	Solution #X: ADC establishment in alerting phase initiated by the UE

### 6.X.0	High-level solution Principles

This solution addresses Key Issue #1: DC establishment in alerting phase for early media

The solution reuses R19 23.228 with the addition that

-	Based on the UE request (via a specific root URL) , the DSCF sends to the UE(s) an application list related with the alerting phase

-	The calling party receive BDC from the terminating network during alerting phase.

-	The UE(s) decide whether and which application to download (as per R19 specifications)

-	The UE(s) decide when to release an application that was running in alerting phase; they may decide to continue this application after the alerting phase has finished. (as per R19 specifications)

The solution may e.g. be used for the case where the originating UE1 calls a help center and is immediately provided with an App that the user will be using when interacting with the call center.

### 6.X.1	Description

This solution provides a procedure on how to establish a P2A ADC in alerting phase.

Editor's Note: Current version of the solution supports P2A ADC establishment in alerting phase. it is FFS whether it can support other forms of ADC, i.e., P2A2P and P2P.

### 6.X.2	Procedure

The pre-condition of ADC establishment in alerting phase is the capability negotiation during the IMS registration phase on whether the UE supports the DC in alerting phase.

Based on the IMS DC capability negotiation defined in clause AC.7.0 of TS 23.228 [xx], when the UE supporting DC in alerting phase and the UE is allowed to use DC in alerting phase, it includes the media feature tag of supporting DC in alerting phase in the Contact header field of the initial REGISTER request and any subsequent REGISTER request to allow the home IMS network to discover its capability of supporting DC in alerting phase.

If the IMS network supports DC in alerting phase, the S-CSCF includes a Feature-Caps header field indicating its capability of supporting DC in alerting phase in the 200 OK response to the initial and any subsequent REGISTER request, which is used by the UE to discover the IMS data channel capability of its home IMS network.

The UE may receive a Feature-Caps header field indicating IMS's capability of supporting DC in alerting phase in the 200 OK response to a subsequent REGISTER request when the network starts supporting DC in alerting phase after successful initial registration of the UE.

When the UE supporting DC in alerting phase initiates an IMS session and if the UE is allowed to use DC in alerting phase, it includes the media feature tag in the Contact header field of the initial INVITE message, regardless of data channel media being part of the SDP or not.

The UE shall not include the media feature tag in the Contact header field and data channel media description in the SDP offer of the initial INVITE request or any subsequent re-INVITE request message, if the S-CSCF has not included the capability indication of supporting DC in alerting phase in the Feature-Caps header field in the 200 OK response either to the initial REGISTER or a subsequent REGISTER request message.

If the UE is not configured whether to use DC in alerting phase, it is up to UE implementation whether or not to include the media feature tag in the Contact header field.

With a successful capability negotiation about the support of DC in alerting phase, the procedure is illustrated in Figure 6.X.2-1, which is based on clause AC.7.2.2 in TS 23.228 [xx].

Figure 6.X.2-1: Procedure of ADC establishment in alerting phase.

The steps in the procedure are as follows:

1.	Same as Step 1-14 of clause AC.7.1 in TS 23.228 [02], UE#1 initiates a SIP INVITE towards the IMS AS. After validating the user’s subscription, the IMS AS notifies the DCSF about DC support. The DCSF provides the IMS AS with instructions and media information for establishing BDC. The IMS AS allocates resources with MF accordingly and updates the INVITE with the negotiated media. The updated SIP INVITE is forwarded to the terminating network/UE#2 for terminating network negotiation.

2-3.	UE#2 and terminating network returns an 18x response with the SDP answer to BDC to originating network. According to the received SDP answer, MF may update data channel media resource information for UE#2.

4.	The IMS AS notifies the successful session establishment event, Nimsas_SessionEventControl Notify (SessionEstablishmentSuccessEvent, Session ID, Media Info List) to DCSF.

5.	The DCSF responds to the Nimsas notification request.

6.	The SIP message18x is forwarded to UE#1 which indicates the BDC has been established and that the alerting phase has started.

7.	The originating network P-CSCF executes QoS procedure for BDC as specified in TS 23.203 [yy] and TS 23.503 [zz].

8.	PRACK and 200 OK for PRACK procedure are performed.

9-10.	The DCSF provides the application list for alerting phase from the specific URL provided by UE#1 via MF, further to UE#1 and UE#2, based on their data channel capabilities and their choices through MF.

when the BDCs have been established between terminating MF and UE#1/UE#2, the DC application list for alerting phase and DC application for alerting phase is requested and downloaded to UE#1 and UE#2 from terminating DCSF.

the DCSF provides via MF the application list for alerting phase from the received specific URL , further to UE#1 and UE#2, based on their data channel capabilities and their choices.

the UE(s) use a specific URL related with the fact that they request a list of applications available in the alerting phase

11.	UE#1 initiates a P2A ADC for the selected DC application for alerting phase. UE#1 sends SIP UPDATE with an updated SDP to IMS AS via originating network P-CSCF and S-CSCF. The updated SDP contains the BDC information, as well as the requested ADC and the associated DC application binding information.

12.	Same as Step 3-14 of clause AC.7.2.2-1 in TS 23.228 [02x], the IMS AS notifies the DCSF about DC support. The DCSF instructs the IMS AS to establish MF anchoring via MDC2. The IMS AS provisions the necessary media resources at the MF and returns the information to the DCSF. The DCSF then initiates ADC setup toward the DC Application Server via DC3/DC4. After receiving the DC AS’s response with its own MDC2 media resource, the DCSF instructs the IMS AS to update the MF resource accordingly. The IMS AS completes the MF configuration and notifies the DCSF.

13-14.	IMS AS sends the SIP UPDATE to remote network side and UE#2, via the originating S-CSCF, which does not include a request in the SDP for the ADC.

15-17.	UE#2 and terminating network returns a SIP UPDATE 200 OK response with SDP answer for audio/video/bootstrap.

18.	IMS AS notifies the DCSF about the successful result of media confirmation.

19.	DCSF replies to the notification.

20.	The IMS AS includes SDP answer for ADC to UE#1 in SIP UPDATE 200 OK response that it sends towards the UE 1 via the S-CSCF and P-CSCF.

21.	The originating network P-CSCF executes QoS procedure for ADC media based on the SDP answer information from the 200 OK response.

22.	SIP UPDATE 200 OK forwarded to UE#1 with ADC SDP answer.

23.	The ADC between UE#1 and DC AS is established via MF. MF forwards data channel traffic between UE#1 and DC AS based on MDC2 media point information.

24-25.	UE#2 answers the call, and SIP INVITE 200 OK response is sent to the IMS AS via I/S-CSCF.

26.	The IMS AS notifies the DCSF, via Nimsas_SessionEventControl_Notify (Session ID) of the call state change.

27.	DCSF replies to the notification.

28-29.	SIP INVITE 200 OK forwarded to UE#1 which indicates the call is answered, and ACK procedure is performed.

30.	Both UE#1 and UE#2  retrieve a new application list via BDC since call is answered. The new list supersedes the previously downloaded list. The UEs send request to MF to request application list by providing a root URL, via the established BDC with its data channel capabilities. The DCSF provides the application list from the root URL provided by UE#1 via MF, further to UE#1 and UE#2, based on their data channel capabilities and their choices through MF.

31.	Subsequent procedures for DC application, as defined in clause AC.7.2 of TS 23.228 [02]. Depending on the application and user interaction, the UE(s) determine when to release the application initiated in alerting phase but this aspect is out of scope of specifications and does not require a new UE behavior..

### 6.X.3	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

UE:

-	supporting retrieving DC application list and DC application for alerting phase; this includes providing a specific URL to get from DCSF the application list for alerting phase,

-	supporting ADC establishment in alerting phase.

IMS AS:

-	supporting negotiation of ADC in alerting phase.

Editor’s Note: impacts on IMS AS and DCSF are FFS

DCSF:

-	providing DC application list based on the specific URL received from the UE and associated with the application list for alerting phase.

# * * * * End of changes * * *