SA WG2 Meeting #170	S2-2507555

25 – 29 Aug 2025, Goteborg, SE 	                                       (was 6302)

Source:	Nokia

Title:	WT#2.2: Solution for DC3/DC4 enhancements

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for FS_NG_RTC_Ph3.

# 1. Discussion

This pCR proposes a solution to the working task about the DC3/DC4 enhancement in Next Generation Real Time Communication Phase 3.

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-56.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #3> |
| #X: DC3/DC4 based on DCSF services |  | Key Issue #3: Supporting dynamic configuration of MDC2 endpoints |

## 6.X	Solution #X: DC3/DC4 based on DCSF services

### 6.X.0	High-level solution Principles

This solution addresses Key Issue #3: Supporting dynamic configuration of MDC2 endpoints.

The DC AS subscribes to UE-initiated P2A/P2A2P ADC establishment event notifications at the DCSF using a new Ndcsf_SessionEvent_Subscribe service operation.

Editor’s Note: how the DC AS event subscription is delivered to the DCSF is FFS.

Upon receipt of a P2A or P2A2P ADC establishment request meeting the subscription filter conditions, the DCSF invokes a new Ndcsf_SessionEvent_Notify to notify the DC AS of the event. This notification includes MF-side information required for MDC2 configuration and an URL where the DCSF expects further instructions from the DC AS.

Based on the received notification from the DCSF, the DC AS derives the DC AS-side MDC2 configuration information and transfers it to the DCSF via a new Ndcsf_ADCControl_Create operation towards the URL received in Ndcsf_SessionEvent_Notify.

### 6.X.1	Description

This solution specifies the DCSF services exposed over the DC3/DC4 interface to support UE-initiated P2A and P2A2P ADC establishment. Specifically, the following service functions are defined: Ndcsf_SessionEvent and Ndcsf_ADCControl. The associated service parameters and corresponding procedures are also defined.

The DC AS subscribes to UE-initiated P2A/P2A2P ADC establishment event notifications at the DCSF using the Ndcsf_SessionEvent_Subscribe service operation. Such an event subscription can be explicitly initiated by the DC AS or implicitly configured per DC AS in the DCSF. Events can be UE related or UE unrelated and associated with filter conditions such as P2A/P2A2P ADC associated with specific applications. Upon receipt of a P2A or P2A2P ADC establishment request meeting the subscription filter conditions, the DCSF invokes Ndcsf_SessionEvent_Notify to notify the DC AS of the event. This notification includes MF-side information required for MDC2 configuration. Based on the notification received, the DC AS derives the DC AS-side MDC2 configuration information and transfers it to the DCSF via the Ndcsf_ADCControl_Create service operation. The DCSF subsequently proceeds with the MDC2 configuration based on the information received from the DC AS.

This enhancement to the DC3/DC4 interface is crucial for ensuring consistent and up-to-date MDC2 media resources between the DCSF and the DC AS. The DC AS and the MF may dynamically deploy/insert MDC2 terminations and thus use the dynamically determined IP address. The DC port and/or DC SCTP port may be dynamically associated to the session by the MF and/or the DC AS. Furthermore, the network (DCSF) may need to provide the DC AS with the UE certificate fingerprint received from the UE over IMS signaling.

The current P2A/P2A2P data channel procedures lack a mechanism for MDC2 media resource negotiation, leading to potential inconsistencies of MDC2 media resource between the two MDC2 endpoints. This proposed enhancement, by enabling the MDC2 media negotiation and dynamic configuration, significantly improves the robustness and flexibility of these procedures, preventing failures due to outdated or missing MDC2 media resource information.

### 6.X.2	Service definition

#### 6.X.2.1	General

The following table shows the DCSF Services and DCSF Service Operations.

Table 6.X.2-1: NF services provided by the DCSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcsf_SessionEvent | Subscribe | Subscribe/Notify | NEF, Trusted AF/DC AS |
|  | Unsubscribe | Subscribe/Notify | NEF, Trusted AF/DC AS |
|  | Notify | Subscribe/Notify | NEF, Trusted AF/DC AS |
| Ndcsf_ADCControl | Create | Request/Response | NEF, Trusted AF/DC AS |
|  | Update | Request/Response | NEF, Trusted AF/DC AS |
|  | Delete | Request/Response | NEF, Trusted AF/DC AS |

Editor's Note: Further information, e.g., QoS, charging, monitoring, is for FFS.

Editor's Note: The content of UE's and DC AS's certificate fingerprint for end-to-end security between UE and DC AS shall be aligned with SA3.

#### 6.X.2.2	Ndcsf_SessionEvent

##### 6.X.2.2.1	General

Service description: This service enables the consumer to subscribe and to be notified when an P2A or P2A2P ADC is established.

The following operations are available for this service:

-	Subscribing and unsubscribing to receive P2A or P2A2P ADC establishment event.

-	Notifying P2A or P2A2P ADC establishment event to consumers, e.g. NEF, trusted DC AS.

##### 6.X.2.2.2	Ndcsf_SessionEvent_Subscribe

Service operation name: Ndcsf_SessionEvent_Subscribe

Description: This service operation enables consumers to subscribe to the event of P2A or P2A2P ADC session establishment.

Inputs, Required: Notification Target address , Event ID

Only one Event Id is currently defined: establishment of a MDC2 channel is needed

Inputs, Optional: Target list of User IDs (IMPUs), list of Application binding information

When no IMPU is given, the subscription is assumed to be non-UE specific and thus to apply to Any UE (otherwise an explicit Any UE target indication may be defined).

Outputs, Required: Result, Subscription correlation ID when the subscription is accepted.

Outputs, Optional: None.

##### 6.X.2.2.3	Ndcsf_SessionEvent_Unsubscribe

Service operation name: Ndcsf_SessionEvent_Unsubscribe

Description: This service operation enables consumers to unsubscribe from the event of P2A or P2A2P ADC session establishment.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.X.2.2.4	Ndcsf_SessionEvent_Notify

Service operation name: Ndcsf_SessionEvent_Notify

Description: This service operation enables the DCSF to notify the NF subscriber about the subscribed event.

Inputs, Required: Session ID, MF-side MDC2 endpoint information (e.g. IP address, port number), Resource URL(for any Ndcsf_ADCControl operation that would correspond to the Notification), DC application ID, certificate fingerprint of UE for end-to-end security between UE and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode.

Inputs, Optional: Calling party and called party IDs, application ID, securitySetup if UE initiates the DTLS negotiation.

Outputs, Required: Result.

Outputs, Optional: None.

#### 6.X.2.3	Ndcsf_ADCControl

##### 6.X.2.3.1	General

Service description: This service enables the consumer to create, update or terminate an ADC.

##### 6.X.2.3.2	Ndcsf_ADCControl_Create

Service operation name: Ndcsf_SessionEvent_Create

Description: This service operation enables the consumer to create information used by the DCSF for ADC establishment in an IMS session.

Inputs, Required: Session ID, DC AS-side MDC2 endpoint information (e.g. IP address, port number), certificate fingerprint of DC AS for end-to-end security between UE and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode.

Inputs, Optional: securitySetup if DC AS initiates the DTLS negotiation.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.X.2.3.3	Ndcsf_ADCControl_Update

Service operation name: Ndcsf_ADCControl_Update

Description: This service operation enables the consumer to update information used by the DCSF for ADC updating in an IMS session.

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.X.2.3.4	Ndcsf_ADCControl_Delete

Service operation name: Ndcsf_ADCControl_Delete

Description: This service operation enables the consumer to provide information used by the DCSF for ADC termination in an IMS session

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

### 6.X.3	Procedure

#### 6.X.3.1	Person-to-Application (P2A) Application Data Channel Setup

Figure 6.X.3.1-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.X.3.1-1: P2A ADC establishment with a trusted DC AS.

1.	IMS session and BDC have been established.

2.	The DC AS subscribes to the session event notification at the DCSF via Ndcsf_SessionEvent_Subscribe service.

3.	The DCSF responds the request to the DC AS.

4-11.	UE#1 requests the establishment of P2A ADC establishment. DCSF determines the DC control policy and indicates the IMS AS to request MF for reserving the originating media resources.

12.	DCSF notifies DC AS about the session establishment via Ndcsf_SessionEvent_Notify message. The message includes calling and called parting id, application id, session id, MF-side MDC2 media resource info, Resource URL used by DC AS to send subsequent ADC Control message, and the certificate fingerprint of UE.

13.	DC AS responds to the Ndcsf_SessionEvent_Notify message.

14.	DCSF waits for ADC control instruction (Ndcsf_ADCControl_Create). If such instructions are not provided then DCSF has the ADC establishment procedure aborted (Nimsas_MediaControl_MediaInstruction).

15.	DC AS allocates DC AS-side MDC2 media resource for this session. DC AS sends Ndcsf_ADCControl_Create message with the instruction for session control and the allocated DC AS-side MDC2 media resource information. The message includes session id, DC AS-side MDC2 media resource info, and may include the certificate fingerprint of DC AS.

16-18.	DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information. Then, the IMS AS sends response to DCSF.

19. The DCSF sends response of Ndcsf_ADCControl_Create to DC AS.

20-32.	IMS AS performs SDP negotiation with terminating network. With a successful SDP negotiation, the application data channel established.

#### 6.X.3.2	Person-to-Application and Application-to-Person (P2A2P) Procedure

Figure 6.X.3.1-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.X.3.2-1: P2A2P ADC establishment with a trusted DC AS.

1.	IMS session and BDC has been established.

2.	The DC AS subscribes to the session event notification at the DCSF via Ndcsf_SessionEvent_Subscribe service.

3.	The DCSF responds the request to the DC AS.

4-11.	UE#1 requests the establishment of P2A2P ADC establishment. DCSF determines the DC control policy and indicates the IMS AS to request MF for reserving the originating and terminating media resources.

12.	DCSF notifies DC AS about the session establishment via Ndcsf_SessionEvent_Notify message. The message includes calling and called parting id, application id, session id, MF-side MDC2 media resource info, Resource URL used by DC AS to send subsequent ADC Control message, and the certificate fingerprint of UE.

13.	DC AS responds to the Ndcsf_SessionEvent_Notify message.

14.	DCSF waits for ADC control instructions (Ndcsf_ADCControl_Create). If such instructions are not provided then DCSF has the ADC establishment procedure aborted (Nimsas_MediaControl_MediaInstruction).

14b The DCSF sends response of Ndcsf_ADCControl_Create to DC AS.

15.	DC AS allocates DC AS-side MDC2 media resource for this session. DC AS sends Ndcsf_ADCControl_Create message to the DCSF with the instruction for session control and the allocated DC AS-side MDC2 media resource information. The message includes session id, DC AS-side MDC2 media resource info, and may include the certificate fingerprint of DC AS.

16-23.	IMS AS performs SDP negotiation with terminating network. And the IMS AS notifies the DCSF about the successful result of the SDP negotiation.

24.	DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information and the media resource information of the terminating network. Then, the IMS AS sends response to DCSF. This follows existing procedures e.g. steps 11 and 12 of TS 23.228 [2] Figure AC.7.2.2-1

25. The DCSF sends response of Ndcsf_ADCControl_Create to DC AS.25. Void

26-33.	IMS AS continues performing SDP negotiation with the originating network. With a successful SDP negotiation, the application data channel established.

### 6.X.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DC AS:

-	subscribing the event of UE-initiated P2A/P2A2P ADC establishment,

-	receiving notification about the event of UE-initiated P2A/P2A2P ADC establishment,

-	receiving MF-side information for MDC2 configuration,

-	receiving the certificate fingerprint of UE for end-to-end security between UE and DC AS,

-	sending DC AS-side information for MDC2 configuration,

-	sending the certificate fingerprint of DC AS for end-to-end security between UE and DC AS.

DCSF:

-	receiving subscription of the event UE-initiated P2A/P2A2P ADC establishment,

-	notifying about the event of UE-initiated P2A/P2A2P ADC establishment,

-	sending MF-side information for MDC2 configuration,

-	sending the certificate fingerprint of UE for end-to-end security between UE and DC AS,

-	receiving DC AS-side information for MDC2 configuration,

-	receiving the certificate fingerprint of DC AS for end-to-end security between UE and DC AS,

-	requesting IMS AS for MDC2 configuration.

IMS AS:

-	receiving MDC2 configuration request from DCSF,

-	instructing MF for MDC2 configuration.

MF:

-	receiving MDC2 configuration instructions from IMS AS,

-	performing MDC2 configuration.

# * * * * End of changes * * *