3GPP TSG-SA2 Meeting #170	S2-2507677

Goteborg, Sweden, August 25-29 2025

Source:	Nokia

Title:	KI#3: proposed agreement of principles

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposes to agree principles for KI#3. Nokia is acting as pen holder of the consolidation effort.

# 1.	Introduction

This paper summarizes the input from

| 20.4.1 | - | KI#3 Interim Agreement and Principles | - |
| --- | --- | --- | --- |
| 20.4.1 | S2-2506242 | 23.700-67: KI#3: proposed agreement of principles | Nokia |
| 20.4.1 | S2-2506265 | 23.700-67: KI#3 interim agreements | CATT |
| 20.4.1 | S2-2506413 | 23.700-67: KI#3 Evaluation and agreements | Ericsson |
| 20.4.1 | S2-2506456 | 23.700-67: KI#3: Interim agreements | NTT DOCOMO |
| 20.4.1 | S2-2506772 | 23.700-67: Key issue#3, the analysis and interim agreement | ZTE |
| 20.4.1 | S2-2507044 | 23.700-67: Interim principles for KI3 | vivo |
| 20.4.1 | S2-2507147 | 23.700-67: Agreed principles for key issue #3 | China Mobile |

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-67.

# * * * * First change (all new text) * * * *

### 7.1.3	Agreed Principles for KI#3

The following principles are agreed for KI#3:

-	For NF selection, the NF profile is enhanced to include the parameters in the table below. This is a feature requiring the software to be upgraded in NRF and its consumers.

| Information | Description |
| --- | --- |
| EnergySaving /Not EnergySaving State | Value of the energy state as specified in TS 28.310 [9], e.g. notEnergySaving state, Energy Saving state. (NOTE 1) |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states, includes the value of EnergySaving or not EnergySaving state which applies at each time interval in the schedule. If this is present, the EnergySaving /Not EnergySaving State attribute is not present. |
| Energy Priority Information | Value of Energy priority relative to other NFs of the same type considering the energy characteristics of the NF. |
| Schedule of Energy Priority Information | The Schedule of the Energy Priority includes the value of Energy Priority information which applies at each time interval in the schedule. If this is present, the Energy Priority information attribute is not present |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310 [9]. |  |

-	For NF profile, additional parameter such as renewable energy consumption ratio may be considered is SA5 indicates support.

-	For UP path selection and reselection, the UPFs are selected/reselected by using NF discovery and selection procedures which may be Energy-aware if NF profile is updated as per bullets above. Changes in NF profile of a UPF may trigger UPF reselection subject to SSC mode of the PDU sessions.

### 7.2.3	Topics for further consideration for KI#3

The following principles are for further discussions for KI#3:

-	whether the NF profile needs to include explicit indication of usage of renewable energy or using energy Priority to implicitly take this into account is sufficient.

-	Whether considering the energy consumption of a PDU sessions or of a NF over a Path should be a trigger for reselection of UPFs (including embedding this information in DNAIs )

-	In case of multiple configured energy policies in network, the "energy priority" is an array of priorities. Each entry of the array is the priority of the NF in the corresponding energy policy configured by the operator.

# * * * * End of change * * * *