SA WG2 Meeting #171	S2-2508286

13 – 17 October 2025, Wuhan, China

Source:	LG Electronics

Title:	KI#5: Interim agreements

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This paper proposes interim agreements for KI#5.

Discussion

For KI#5 "Sensing Result Exposure", 13 solutions have been proposed. An analysis and summary of these solutions are provided in the following table.

| Sol.# | Exposed Sensing Result Type (w/ or w/o contextual info) | Sensing Result Exposure Method (e.g., one-time, periodic, event-trigger) |
| --- | --- | --- |
| #1 | Sensing result generated by SF (based on sensing data + associated info from SE); contextual info: Not mentioned | Configured in Sensing Service Request (e.g., periodicity, event report criteria) |
| #2 | Sensing result determined by SSF (based on sensing data + associated info collected by SSF); contextual info: Not mentioned | Reporting mode requested by UE (e.g., one-time, periodic, event-trigger) |
| #3 | Sensing result determined by SF or SPF (from 3GPP sensing data); contextual info: Not mentioned | Exposure via NEF to AF (one-time, periodic, or event-trigger, depending on sensing service request from AF) |
| #19 | Sensing result produced either by SPF or SE; contextual info: Not mentioned | Sensing result exposure alternatives (from SPF to SSC UE, from exposure function to 3rd Party SSC AF); Sensing result exposure method is FFS |
| #23 | Sensing result(s) generated by one or multiple SePFs; contextual info: Not mentioned | Sensing result delivered to SSC; Detailed result delivery mechanism: No details (FFS) |
| #27 | Sensing result generated by SF (SCF+SPF); contextual info: included (e.g., target object type, target object mobility profile, environment of sensing) | Sensing result exposure to sensing service consumer (i.e., trusted/untrusted AF or NF); (one-time, periodic, or event-based result notification) |
| #28 | Sensing result determined by SnCF; (Notify sensing result only, notify sensing result with associated information); contextual info: Not mentioned | AF specifies optional event reporting mode (one-time, periodic, event-triggered) |
| #29 | Sensing result generated by SPF; Sensing result types: Real-time sensing results; Subscription/Notification sensing results; contextual info: Not mentioned | Real-time sensing result via data connection; Event-triggered sensing result via CP or data connection |
| #30 | Sensing result generated by SF; Sensing result payload (e.g., location, speed); contextual info: included (e.g., timestamp, accuracy estimation, confidence level, serving area) | Direct exposure for trusted AFs or indirect exposure for untrusted AFs via NEF; (one-time, periodic, event-trigger reporting) |
| #31 | Sensing result generated by SSMF or retrieved from Sensing result storage NF; Sensing result types: sensing result format; contextual info: Not mentioned | Sensing result transferring mode: CP or data connection; |
| #32 | Sensing result determined by SeMF; Sensing service response: object presence, object count, object definition, quality indicators, velocity, time stamp; contextual info: Not mentioned | Sensing result request of the sensing service consumer; Event reporting type: one time, periodic, event triggered, maximum number of event reports |
| #33 | (distributed) Sensing NF calculate sensing result (based on sensing data with sensing operation ID + associated info); contextual info: operation ID, location info | SSC may request sensing service (for a one-time, periodic, and event-triggered report); sensing result exposure use signalling or data connection depending on configuration. |
| #37 | Sensing result generated by SPF/SCF (based on 3GPP sensing data + associated sensing info); contextual info: supported (No details) | Configured in Sensing Service Request (supports one-time, periodic, and event-triggered reporting) |

Summary

- All solutions agree that (existing or new) sensing function(s) generate sensing results based on sensing data and/or associated information collected from sensing entity(ies). - 9 solutions (Sol#1, Sol#2, Sol#3, Sol#27, Sol#28, Sol#30, Sol#32, Sol#33, Sol#37) explicitly define sensing result exposure methods such as one-time, periodic, and event triggered reporting. - Most solutions (Sol#1, Sol#2, Sol#3, Sol#19, Sol#23, Sol#28, Sol#29, Sol#31, Sol#32, Sol#37) mostly left the use of contextual information as FFS or not mentioned, requiring further study.

Proposal

It is proposed to agree the following changes into TR 23.700-14.

* * * * Start of 1st Change * * * *

!! All New Texts !!

7.1	Agreed Principles

7.1.Y	Agreed Principles for KI#5

The interim agreements on principles for KI#5 are as follows:

-	The Sensing Function is responsible for generating sensing results based on sensing data and/or associated information collected from Sensing Entity(ies).

-	The exposure of sensing result supports various reporting modes (i.e., one-time, periodic, and event-triggered) according to service requests from Sensing Service Consumer(s).

7.2	Topics for further consideration

7.2.Y	Topics for further consideration for KI#5

Topics for further consideration for KI#5 are as below:

-	How to define the accuracy level requirements of sensing results, including confidence levels and methods for processing.

-	Whether and what specific types of contextual information are included with sensing results.

-	How to determine the appropriate sensing exposure method (e.g., via control plane, via user plane), considering factors such as data volume, latency, QoS, and Sensing Service Consumer capabilities.

-	Whether and how sensing results are stored, and how stored results can be reused for new sensing service requests.

* * * * End of Changes * * * *