SA WG2 Meeting #171	S2-2508412

13 - 17 October, 2025, Wuhan, China

Source:	CATT

Title:	KI #4, New Sol: EPC-NI-LR based location service for emergency situation

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: Introduce a new solution to key issue#4 in TR 23.700-19.

1. Discussion

1.1	Introduction

This pCR is proposed to add a new solution to key issue#4: Location service for IMS emergency call and regulatory services over NB-IoT NTN.

1.2	Existing mechanisms for core network to obtain UE location

1.2.1	EPC-NI-LR Procedure

When an emergency situation (i.e. after emergency Attach, UE requested emergency PDN Connectivity) is detected, the MME may initiate the EPC Network Induced Location Request (EPC-NI-LR) procedure in clause 9.1.17 in TS 23.271, as shown in Figure 1, to obtain UE location and send the UE location to the GMLC which is responsible to provide the UE location to an external emergency services client to determine the proper Public Safety Answering Point (PSAP) destination as described in TS 23.167.

Figure 1: Network Induced Location Request for EPC

1.2.2	Coarse Location reporting via NAS Security Mode Command Procedure

In the case of satellite NB-IoT, the MME requests coarse location information from UE via NAS Security Mode Command procedure. Based on TS 36.331, the coarse location can meet the accuracy requirement which corresponds to a granularity of approximately 2km.

1.3	Analysis on existing mechanisms

1.3.1	Analysis on EPC-NI-LR Procedure

The general principles of EPC-NI-LR can be reused, as follows:

- The MME determines the emergency situation happens.

- When the emergency situation is detected, the MME obtains UE location and sends the location to GMLC/LRF.

- When the MME determines the emergency service is no longer active, the MME notifies the event (EMERGENCY_CALL_RELEASE) to GMLC/LRF.

For the principle 2 above, in the existing EPC-NI-LR, the MME sends Location Request message to E-SMLC to obtain UE location. In this case, the E-SMLC selects RAT dependent positioning methods and/or RAT independent positioning methods to determine UE location. The analyses are as follows.

- RAT dependent positioning methods

The RAT dependent positioning methods are based on LTE signals and include OTDOA, UTDOA and E-CID methods.

The OTDOA and UTDOA methods make use of the measured timing of downlink/uplink signals received from/to multiple Transmission Points (TPs)/Location Measurement Units (LMUs). The resulting measurements are used along with other configured information to locate the UE. In case of multiple satellite in view, the existing methods can be in principle used also in NTN. However, in the case of GEO, in general there is only one GEO satellite in view which is motionless. So it is impossible to guarantee there are multiple TPs/LMUs used in the measurement.

The E-CID method also cannot be applied because of the large coverage of GEO satellite beams.

So the RAT dependent positioning methods do not work in the case of GEO satellite.

- RAT independent positioning methods

The RAT independent positioning methods are based on external systems/sensors and network assistance, such as A-GNSS, Sensor, WLAN, Bluetooth and TBS. The existing methods can be used in GEO case, but the UE is required to support LPP.

The key issue#4 focuses on the GEO satellite access for NB-IoT which could be used by NB-IoT device, considering it is very likely that the NB-IoT device does not support LPP, so the probability that the RAT independent positioning methods do not work is also high.

Based on the analyses above, the general principles of EPC-NI-LR can be reused when emergency situation happens for the UE using NB-IoT NTN via GEO satellite connecting to EPC. However the steps 2-4 in Figure 1 can only work when RAT independent positioning method is used and the LPP is supported by the target UE. The applied scenario for steps 2-4 is very limited.

1.3.2	Analyses on coarse location reporting via SMC procedure

Based on the following description in clause 5.4.3.1 in TS 24.301, the MME can initiate the SMC procedure in the following cases:

Furthermore, the network may also initiate the security mode control procedure in the following cases:

-	in order to change the NAS security algorithms for a current EPS security context already in use;

-	in order to change the value of uplink NAS COUNT used in the latest SECURITY MODE COMPLETE message as described in 3GPP TS 33.401 [19], clause 7.2.9.2;

-	in order to request the UE radio capability ID from the UE; and

-	in order to request the UE to report the coarse location information.

The SMC procedure can be used by MME to request UE coarse location without requiring the UE to support LPP capability. However, based on TS 36.331, the coarse location can meet the accuracy requirement which corresponds to a granularity of approximately 2km. So in emergency case, whether the accuracy of 2km can fulfil the requirement (i.e. determine the proper PSAP destination as described in TS 23.167) or not needs further check with SA1.

1.4	Conclusion

Based on analyses in clause 1.3, it is proposed to reuse the general logic of EPC-NI-LR procedure with the difference that the MME obtain the UE location via SMC procedure, instead of requesting UE location from E-SMLC.

2. Proposal

It is proposed to add a new solution for KI#4: Location service for IMS emergency call and regulatory services over NB-IoT NTN.

* First change *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.101: "Service aspects; Service principles".

[10]	3GPP TS 29.211: "Rx Interface and Rx/Gx signalling flows".

[11]	3GPP TS 29.213: "Policy and charging control signalling flows and Quality of Service (QoS) parameter mapping".

[12]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[13]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[14]	3GPP TS 24.294: "IP Multimedia Subsystem (IMS) Centralized Services (ICS) protocol via I1 interface".

[15]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[16]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[17]	3GPP TS 23.203: " Policy and charging control architecture".

[18]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[19]	IETF RFC 4566: "SDP: Session Description Protocol".

[20]	IETF RFC 5049: "Applying Signaling Compression (SigComp) to the Session Initiation Protocol (SIP)".

[a]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[b]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

* Second change *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| #1 | X | X |  |  |
| #2 | X |  |  |  |
| #3 | X |  |  |  |
| #4 | X | X | X | X |
| #5 | X |  |  |  |
| #6 | X |  |  |  |
| #7 | X | X |  |  |
| #8 | X |  |  |  |
| #9 | X |  |  |  |
| #10 | X |  |  |  |
| #11 | X |  |  |  |
| #12 |  | X |  |  |
| #x |  |  |  | X |

* Third change (all text new) *

6.X	Solution #X: EPC-NI-LR based location service for emergency situation

6.X.0	High level principles

The high level principles of the solution include:

-	Reuse the general logic of EPC-NI-LR procedure described in clause 9.1.17 in TS 23.271[a], as follows:

-	The MME is responsible to determine the emergency situation happens, i.e. after emergency Attach, UE requested emergency PDN Connectivity.

-	When the emergency situation is detected, the MME obtains UE location and sends the location to GMLC/LRF.

-	When the MME determines the emergency service is no longer active, the MME notifies the event (EMERGENCY_CALL_RELEASE) to GMLC/LRF.

-	The enhancement to the EPC-NI-LR procedure is if the UE is using NB-IoT NTN via GEO satellite connecting to EPC, the MME initiates the coarse location reporting via NAS Security Mode Command Procedure described in clause 4.13.4 in TS 23.401[5] to obtain UE location, instead of sending location request to E-SMLC.

Editor's note:	Based on TS 36.331[b], the coarse location can meet the accuracy requirement which corresponds to a granularity of approximately 2km. Whether the UE location with accuracy of 2km can support the function to support emergency service, i.e. the external emergency services client determines the proper Public Safety Answering Point (PSAP) destination based on the UE location as described in TS 23.167[7], needs further co-ordination with SA1.

6.X.1	Description

When an emergency situation (i.e. after emergency Attach, UE requested emergency PDN Connectivity) is detected by MME, the MME initiates the EPC-NI-LR procedure to obtain UE location and sends the UE location to the GMLC which is responsible to provide the UE location to an external emergency services client to determine the proper Public Safety Answering Point (PSAP) destination as described in TS 23.167[7]. When the MME obtains UE location, if the UE is using NB-IoT NTN via GEO satellite connecting to EPC, the MME initiates the NAS Security Mode Command procedure.

When the MME determines the emergency service is no longer active, the MME notifies the event (EMERGENCY_CALL_RELEASE) to GMLC/LRF.

6.X.2	Procedures

The procedure is shown in Figure 6.x.2-1.

Figure 6.x.2-1: Network Induced Location Request for EPC

The procedure in Figure 6.X.2-1 follows the EPC-NI-LR procedure defined in clause 9.1.17 in TS 23.271[a] with the following differences:

-	Step 2a is the original step 2 in the EPC-NI-LR procedure. The difference is that step 2a is changed to an optional step which is performed when the UE is not using NB-IoT NTN via GEO satellite connecting to EPC.

-	Step 2b is a new step added to the EPC-NI-LR procedure. This step is performed by MME when the UE is using NB-IoT NTN via GEO satellite connecting to EPC to request the UE to provide the location.

-	Step 3 is changed to an optional step which is performed if step 2a is performed.

-	Step 4a is the original step 4 of the EPC-NI-LR procedure. The difference is that step 4a is changed to an optional step which is performed if the step 2a is performed.

-	Step 4b is a new step added to the EPC-NI-LR procedure. This step is performed if step 2b is performed. In this step, the UE provides location to MME.

6.X.3	Impacts to Services, Entities and Interfaces

MME:

-	When the emergency situation is detected, if the UE is using NB-IoT NTN via GEO satellite connecting to EPC, the MME obtain UE location via NAS Security Mode Command Procedure.

* End of changes *