SA WG2 Meeting #171	S2-2508883

13 -17 October, 2025, Wuhan, China                                              (Revision of S2-25088136)

Source:	Ericsson [, InterDigital Inc., LG Electronics (?), vivo (?), Apple (?), CEWiT (?), IITB (?),Turkcell (?), Nokia, T-Mobile USA (?), CATT (?), Xiaomi (?), Huawei (?), NEC]

Title:	[WT#1.2, Exposure Framework] Key Issue and WT update for 6G Exposure Framework

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides more details about what can be studied within Work Task #1.2 on the topic of Network Exposure Framework.

# 1. Discussion

This postponed paper S2-25088136 is a merge of the following documents and SA2#170 meeting discussions.

| AI | TD# | Type | Doc For | Subject | Source | Rel | Work Item |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Network Exposure |  |  |  |  |  |  |  |
| 20.6.1.2 | S2-2506522 | P-CR | Approval | 23.801-01: [WT#1.2, Network Exposure Framework] Network Exposure Framework | InterDigital Inc. | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2506625 | P-CR | Approval | 23.801-01: [WT#1.2, Exposure Framework] Scope and New Key Issue for Exposure framework | LG Electronics | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2506640 | P-CR | Approval | 23.801-01: [WT#1.2, Network exposure] Support for network exposure to UE in 6G network | vivo | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2506802 | P-CR | Approval | 23.801-01: [WT#1.2, Network Exposure] Scope and KIs definition for Network Exposure in 6G | CEWiT | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2506863 | P-CR | Approval | 23.801-01: [WT#1.2] Proposal on a common network exposure framework with in-band information exposure support | Nokia | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2506890 | P-CR | Approval | 23.801-01: [WT#1.2, Network Exposure Framework] Key issue for 6G network exposure | Apple | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2506933 | P-CR | Approval | 23.801-01: [WT#1.2, Network Exposure] Exposure Framework architecture in 6G | Ericsson, T-Mobile USA | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2507108 | P-CR | Approval | 23.801-01: Intelligent network exposure | TURKCELL | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2507195 | P-CR | Approval | 23.801-01: WT#1.2, 6G Exposure Framework and study proposal | CATT | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2507363 | P-CR | Approval | 23.801-01: [WT#1.2, network exposure] WTs and KIs for network exposure framework | Xiaomi | Rel-20 | FS_6G_ARC |
| 20.6.6 | S2-2506995 | P-CR | Approval | 23.801-01: [WT#6] Computing Enabler service, coordination of communication and computing service | Huawei, HiSilicon | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2507305 | P-CR | Approval | 23.801-01: [WT#1.2, QoS Framework] QoS Framework Enhancements | Huawei, HiSilicon | Rel-20 | FS_6G_ARC |
| 20.6.3 | S2-2507223 | P-CR | Approval | 23.801-01: [WT#3, Network AI Agent] Agentic core network for 6G | Huawei, HiSilicon | Rel-20 | FS_6G_ARC |

This document makes additional updates and clarification on the postponed content as input to make further progress. Change marks are shown compared to postponed baseline document.

**** First Change – All New Text ****

# Annex A.X. WT#1.2 Network Exposure Framework

One aspect of this work task is to study whether and how to support and/or enhance the network exposure framework in 6G. Aspects that can be studied include:

- X.1 How to support a common network exposure framework for the exposure of network capabilities

  - Whether and how to support a common framework for the exposure of network capabilities that is exposed to AFs. The exposure framework of the 5GS (i.e. exposure via the NEF) is assumed to be the starting point for discussion of this work task.

  - How to seamlessly support the co-existence of 4G, 5G and 6G accesses and determine the selection of 4G, 5G and 6G system domain via a common G-agnostic network exposure framework.

NOTE x: the Common Network Exposure Framework in SA2 does not overlap/duplicate with the SA6 CAPIF Framework.

NOTE 1:	Examples of network capabilities include information exposure such as UE related events, network events, analytics, and information provisioning such as UE related parameters, network configuration parameters, and service specific parameters.

  - How to support enhanced privacy protection, user consent, data anonymization, and service authorization mechanisms.

NOTE x+1: the privacy protection and user consent aspects is addressed by SA3 and SA6 working group.

NOTE 2:	This aspect of work task has dependency on Work Task #5 (Data Collection).

  - Whether and how to restrict Network Exposure procedures in roaming, aiming to avoid information being sent between the home network and visited network when not allowed or when not necessary.

NOTE 3: 	The scope of this work task is to specify the common exposure framework, including possible impact to NEF. This work task will not address the information being exposed defined by other work tasks. Other work tasks can cover services that have an exposure component (e.g. computing and sensing). The exposure framework that is developed in this work task should be adaptable to and suitable for those other services (e.g. computing and sensing).

NOTE 4:	What capabilities are exposed in-band will be coordinated with the work that is done under other work tasks.

- X.2 How to support network capabilities exposure to UE.

- Whether and how network capabilities should be exposed to a consumer on the UE, considering their untrusted environment and limiting exposure to the hosting UE’s context.  Study which network capabilities and what use cases will be beneficial to support network capabilities exposure to UE.

- Whether and how to authorize consumers of network capabilities when the consumer is resident on the UE.

- Whether and how to support and authorize the UE request for network information exposure.

- X.3 How to support on-path Information Exposure.

  - Whether, what and how to expose information in-band between application endpoints and 6G network in an operator-controlled manner.

  - Whether the exposure/reporting path of an exposed capability needs to be optimized (e.g. in-band or not in-band) and if so how.

  - How to discover, authorize and activate in-band exposure services offered by the network and whether and how to integrate on-path exposure in the common exposure framework.

Note x:  "on-path" refers to exchanging the information on the same path as the application traffic, share the same definition (i.e. on-path) with the paper for the user plane of Work Task 1.2.

Note x: bidirectional information exposure exchange between network and application endpoints

NOTE 5:	In-band information exposure will not replace existing NEF APIs, e.g., for parameter provisioning.

NOTE 6:	This work task will cover how capabilities and information are exposed in-band. What capabilities and information are exposed on path  will be coordinated with the work that is done under the User Plane and QoS aspects of Work Task 1.2.

- X.4 Considerations for Intent-Based Exposure.

  - If, as a result of work that takes place under other work tasks (e.g. WT#3), it is decided that intent-based exposure is supported in the 6G system, what capabilities, if any, are needed in the network exposure framework to support intent-based exposure. The scenarios for intent based exposure (e.g. the goal of an intent) will be defined by the work that is done under Work Task #3. This wok task will study how the procedures on the southbound interfaces can achieve any intents that are identified within work task #3.

NOTE 7:	Information exposure is based on operator policies/SLAs.

NOTE 8:	The goal of intent-base exposure is to make accessing exposed services more user friendly.

NOTE 9:	This aspect of work task may have dependency on Work Task #3. In terms of supporting intent, this work task will consider what functionality is need on the southbound interfaces to support intent based northbound APIs.

NOTE 10:	This work task requires coordination with SA3 (e.g. user consent, privacy and anonymization), SA6 (e.g. CAPIF), and SA5 (e.g. OAM Services).

NOTE 11:	This work task has dependency on service requirements to be specified by SA WG1.

**** End of Changes ****