SA WG2 Meeting #171	S2-2509225

October 13-17, 2025 Wuhan, China

Source:	Xiaomi

Title:	[WT#6, computing] supporting for computing service

Document for:	Agreement

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper is to provide justification and gap analysis for WT#6 computing, and propose the potential key issues

1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

SA1 6G study is working on the use cases and requirements for 6G system which covers the computing service (6G Computing Service: A service provided by 6G network utilizing computing resources in Service Hosting Environment, which can be used by a subscriber (via UE)/3rd party.). Up to August 2025, 19 use cases and more than 30+ potential requirements were agreed for computing service seen in TR 22.870, including:

- AI aspect in clause 6, 11 use cases and 17+ potential requirements are related for computing service, including, 6G network offload the traffic/workload from UE/3rd party to network, computing resource selection, QoS, etc.

- Immersive service aspect in clause 9, 5 use cases and 11+ potential requirements are related for computing service, including, network provides computing service to UE or 3rd party for rendering/computing task, NW maintains the computing information of 3rd party, and coordinate with UE to support the computing service, etc.

- Other computing aspect in clause 11, 3 use cases and 6 requirements are agreed for computing service, including, 6G system provides computing service to user(via UE), security and privacy when providing computing service, sharing the information of computing service within service hosting environment, expose information related to computing service to an authorised 3rd party.

SA2 Study on Architecture for 6G System was approved in SP-250806, which included the WT#6 “computing” as following:

WT#6: Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

NOTE 7: Application layer mechanism and exposure framework may require coordination with SA6.

Gap Analysis: What are the limitations or gaps in existing architecture (from Rel-15 to Rel-20) that this proposal aims to resolve?

In order to support the edge computing and its deployment together with 5GS, some enablers have been specified since Release 15 as described in clause 5.13 in 3GPP TS 23.501, e.g. LADN, local access to a DN by locally deployed UPF (supporting  UL CL or Branching Point) utilizing Local Routing and Traffic Steering, User plane (re)selection, and AF influenced traffic routing. In Rel-17, Rel-18 and Rel-19 further enhancements are specified for supporting Edge Computing, including discovery and re-discovery of EAS, edge relocation, and other optimization of traffic delivery, etc.

in 5G system, Edge computing is supported to enable operator and 3rd party services to be hosted close to the UE's access point of attachment, so as to achieve an efficient service delivery through the reduced end-to-end latency and load on the transport network. 5GS supports Edge Hosting Environment (EHE) deployed in the DN beyond the PSA UPF. An EHE may be under the control of either the operator or 3rd parties.

However, in order to support new services (e.g., AI, immersive service) with high computing requirements in 6G system, 5G system can’t provide its computing resources to UE or 3rd party. In 6G, it is possible for NMO to leverage operator’s efficient and powerful computing resources to offload the computing task to network from UE or 3rd party, by closely coordination between UE/NW and 3rd party.

Technical Impact: How would this WT/KI influence:

- WT#1, Overall system architecture, e.g., to define NF/control signalling to support computing service, network exposure framework to expose computing resources.

- WT#2, interworking and migration, to enable 6G system to manage the computing resources between 5G network and 6G network.

- WT#3, AI aspect, e.g., how to use the computing service to serve AI service.

**** First Change ****

Annex A.X. WT#X Scope supporting for computing service

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#X: Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

NOTE 7: Application layer mechanism and exposure framework may require coordination with SA6.

- Architecture and function(s), interface(s) design for 6G system to support for computing service.

- authorization/authentication of computing service for AF or UE, e.g., whether and how to authorize UE/AF to request the computing service, etc.

- Coordination and management for Computing resources, including computing resource information collection and exposure when needed, discovery and selection of the computing resource per requested by UE/AF, etc.

- Service continuity when re-select the computing resources, due to computing load balance, UE mobility, etc.

**** Second Change ****

5.X. Key Issue #X: supporting for computing service

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

This key issue will study the architecture and function(s) design for 6G system to support computing service:

- Study on whether and how to define the computing architecture, network function (s), interface(s) for 6G system to support for computing service.

- Study on whether and how to manage the computing resources from network domain and 3rd party domain, , including, Computing resource metrics definition, Computing resources registration; Computing resource information registration and exposure to authorized UE/AF;

- Study whether and how to discover and select the computing resources, and coordinate computing resources between UE, Network and 3rd party server to support the service(s) with high computing requirements, e.g., AI model training/inference service, immersive service, Sensing service, etc.

- Study on Exposure framework to offer computing service/resources to authorized (third-party) applications/UE.

NOTE X: Charing aspects will be in scope of SA5

**** End of Changes ****