3GPP TSG-SA2 Meeting #171	 S2-2508635

# Wuhan, China, October 13-17	2025															(revision of )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6438 | rev | 2 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on handling of application ID/predefined PDRs in reporting of usage data volume to EIF- option SMF is aware of App ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At SA2#170 the following papers S2-2506407 S2-2506408 S2-2506409 were submitted, proposing to shift from the PFCP based approach on N4 for reporting of usage data, at the granularity requested by the EIF, to the SMF and then onwards to the EIF, to the SBI-based UPF exposure service to EIF. The reason to do this was quoted to be the fact that for the case of predefined PDRs (e.g. based on application identifier) the SMF is not aware of the actual Packet Filters, and only the UPF is. In fact the supporting company believes the SMF can be made aware of Application IDs and Predefined PDRs semantics and then apply logic to resolve any conflict with PDRs as part of the SDF level reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the solution assumes that the SMF can be Application ID semantics aware. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of SDF/Application ID level reporting in rel-19 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.51.2.2.2	Energy Consumption information collection from SMF

The serving SMFs are retrieved by the EIF from the UDM of the UE based on the provided input parameters including the UE ID and optional S-NSSAI or (S-NSSAI, DNN). Also, the EIF subscribes to the UDM to be notified with the applicable SMFs.

The EIF invokes Nsmf_EventExposure_Subscribe as defined in TS 23.502 [3] with the required granularities (UE ID, DNN/S-NSSAI, application information (i.e. Application Identifier, or Flow Info (i.e. Packet Filters)) to retrieve the information from SMF, which is shown in Table 5.51.2.2.2-1.The SMF receives the data volume of the required granularities from the PSA UPF. The SMF derives the gNB ID(s) from the AMF provided ULI as described in clause 4.3.2. If the I-SMF is involved in the PDU session, the I-SMF reports to the SMF of PDU session via Nsmf_PDUSession_Update the current I-UPF ID and subsequent change of the I-UPF ID(s). If the I-SMF is involved in the PDU session, the SMF of PDU session requests the I-SMF via Nsmf_PDUSession_Update to report the current ULI and subsequent change of ULI only when the gNB ID changes.

The SMF then sends the collected data volume, along with the serving gNB ID(s) and (I-)UPF ID(s) to EIF for energy consumption calculation. The information collected from SMF by EIF, is shown in Table 5.51.2.2.2-2. Upon release of a PDU session, the SMF terminates subscriptions related to the PDU session. The SMF periodically notifies EIF with the information at the end of each time interval T until the EIF unsubscribes as defined in TS 23.502 [3]. The SMF shall collect data volume only for the PDU session carried over the 3GPP Access in 5GS. When the PDU session is over Non-3GPP Access type, or it is a PDN connection over EPS handled at a SMF+PGW-C, the SMF reports UL/DL Data volume in the measurement period equal to 0 (zero) and the gNB ID is not provided.

NOTE 1:	The SMF can derive gNB ID(s) according to the AMF provided ULI as described in clause 4.3.2 of TS 23.502 [3].

In the case of MA PDU session, the SMF shall not include in the report the UL/DL Data volume related to the traffic over the Non-3GPP access leg and the serving gNB ID refers to the traffic over the 3GPP leg.

When the SMF receives the Nsmf_EventExposure_Subscribe with application information (i.e. Application Identifier, or Flow Info (i.e. Packet Filters)) from the EIF for energy consumption, the SMF generates a URR for the Data Volume counting with a Periodic measurement threshold set according to the Timer interval T and associates the subscription from EIF to the generated URR. The SMF is then associating the generated URR to an appropriate PDR. SMF logic ensures that the PDRs and their precedence values are configured appropriately in the UPF so that both, the URRs for Data Volume counting for EIF and the traffic handling instructions derived from the existing PCC rules can co-exist and the traffic  is treated in the same way as before. For example, when the SMF receives the Nsmf_EventExposure_Subscribe with application information, the SMF may behave as follows:

-	If there is an existing PCC rule for exactly the same Service Data Flow as the application information in the energy consumption request (i.e. with same Application Identifier or same Packet Filters), the SMF associates the generated URR with the PDR of that PCC rule.

-	If there is no PCC rule matching with the application information in the energy consumption request (i.e. the Application Identifiers are different or no packet filter overlap exists) but a match-all PDR has been provisioned for the PDU Session, the SMF generates a PDR for the application information and associates the generated URR with that PDR. All other instructions of the PDR related to the match-all PCC rule are copied (so that the traffic is treated in the same way as before).

NOTE x1:	The SMF cannot generate a PDR due to the energy consumption request and request the UPF to forward the matched traffic if the same is not allowed by the PCF or the local policy. If there is no match-all PDR provisioned for the PDU Session, any application traffic without being authorized by the PCF or the local policy has to be discarded.

-	If the application information in the energy consumption request is partially overlapping with the Service Data Flow(s) of (one or more) existing PCC rule(s) other than the match-all PCC rule (which can only occur when the subscription contains Packet Filters), the SMF may need to generate two or more PDR(s) and, for the PDR(s) with overlapping filters, associate the generated URR with the(se) PDR(s) and copy all other instructions of the PDR from the existing PCC rule(s) with overlap and for the PDR with the no overlapping filters, copy all other instructions of the match-all PCC rule so that both, the Data Volume counting for EIF and the traffic handling instructions derived from the existing PCC rule and the match-all PCC rule can be realized.

NOTE x: one or more of the above can apply to an energy consumption request.

NOTE w: For MA PDU Sessions, the generated URRs shall be included specific for the 3GPP access in the Multi-Access Rule (see 5.8.5.8).

If the PCC rules are changed, the SMF shall again perform the above checks and actions.

The same type of application information (Application Identifier or Flow Info) should be used by EIF in the subscription request as it is used in the corresponding  PCC Rule(s) for service data flow detection.

NOTE y:	SMF may be unable to determine the PCC rule and PDRs to update if the PCC rule includes Application Identifier and EIF provides Packet Filters and the other way around.

If predefined PCC rules (see clause 5.8.2.8.1) associated in SMF to  packet detection rules locally defined in UPF (see TS 29.244 [65]) apply in the deployment, the procedure described above requires that the SMF shall also be aware of the semantics of such rules in order to behave as described above and to avoid conflicts if they arise.

When the SMF receives the Nsmf_EventExposure_Subscribe for the PDU Session from the EIF (i.e. no application information is provided by the EIF), the SMF associates the subscription from EIF to the PDU Session, generates a URR for the Data Volume counting (with a Periodic measurement threshold set according to the Timer interval T) and associates it with all PDRs of the PDU Session.

In both cases, the UPF will send the collected data volume information of this URR at the end of each Time interval to the SMF in a Usage Report (see clause 4.4.2.2 of TS 23.502 [3]). If there is a change in the user plane path (e.g. handover between gNBs or insertion/removal of I-UPF) during the Time interval, the SMF shall request the collected data volume information of this URR from the UPF. If this happens, the UPF will immediately send the collected data volume information to the SMF in a Usage Report (see clause 4.4.2.2 of TS 23.502 [3]) and the SMF shall then store this information together with the gNB ID and I-UPF ID that belong to the user plane path before the change happened.

Table 5.51.2.2.2-1: Information to SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE ID | SUPI. |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU Session. |
| Application Identifier or Flow Info. | Information shall be reported with granularity of Service Data Flow (traffic is identified by service data flow detection). |

NOTE 2:	The user-plane energy consumption information reporting interval from the SMFs is the network-wide configurable starting time and interval T.

Table 5.51.2.2.2-2: Information from SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE IP address | UE IP address. |
| UE ID | SUPI. |
| S-NSSAI | Network Slice applicable to a UE |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session. |
| Packet Filters | Packet Filters as in clause 5.7.6 for IP or Ethernet traffic. |
| Application Identifier | Identification for the traffic of the service data flow. |
| List of Data Volume information | The data volume and the associated UPF(s) and gNB(s) serving the UE within the time interval. |
| > UL/DL Data Volume | The UL/DL Data Volume of a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a Service Data Flow (UE ID, S-NSSAI, DNN, Packet Filters/Application Identifier). |
| > (I-)UPF ID(s) | Identifier of any (I-)UPF(s) associated to a reported data volume used by a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a Service Data Flow (UE ID, S-NSSAI, DNN, Packet Filters/Application Identifier). |
| > gNB ID | Identifier of the gNB serving the UE. |
| Reference to Time Interval | Indicate the time interval of the collected information (e.g. time stamps). |

NOTE 3:	Each entry of List of Data Volume information represents the serving gNB and UPF(s) corresponding to the same Data Volume regarding the required granularities. A new entry is added when the gNB or UPF(s) is changed.

End of changes