SA WG2 Meeting #171	S2-2508543

Oct. 13-17, 2025, Wuhan, China	(revision of S2-2508121)

Source:	[Futurewei (pen holder), InterDigital Inc., Ofinno, Vivo, China Telecom, Ericsson, Cisco Systems, Deutsche Telecom, LG Electronics, Huawei, HiSilicon, Qualcomm, CATT, VODAFONE, NEC, AT&T, TNO], CMCC

Title:	[WT#1.2, Enhanced User Plane and QoS] Scope and Key Issue for Enhanced User Plane and QoS

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution discusses areas to study for enhanced user plane and QoS mechanisms that enable high scalability for user plane transport and functional entities, and mechanism to provide QoS that is adaptable over the life of flows.

- Justifications

The changes proposed in this pCR are based on the postponed baseline document S2-2508121.

This pCR are comprised of the discussions and revisions for the items, i.e., the enhancement of application-network collaboration (#1 from the previous WT#1.2), and flexible user plane & handling (#4 and #5).

- Enhancement of application-network collaboration

The collaborations between applications and networks span both ways, i.e., the way for UE/AS to offer App-level information for network to adapt reactively to the variations of application traffic and the other way for network to provide proactively advisory information, based on the network runtime states, to UE/AS so that UE/AS reacts accordingly. The existence of large amount of (bursty) media flows, with high-throughput and low-latency requirements, over mobile network further strengthens the demand to have network feedback proactively to endpoints (UE/AS). While many technologies may exist, a particular protocol suitable for mobile network, SCONE, is being standardized in IETF that can leverage on-path network elements (e.g., UPFs) to provide the network’s perspective (i.e., advisory information) to endpoints.

In summary, the 6G UP should accommodate the application-network collaborations for both ways.

- Flexible user plane & handling

The 6G user plane should not only enhance the performance & capability of the mobile communication system (or MCS) itself, but also consider the path characteristics between access network and data network, i.e., including the capability of the transmission network in between. While a MCS may involve diversified UP entities, e.g., (I-)UPF, (central/local) PSA-UPF, and distributed UPF, that surely lead to the UP flexibility, the end-to-end (E2E) path from UE to AS, spanning (the MCS) AN/CN, the transmission network/TN and the data network/DN, introduces more variations that will impact how flexible a UP network entity (e.g., UPF) may select a path and then handle traffic accordingly.

In summary, the 6G UP should have the flexibility to steer traffic to achieve the optimal E2E path performance, with the consideration of (the 6G) AN/CN, the transmission network/TN and the data network/DN.

# **** First Change ****Annex A.X. WT#1.2 User Plane Architecture for 6G Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

[WT#1.2 UP Architecture] In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied:

- Whether and how to enhance application-network collaboration for 6G user plane traffic to enable the UE/AS to provide the network with information that allows the network to adapt to application traffic as well as enable the network to provide the UE/AS with information that allows the UE/AS to react to network operation states.
Editor’s Note: whether this issue belongs to UP or QoS is FFS.

- Whether and how to enhance CP-UP interaction for better interoperability.

- Whether and how to extend the service-based architecture for the user plane control interface to UPF.

- Whether and how to support flexible user plane (i.e., UPF (re)selection) for different service requirements considering path performance and capability between access network and data network.

- Whether and how to enhance the user plane handling for session continuity and mobility between access network and data network (i.e., best path selection considering ULCL and PSA-UPF path metrics, and/or for specific services).

NOTE： Whether and how the capability of the transmission network impacts the user plane flexibility will be considered as part of the study.

**** Second Change ****

# 5.X. Key Issue #X Enhanced User Plane Architecture for 6G

**** End of Changes ****