3GPP TSG-SA WG2 Meeting #171	S2- 2508768

13 – 17 October, 2025, Wuhan, China

Title:	23.700-14: Interim Agreements for Key Issue#5: Sensing Result Exposure

Source:	ETRI

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract:	This contribution analyses candidate solutions for sensing result exposure (Sol#1, 2, 3, 19, 23, 27–33, 37) and proposes agreed principles for KI#5 in TR 23.700-14.

Proposal

The study of sensing result exposure in 6G has produced a range of solutions (Sol#1, 2, 3, 19, 23, 27–33, 37). These solutions address different aspects of how sensing results can be requested, authorized, delivered, and consumed by authorized entities. Key observations are:

-	Exposure to authorized consumers: Sol#1, Sol#2, Sol#27, Sol#28, and Sol#30 propose mechanisms to expose results to trusted Application Functions (AFs) directly or to untrusted AFs via the Network Exposure Function (NEF), ensuring authorization and security. This demonstrates the fundamental requirement for controlled exposure mechanisms.

-	Multiple reporting modes: Sol#2, Sol#3, Sol#19, Sol#27, Sol#28, Sol#30, and Sol#31 support one-time, periodic, and event-triggered reporting, indicating consensus that diverse modes are required.

-	Processed results: Sol#23, Sol#31, Sol#33, and Sol#37 propose exposing aggregated, pre-processed, or AI/ML-assisted results to enable efficiency and usability.

-	Real-time and subscription/notification exposure with QoS: Sol#29, Sol#30, Sol#32, and Sol#33 mechanisms to differentiate exposure types and to ensure QoS/SLA guarantees, which are critical for latency-sensitive applications.

| Solution | Description | Addressed Issue | Principle |
| --- | --- | --- | --- |
| Sol#1 | Initial sensing service framework | A basic framework to expose sensing results to consumers | Principle 1: Exposure to authorized consumers |
| Sol#2 | Basic procedures for sensing data/result exposure | One-time sensing result exposure procedure (via NEF) | Principle 1: Exposure to consumers Principle 2: One-time reporting |
| Sol#3 | Enhancements to sensing result delivery | Support repeated/periodic result exposure | Principle 2: Periodic reporting |
| Sol#19 | Edge-related sensing data exposure | Event-triggered exposure at the edge for low latency | Principle 2: Event-triggered reporting |
| Sol#23 | Distributed sensing data processing before exposure | Expose pre-processed or aggregated sensing results | Principle 3: Aggregated or pre-processed results |
| Sol#27 | Sensing service request & result exposure | Request/authorization procedure and multiple reporting modes | Principle 1: Exposure to consumers Principle 2: Multiple modes |
| Sol#28 | Direct exposure of sensing results to AF | Direct SF to AF exposure across reporting modes | Principle 1: Exposure to consumers Principle 2: Multiple modes |
| Sol#29 | Sensing result exposure types | Differentiate exposure types: real-time vs subscription/notification | Principle 4: Real-time and subscription/notification exposure |
| Sol#30 | Exposure to trusted & untrusted AFs | Handle trusted vs untrusted AFs; add contextual info | Principle 1 Exposure to consumers Principle 2: Multiple modes |
| Sol#31 | Aggregated sensing results | Aggregated result exposure from multiple sensing sources | Principle 3: Aggregated results |
| Sol#32 | Exposure with reliability & QoS | Ensure QoS/reliability for exposure (SLA support) | Principle 4: Real-time/subscription exposure with QoS |
| Sol#33 | Multi-domain sensing result exposure | Cross-domain exposure with contextual info | Principle 3: Cross-domain aggregation Principle 4: Real-time exposure |
| Sol#37 | AI/ML-assisted exposure (NWDAF-based optimization) | AI/ML-assisted optimization of exposed results | Principle 3: AI/ML-assisted processed results |

Based on these findings, agreed principles have been derived. It is proposed to approve the following Interim Agreements for Key Issue#5 in TR 23.700-14.

* * * First Change * * * *

7.1.Y	Agreed Principles for KI#5: Sensing Result Exposure

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

To support KI#5 of Sensing Result Exposure, the following principles are concluded:

Principle 1: The system supports exposure of sensing results to authorized consumers (e.g., AFs, NFs). For untrusted AFs, the exposure is mediated via the Network Exposure Function (NEF). (Sol#1, Sol#2, Sol#27, Sol#28, Sol#30)

Principle 2: The system supports multiple reporting modes, including one-time, periodic, and event-triggered reporting. (Sol#2, Sol#3, Sol#19, Sol#27, Sol#28, Sol#30, Sol#31)

Principle 3: The system supports exposure of processed sensing results, including aggregated, pre-processed, or AI/ML-assisted results, complemented by contextual information (e.g., time, location, and confidence levels). (Sol#23, Sol#31, Sol#33, Sol#37)

Principle 4: The system supports both real-time and subscription/notification-based exposure of sensing results, with QoS and reliability considerations. (Sol#29, Sol#30, Sol#32, Sol#33)

NOTE 1: Privacy and security aspects related to sensing result exposure shall be coordinated with SA3.

NOTE 2: Charging and OAM implications of exposing sensing results shall be coordinated with SA5.

* * * End of Changes * * * *