SA WG2 Meeting #171	S2-2509140

13 – 17 Oct, 2025, Wuhan, China

Source:	IIT Bombay

Title:	[WT#1.1, all topics] Scope and Key Issue proposal for UE-Core Network Interaction

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution discusses the Scope of WT1.1 and related Key Issue for generic mechanisms for UE-Core Network interaction to support operator services

# 1. Justifications

Work task 1.1 scope as agreed in 6G System Study item S2-2506096:

1.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

The 6G system envisions supporting a wide range of scenarios such as massive connectivity, ubiquitous connectivity and hyper reliable/low-latency communication as well as diverse UE types with varying capabilities. It is also expected to support new services such as AI/ML, sensing and computing. Consequently, 6G system needs to cater to broader service requirements and handle the enormous surge in the number of users which could lead to a significant rise in signalling load. It is also imperative to foresee the possible increase in complexity of the control plane with the introduction of new services.

In this context, it becomes essential to revisit the 5G architecture and examine its inherent limitations in UE-Core Network interactions and signalling exchanges over the control plane. Further, there is a need to explore generic mechanisms for UE-Core Network interaction (For Example, over the user plane / a new service plane)
to enable the 6GS to be flexible and to efficiently support all the services.

**** First Change ****

# Annex A.X. WT#1.1 Scope

WT#1.1 Study the support of control signalling for 6G System including the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

b. Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

NOTE: 	It is assumed that this WT covers impacts to 6G System procedures for functionalities such as mobility management, session management and NAS identifiers.

**** Second Change (New Text) ****

# 5.X. Key Issue #X Aspects for non-access stratum functionality

This Key issue studies the following aspects:

1)	Whether and how to enable the introduction of new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

NOTE: Coordination with CT1 could be required

2)	NAS termination, transport of services via NAS and NAS messages routing

3)	Whether and how to identify a minimum, common set of NAS functionalities

4)	Study system procedures for non-access stratum functionality, UE-CN signalling procedures, including impact to RAN-CN interface and 6G CN interfaces related  to these system procedures.

NOTE 1: Any impact on the UE and RAN interaction aspects will be coordinated with RAN3 and the decision for RAN-Core interface will be aligned with RAN3.

NOTE 2: Any impact on the RAN and core network interaction aspects will be coordinated with RAN2.

NOTE3: Interworking and migration aspects, especially inter-PLMN aspects (e.g., roaming), where applicable, are covered by WT#2.

NOTE4: Potential security and privacy impacts require coordination with SA3.

NOTE 5: 	It is assumed that this key issue covers impacts to 6G System procedures for functionalities such as mobility management, session management and UE NAS identifiers.

NOTE 6: Coordination with KIs related to WT1.2, WT2, WT#3, WT#4, WT#5, WT#6 is required.

**** Third Change (New Text) ****

# 5.X. Key Issue #Y: Generic mechanisms for UE-CN interactions to support Operator Services offered by 3GPP System

This Key Issue focuses on studying the support for UE-CN interactions for operator services offered by 3GPP System, including the following:

- Study whether and how to define generic mechanisms including architecture, procedures and UE-Core Network interactions to support all services offered by 3GPP system

  - Study whether user plane or a new service plane can be considered for UE-CN signalling procedures to support operator services

  - How to support transfer and routing of signaling (i.e., service-specific messages) for operator services between UE and the CN

  - Study how such a mechanism can minimize signalling to efficiently support diverse UE types

  - Investigate potential impact to RAN-CN interface and 6G CN interfaces related to the generic mechanism considered for UE-Core Network interaction

NOTE 1: The key issue includes studying mechanism for transporting signaling for operator services. It is not precluded that signaling for operator services may use transport over NAS

NOTE 2: Any impact on the UE and RAN interaction aspects will be coordinated with RAN3 and the decision for RAN-Core interface will be aligned with RAN3.

NOTE 3: Any impact on the RAN and core network interaction aspects will be coordinated with RAN2.

NOTE4: Potential impact on Interworking to be coordinated with KIs related to WT#2.

NOTE5: Coordination with KIs related to WT#1.2, WT#2, WT#3, WT#4, WT#5, WT#6 is required.

NOTE6: Potential security and privacy impact to be coordinated by SA3.

**** End of Changes ****