SA WG2 Meeting #172	S2-2510006

17 – 21 November, 2025, Dallas, US

Source:	Apple

Title:	KI#4 Conclusion

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: KI#4 conclusion proposal based on agreed interim agreements.

# 1.	Discussion

This contribution proposes KI#4 conclusion proposal based on agreed interim agreements agreed at SA2#171.

Changes are highlighted in this section for clause 7 in track changes mode and proposed as all new text for clause 8 afterwards to avoid unnecessary discussion on two different clauses with the same contents.

### 7.1.X	Key Issue #4: Sensing Data and the Associated Information Collection and Transport

The following principles are agreed for KI#4:

Principle 1: Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurements to collect the Sensing data.

Principle 2: Sensing Entity can provide one-time, periodical Sensing Data to the Sensing Function.

NOTE 1: The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

Principle 3: Sensing data can be collected from one or multiple gNBs by the Sensing Function for the same sensing service request.

NOTE 2: To account for the possible deployment split of SCF and SPF, it should be considered that the SPF can be the functionality that collects the sensing data from the one or multiple gNB(s).

Principle 4: The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s).

NOTE 3: To account for the possible deployment split of SCF and SPF it should be considered that the SPF is generating sensing results based on the information provided by the Sensing Entity (gNB) node(s).

Principle 5: A direct interface for sensing data transport shall be specified between the Sensing Entity (SE) and the SPF.

# 2.	Proposal

It is proposed to agree the following changes in TR 23.700-14.

**** First Change (ALL NEW TEXT) ****

## 8.X	Key Issue #4: Sensing Data and the Associated Information Collection and Transport

The following principles are agreed for KI#4:

Principle 1: Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurements to collect the Sensing data.

Principle 2: Sensing Entity can provide one-time, periodical Sensing Data to the Sensing Function.

NOTE 1:	The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

Principle 3: Sensing data can be collected from one or multiple gNBs by the Sensing Function for the same sensing service request.

NOTE 2: To account for the possible deployment split of SCF and SPF, it should be considered that the SPF can be the functionality that collects the sensing data from the one or multiple gNB(s).

Principle 4: The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s).

NOTE 3: To account for the possible deployment split of SCF and SPF it should be considered that the SPF is generating sensing results based on the information provided by the Sensing Entity (gNB) node(s).

Principle 5: A direct interface for sensing data transport shall be specified between the Sensing Entity (SE) and the SPF.

**** End of Changes ****