3GPP TSG-SA WG2 Meeting #172	S2-2510082

Dallas, USA, November 17 - 21, 2025	(revision of S2-2508662)

Source:	Ericsson

Title:	KI#2, New Solution: Application-Specific Command

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: The contribution discusses and proposes a new solution for Application-Specific Command for KI#2.

1. Introduction

In TS 23.700-30, KI#2 was agreed in SA2#170 to support DO-A capable AIoT Devices:

## 5.2	Key Issue #2: Support of DO-A Capable AIoT Devices

This key issue will study the system architecture to support DO-A capable Ambient IoT Devices in Topology 1 and Topology 2.

The following aspects will be studied:

-	How the AIoT Device informs the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure) and what are the triggers for the DO-A capable device to inform the network of its presence.

-	Whether and how to consider power consumption of DO-A Capable AIoT Devices.

-	How an AIoT Device sends data to the AIOTF autonomously.

-	Support for routing the data received by AIOTF from an AIoT Device to an AF.

-	Whether and how to enhance the Inventory and Command procedures defined in TS 23.369 [3] to support DO-A capable AIoT Devices.

-	Naiotf, Namf and Nnef interface enhancements to support DO-A capable AIoT Device.

NOTE 1:	The conclusions from Key Issue #1 are the basis for supporting DO-A capable AIoT Devices in topology 2 in this key issue.

NOTE 2:	Coordination with RAN WGs is required.

This solution discusses and focuses on the following aspects of KI#2 by supporting application-specific command:

-	Whether and how to enhance the Inventory and Command procedures defined in TS 23.369 [3] to support DO-A capable AIoT Devices.

-	Naiotf, Namf and Nnef interface enhancements to support DO-A capable AIoT Device.

Update after SA2 CC on November 3rd: the Size of the Command Response message is to be provided by the AF, as the AIOTF is transparent to the Application-Specific Command.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-30:

* * * Start of Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | Key Issue #1 | Key Issue #2 |
| #1 |  | X |
| #2 |  | X |
| #3 |  | X |
| #4 |  | X |
| #5 |  | X |
| #6 |  | X |
| #7 |  | X |
| #8 |  | X |
| #9 |  | X |
| #10 |  | X |
| #11 |  | X |
| #12 |  | X |
| #13 |  | X |
| #14 |  | X |
| #15 |  | X |
| #16 |  | X |
| #17 |  | X |
| #18 |  | X |
| #19 |  | X |
| #X |  | X |

* * * Next Change * * * *

## 6.X	Solution #X: Application-Specific Command

### 6.X.0	High-level Solution Principles

The solution is based on the following general principles to support application-specific command:

-	The network supports the application-specific command by transmitting the command and command result in application layer containers between the AF and the AIoT devices.

### 6.X.1	Description

This solution addresses KI#2 and focuses on the following aspects to support application-specific command:

-	Whether and how to enhance the Inventory and Command procedures defined in TS 23.369 [3] to support DO-A capable AIoT Devices.

-	Naiotf, Namf and Nnef interface enhancements to support DO-A capable AIoT Device.

In Rel-19, the AIoT command service supports read, write and disable operations, which are the very basic operations for type 1 devices. The DO-A capable devices (type 2b/C devices) are more powerful compared with type 1 devices. They are not necessarily limited by those very basic operations.

RFID tags (espcially the active tags) may support vendor-specific commands, which makes the function implementation more flexible and communication more efficient. Similar ideas can be introduced for those DO-A capable devices.

The AF and the AIoT devices support application-specific command and the command result, which are transferred in application layer containers. The AIOTF and the NEF support the transfer of application layer containers in Command Request and Notify messages. The AIOTF and the AIoT devices support the application-specific command in the AIoT NAS layer.

### 6.X.2	Procedures

#### 6.X.2.1	Command Procedure for Application-Specific Command

Figure 6.X.2.1-1 Command Procedure

Figure 6.X.2.1-1 is the Command Procedure in clause 6.2.3 of TS 23.369 [3]. The additions to the Command Procedure are described as below:

- In step 1, in Nnef_AIoT_Command Request, the AF set the Command Type to application-specific command and include the command details in "application command information" parameter which is an application layer container. The AF also provides Size of the Command Response message.

-	In step 3, the NEF sends the Naiotf_AIoT_Command Request message to the AIOTF(s) which contains the application-specific command.

-	In step 4, the AIOTF performs AF authorizaiton to check whether the AF is authorized to send application-specific command.

-	In step 8, for application-specific command, the AIOTF constructs the NAS Application-Specific Command Request to the AIoT device. Within the NAS Request message, the "application command information" parameter is included. In the assistance information provided to the NG-RAN, the AIOTF uses Size of the Command Response message from the AF.

-	In step 9, the AIoT device receives the NAS Application-Specific Command Request in AS R2D message and executes the command within the "application command information" parameter.

-	In step 10, the AIoT device constructs the NAS Application-Specific Command Response message and includes the command execution result in "application command result" parameter which is also an application layer container. The AIoT device sends the NAS Application-Specific Command Response message to the AIOTF.

-	In step 12, the AIOTF includes the "application command result" as AIoT data in Naiotf_AIoT_Command Notify message.

-	In step 13, the NEF forwards the "application command result" as AIoT data in Nnef_AIoT_Command Notify message.

### 6.X.3	Impacts on Services, Entities and Interfaces

The following NFs are impacted::

-	NEF:

-	Supports a new Command Type for application-specific command and includes "application command information" (an application layer container) in Nnef_Command Request.

-	Support "application command result" (an application layer container) in Nnef_Command Notify message.

-	AIOTF:

-	Supports a new Command Type for application-specific command and includes "application command information" in Naiotf_Command Request.

-	Support "application command result" in Naiotf_Command Notify message.

-	AIoT NAS layer support for application-specific command

-	Enhance AF authorization for application-specific command from the AF.

-	ADM:

-	Enhance AF authorization data for the application-specific command

-	AIoT Device:

-	AIoT NAS layer support for application-specific command.

* * * End of Change * * * *