3GPP TSG-SA WG2#172	S2-2510124

Dallas, USA, 17-21 November 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	Conclusion for KI#1

Document for:	Discussion/Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposed the conclusion for KI#1.

# 1.	Discussion

This contribution proposed the conclusion for KI#1.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-04:

# >>>>BEGINNING OF CHANGES (all new text)<<<<

# 8	Conclusions

The conclusion for KI#1 is agreed as follows:

Architecture

- A new 5GC entity called DCF is defined as the UP path termination entity to support the transfer of data over UP for UE data. A UP path is used between the UE and DCF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session. The DCF will further transfer the received data to the AF for UE side model training. The DCF can be standalone or co-located with an existing function in 5GC.

- The DCF supports the following functions:

  - May receive the data transfer request from an AF which requires data for UE side model training.

  - Initiating the data transfer request to UE

  - Receiving data report from UE

  - Further transferring the received data to the AF for UE side model training

UP Data transfer trigger

The following triggers are supported optionally as below:

- AF may send the data transfer request which includes the requested feature ID (e.g., beam forming, .etc), reporting period, time window, geographic area, or

- AF may communicate UE about Data Transfer via application layer signalling which is out of 3GPP scope.

DCF discovery

If AF sends the data transfer request to 5GC, the NEF may discover the DCF via

- Local configuration in NEF, or

- NF discovery from NRF

UE list determination

The DCF determines the UE list for data transfer with the following optional information:

- Candidate UE list which provided by AF,

- UE in connected mode,

- User consent for each UE

UP path information Configuration

The following mechanism may be supported optionally as below:

- The FQDN of DCF is self-constructed in UE, UE discovers the DCF via DNS query, or

- The DCF sends the FQDN / IP address to UE via NAS signalling

Data Transfer Configuration

The following mechanism may be supported optionally as below:

- The DCF sends the Data Transfer Configuration to UE via UP signalling, UE negotiates the data collection configuration with gNB which will be defined in RAN2. UE may also reject the Data transfer due to implementation. or

- The DCF sends the Data Transfer Configuration to UE via NAS signalling, the DCF may also send the Data transfer assistance information to gNB, gNB may consider the Data transfer assistance information for data collection configuration. gNB may also reject the data transfer due to the UE radio capability.

Traffic differentiation

The following mechanism may be supported optionally as below:

- Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level. The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules. or

- The packet filter set (e.g., IP address / IPv6 prefix or port number of the DCF) is used in the QoS rule and the PDR to identify the packet flow used for UE data transfer.

UE data transfer report

- The standardized collected data is transferred to DCF via UP path, the format of the standardized collected data will be defined in stage 3.

- DCF forwards the received data to the AF without data handling.

# >>>>NEXT CHANGE<<<<

# >>>>NEXT CHANGE<<<<

# >>>>END OF CHANGES<<<<