3GPP TSG-WG SA2 Meeting #172	S2-2510237

November 17 – 21, 2025, Dallas, TX, USA

Source:	Ericsson

Title:	ROHC issue during talk and silent transitions

Document for:	Discussion

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: This contribution discusses the root cause of ROHC issue during talk and silent transition and proposes to solve the problem with new voice codec/SID design, rather than seeking for alternative of ROHC.

# Discussion

In SA2#171, the logs in discussion paper S2-2509144 showed a large ROHC packet UOR2 Ext3 is often used during talk/silent transitions. There are other scenarios listed when the UOR2 Ext 3 is used, but this paper focuses on the talk/silent transitions.

The problem:

- 	With ROHC IP/UDP/RTP profile to compress the voice packets, the protocol overhead of the voice media traffic is supposed to be very limited during a call, no matter talk or silent. In a voice call, the transition between talk/silent can happen very often. If many large ROHC packets (UOR2 Ext3) are required to handle such talk/silent transitions, it is not optimal, since compression lose its purpose.

When talking, a regular RTP packet includes voice speech frames. When silent, for the other party of the call to generate comfort noise, a special RTP SID (Silence Descriptor) packet is transmitted. How a SID packet is sent is described in different specifications copied here.

According to TS 26.093 (AMR) and TS 26.193 (AMR-WB), the SID packet is sent as below:

- SID_UPDATE frames shall be generated every 8th frame. The first SID_UPDATE shall be sent as the third frame after the SID_FIRST frame

But according to TS 26.445 (EVS), the SID packet can be sent in a flexible way:

- The CN parameters are transmitted at a fixed or adaptive rate during inactive signal periods using a command line parameter. By default in the command line the transmission rate of CNG update is fixed to 8 frames. However, the CNG update rate can also be set to another fixed value or a variable rate by means of a command line parameter. The fixed rate is limited to between 3 and 100 frames.

Observation 1: How SID packet is sent has dependency on what codec is used.

Both voice speech frames and SID packets are considered as RTP payload, and carried by RTP header.

- 	For RTP packets carrying voice frames, the sequence number and timestamp in the RTP header increase in the same pace for two consecutive RTP packets, e.g. sequence number increase by 1, and timestamp increase by 320 (16khz, 20ms).

-	For RTP packets carrying SID, the sequence number still increases by 1 between two consecutive RTP packets. However, the timestamp increment will depend on the SID interval as described in the above specifications between two consecutive RTP packets, which is usually larger than the increment between two consecutive voice RTP packets. In the AMR example, when a SID is sent every 8th frame, the increment of the RTP timestamp for SID packets will be 8 times compared to two consecutive voice RTP packets (assuming one voice frame per RTP packet).

The ROHC issue during talk/silent transition is about whether small/compressed ROHC packet can capture the timestamp delta for SID RTP packets.

When a SID packet is sent in a small interval and fixed rate (e.g. every 8th frame), the RTP timestamp of the SID packet can be captured by the ROHC UO-1 packet, with existing TS_STRIDE. The UO-1 format is copied below from RFC 3095. The TS field has 6 bits which can capture a range of small differences between (-15 to 48). In another words, UO-1 is able to handle SID packet intervals up to every 48th frames.

UO-1 packet is also visible in the logs shown by S2-2509144, when it is sent every 8th frame (2135 - 2127), where the delta (8) is within the UO-1’s capability.

But if the SID packet rate is changed to another value, for example from every 8 frames to every 50 frames, then the RTP timestamp delta between consecutive SID packets will exceed what a UO-1 packet is capable of, based on the existing TS_STRIDE. In such cases, the ROHC compressor has to fall back to FO state and suggest a new TS_STRIDE (larger than the old one) to the decompressor using UOR2 Ext3 packet.

With the new TS_STRIDE updated on both sides, the compressor can advance to SO state again and continue to use small ROHC packets to forward the SID (UO-0, not even UO-1, see picture from S2-2509144 below), because the large timestamp delta is covered by the new TS_STRIDE in the context parameter.

Observation 2: Large UOR2 Ext 3 is used to update TS_STRIDE in context parameter, because the SID interval is changed to a large value and the RTP timestamp delta between consecutive packets is no longer within the UO-1’s capability with the old TS_STRIDE.

# Conclusion

The ROHC issue for talk/silent transitions outlined in S2-2509144 is based on existing EVS codec, which will not be used for IMS over GEO NB-IoT satellite. It is thus not clear that the issue will appear in NB-IoT/GEO systems. However, it still demonstrates a good learning from the existing EVS codec, that is when SID rate is changed from small interval to large interval, it is very likely the RTP timestamp differences between consecutive packets can no longer be captured by small ROHC packet (e.g. UO-1). Then a large ROHC packet UOR2 Ext3 is needed to update context parameter (TS_STRIDE), in order to converge back to small ROHC packets.

Though the symptom is shown in the ROHC layer as large ROHC packet is triggered, the root cause resides in the upper layer application packet behavior (i.e. how SID packet is sent). EVS codec may have an overall better end user experience by utilizing flexible SID rate, but AMR/AMR-WB codec may survive better during such talk/silent transition in a limited physical bandwidth.

Conclusion:

The large ROHC packet (UOR2 Ext3) during talk/silent transition is caused by RTP SID packet behavior, but not a problem of ROHC itself.

# Proposal

With this finding in mind, SA4 should take this into account for the new codec design, particularly how the SID rate should be (e.g. fixed as AMR, AMR-WB) in case of the very limited GEO satellite link, so that it is not necessary to use large ROHC packets for handling a normal talk/silent transition.

For the talk/silent transition ROHC issue, instead of considering alternative solutions to ROHC, it is proposed to address the issue with a proper voice codec and SID design.