SA WG2 Meeting #172	S2-2510434

17 – 21 Nov, 2025, Dallas, US

Source:	Huawei, Hisilicon

Title:	DC3/DC4 Enhancement to Support Subscription, Notification and Instruction for P2A and P2A2P DC establishment

Document for:	Agreement

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / R20

Abstract of the contribution: This contribution is intended to update the solution#10 of Key Issue 3: Supporting dynamic configuration of MDC2 endpoints

1. Discussion

This pCR proposes to update the solution to the working task about the DC3/DC4 enhancement in Next Generation Real Time Communication Phase 3.

The following EN should be removed as those questions should be answered during evaluation phase.

Editor’s Note: Whether the explicit subscription is needed is FFS.

Editor’s Note: Whether the service operation type is Subscribe/Notify (and provided by DCSF) or Request/Response (and provided by DC AS) is FFS.

2. Text Proposal

It is proposed to capture the following changes into TR 23.700-56.

# * * * * First change * * * *

6.10	Solution #10: DC3/DC4 Enhancement to Support Subscription, Notification and Instruction for P2A and P2A2P DC establishment.

6.10.0	High-level solution Principles

This solution addresses Key Issue #3: Supporting dynamic configuration of MDC2 endpoints. This solution enhances the current P2A and P2A2P ADC establishment procedure defined in TS 23.228 [2] Annex AC.7.2.2 and AC.7.2.3.

6.10.1	Description

This solution requires the DC AS to subscribe to the call events from DCSF so that DC AS can give media instruction after receiving the notification from DCSF. This solution defines two services to enable DC AS to subscribe to the DCSF and to instruct DCSF.

This solution also enhances and unifies the timing of notifying DC AS in the current P2A and P2A2P ADC establishment procedure defined in TS 23.228 [2] Annex AC.7.2.2 and AC.7.2.3.

6.10.2	Service definition

6.10.2.1	General

The following table shows the DCSF Services defined.

Table 6.10.2-1: NF services provided by the DCSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcsf_EventExposure | Subscribe | Subscribe/Notify | NEF, DC AS |
|  | Unsubscribe | Subscribe/Notify | NEF, DC AS |
|  | Notify | Subscribe/Notify | NEF, DC AS |
| Ndcsf_MediaControl | MediaInstuction | Request/Response | NEF, DC AS |

6.10.2.2	Ndcsf_EventExposure

6.10.2.2.1	General

Service description: This service enables the consumer to subscribe and to be notified when the given event is detected.

6.10.2.2.2	Ndcsf_EventExposure_Subscribe

Service operation name: Ndcsf_EventExposure_Subscribe

Description: This service operation enables consumers to subscribe to specific events.

Inputs, Required: Notification Target address, Event ID

Three Event ID is currently defined: application data channel establishment request, application data channel establishment success, application data channel establishment failure.

Inputs, Optional: Target list of User IDs (IMPUs), list of Application binding information

When no IMPU is given, the notification will be sent to the consumer for all users.

When no application binding information is given, the notification will be sent to the consumer for all IMS DC applications.

Outputs, Required: Result indication, Subscription correlation ID when the subscription is accepted.

Outputs, Optional: None.

6.10.2.2.3	Ndcsf_EventExposure_Unsubscribe

Service operation name: Ndcsf_EventExposure_Unsubscribe

Description: This service operation enables consumers to unsubscribe from the subscription.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

6.10.2.2.4	Ndcsf_EventExposure_Notify

Service operation name: Ndcsf_EventExposure_Notify

Description: This service operation enables the DCSF to notify the comsumer about the subscribed event.

Inputs, Required: Session ID, Event ID

Inputs, Optional: User IDs of calling party and called party, application binding information.

Outputs, Required: Result.

Outputs, Optional: None.

6.10.2.3	Ndcsf_MediaControl

6.10.2.3.1	General

Service description: This service enables the consumer to give media instruction to DCSF. In this release, the instruction is only used to establish P2A and P2A2P data channel.

6.10.2.3.2	Ndcsf_MediaControl_MediaInstruction

Service operation name: Ndcsf_MediaControl_MediaInstruction

Description: This service operation enables the consumer to provide instruction to the DCSF.

Inputs, Required: Session ID, Media information of MDC2 interface.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

6.10.3	Procedure

6.10.3.1	Service Subscription Procedure

Figure 6.X.3.1-1: service subscription procedure.

1.	DC AS sends Ndcsf_EventExposure_Subscribe to DCSF to subscribe to specific ADC establishment event. The DC AS can find the DCSF using NRF or configuration.

2.	DCSF sends 200 OK.

6.10.3.2	P2A Application Data Channel Establishment Procedure

Figure 6.10.3.2-1 depicts the P2A application data channel establishment procedure.

Figure 6.10.3.2-1: P2A ADC establishment procedure.

1.	Steps 0-3 of TS 23.228 [2] Annex AC.7.2.2 applies.

2.	The DCSF receives notification from IMS AS and detects this is an application data channel establishment request. The DCSF decides to notify DC AS based on the subscription from DC AS.

3.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that an application data channel establishment request is received.

4.	The DC AS sends 200 OK indicating it receives the notification.

5.	The DC AS sends Ndcsf_MediaControl_MediaInstruction to instruct the DCSF about the MDC2 related media information.

6.	The DCSF sends 200 OK indicating it receives the instruction.

NOTE:	It is up to operator’s configuration and implementation that how the DCSF knows that the DC AS will send instruction to the DCSF. A possible implementation is that the DCSF will continue the following steps after a fix timer is expired.

7.	Steps 11-20 of TS 23.228 [2] Annex AC.7.2.2 applies.

8.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that the application data channel establishment success event. The DCSF will also forward MDC2 related information from the MF to the DC AS.

9.	The DC AS sends 200 OK indicating it receives the notification.

10.	Steps 21-27 of TS 23.228 [2] Annex AC.7.2.2 applies.

6.10.3.3	P2A2P Application Data Channel Establishment Procedure

Figure 6.10.3.3-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.10.3.3-1: P2A2P Application Data Channel Establishment Procedure.

1.	Steps 0-3 of TS 23.228 [2] Annex AC.7.2.3 applies.

2.	The DCSF receives notification from IMS AS and detects this is an application data channel establishment request. The DCSF decides to notify DC AS based on the subscription from DC AS.

3.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that an application data channel establishment request is received.

4.	The DC AS sends 200 OK indicating it receives the notification.

5.	The DC AS detects that this is a P2A2P ADC so it allocates two sets of MDC2 endpoints. The DC AS sends Ndcsf_MediaControl_MediaInstruction to instruct the DCSF about the MDC2 related media information.

6.	The DCSF sends 200 OK indicating it receives the instruction.

NOTE:	It is up to operator’s configuration and implementation that how the DCSF knows that the DC AS will send instruction to the DCSF. A possible implementation is that the DCSF will continue the following steps after a fix timer is expired.

7.	Steps 6-8 of TS 23.228 [2] Annex AC.7.2.3 applies. The DCSF shall send all the MDC2 media related information received from the DC AS to the MF. In this case, the MF receives two sets of MDC2 endpoints.

8.	Steps 10-18 of TS 23.228 [2] Annex AC.7.2.3 applies. The DCSF will instruct the MF via IMS AS to update the media related information after receiving SDP answer from the terminating side.

9.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that the application data channel establishment success event. The DCSF will also forward MDC2 related information from the MF to the DC AS. In the P2A2P scenario, two sets of endpoint information shall be sent to the DC AS.

10.	The DC AS sends 200 OK indicating it receives the notification.

11.	Steps 19-26 of TS 23.228 [2] Annex AC.7.2.3 applies.

6.10.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DC AS:

-	sending subscription request to the DCSF.

-	receiving notification from the DCSF which may include MDC2 related media information.

-	sending DC AS side MDC2 related media information to DCSF.

DCSF:

-	receiving subscription from DC AS.

-	notifying the session events to the DC AS.

-	sending MF side MDC2 related media information to DC AS.

-	receiving DC AS side MDC2 related media information from DC AS.

# * * * * End of changes * * *