3GPP TSG-SA WG2 Meeting #172                                                                   S2-2510119

Dallas, USA, 17 – 21 November 2025

Source:	vivo

Title:	[WT#2, Migration and interworking] Migration and interworking

Document for:	Approval

Agenda Item:	20.6.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: this contribution introduces the work task and key issues for WT#2 (migration and interworking) based on the baseline tdoc S2-2509802.

1. Justifications

During SA2#171 meeting, the baseline paper S2-2509802 contains the WT of migration and interworking, as following:

Study migration and interworking, including

-	How to support migration to 6GS

-	How to support interworking with 5GS

-	Whether and how to support interworking with EPS

-	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 1:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is required to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 2: The detailed migration study scope will be coordinated and aligned with RAN.

NOTE 2a:	Additional migration options, if included by TSG RAN as per [ref: RP-252912], beyond stand-alone, MRSS and inter-RAT mobility between NR and 6GR, will be studied (if needed) in alignment with TSG RAN timeline.

NOTE 3:	It is assumed that interworking for roaming is within scope of this task.

NOTE 4:	WT2 focuses on general interworking procedures. The specific interworking aspects studied in other WTs (if any) need to follow the general interworking procedures.

- General interworking procedure

For the NOTE4, it is not clear what is the general interworking procedure.

As specified in TS 23.501 and TS 23.502, the interworking procedure in 5GS contains the following procedures:

- inter-RAT handovers procedure

- idle-mode mobility procedure

- PDU Session establishment/modify procedure with mapped QoS parameters

Proposal 1: the general procedure in NOTE4 should contain MM and SM related procedures.

- Interworking architecture:

In 5GS, architecture for interworking between 5GS and EPC/E-UTRAN is specified as figure 1:

Figure 1: Non-roaming architecture for interworking between 5GS and EPC/E-UTRAN

The key features of interworking between 5GS and EPC/E-UTRAN as blow:

- A N26 interface between 5G AMF and MME is specified to transfer UE context during interworking.

- The combined NF, i.e. SMF+PGW-C and UPF+PGW-U, dedicated for interworking between 5GS and EPC are specified to guarantee IP continuity during interworking.

For interworking between 6GS and 5GS, the same key features may also need to be considered:

Whether and how to define the interface between 5GC and 6G CN

To support interworking between 6G and 5G, the UE contexts may also need to be transferred between 6G CN(e.g. 6G AMF) and 5GC(e.g. 5G AMF), so the interface is needed.

But whether the interface is a N26 like interface or service-based interface needs to be studied considering both 5GS and 6GS support SBA.

Proposal 2: Study whether and how to define the interface between 5GC and 6G CN.

Whether and how to introduce combined NF

For User Plane (UP), to support IP continuity from 6GS to 5GS for UE, and considering the UE may need to further transfer from 5GS to EPS, the 6G UPF may need to combined with 5G UPF+PGW-U i.e., the combined 6G UPF+5G UPF+PGW-U needs to be introduced.

For Control Plane(CP), two options exist：

- Option 1: introduce a combined NF: 6G SMF+5G SMF+PGW-C

- Option 2: separate 6G SMF and 5G SMF+PGW-C,

both 6G SMF and 5G SMF+PGW-C can control 6G UPF+5G UPF+PGW-U

Option 1 may lead to the combined CP NF complex and not easy to support new 6G features.

Option 2 may need to specify the interface between 6G SMF and 5G SMF+PGW-C, but much easier to support new 6G features.

Proposal 3: study whether and how to define the combined NFs dedicated for interworking between 6GS and 5GS.

- Interworking scenario:

When a UE has a service that requires IP continuity in 6GS (e.g., native IMS voice over PS session in 6GS) and the UE hands over from 6GS to 5GS due to a coverage problem, the network should guarantee that the UE can further hand over from 5GS to EPS if the UE loses 5G coverage.

Proposal 4: the scenario of interworking from 6GS to 5GS should be studied, and if the UE has been transferred from 6GS to 5GS, the UE can be further transferred to EPS.

2. Proposal:

It is proposed to add following changes to TR 23.801-01 V0.2.0.

**** First Change (based on paper S2-2509802) ****

Annex A.X. Migration and Interworking Scope

Study migration and interworking, including

-	How to support migration to 6GS

-	How to support interworking with 5GS

-	Whether and how to support interworking with EPS

-	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 1:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is required to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 2: The detailed migration study scope will be coordinated and aligned with RAN.

NOTE 2a:	Additional migration options, if included by TSG RAN as per [ref: RP-252912], beyond stand-alone, MRSS and inter-RAT mobility between NR and 6GR, will be studied (if needed) in alignment with TSG RAN timeline.

NOTE 3:	It is assumed that interworking for roaming is within scope of this task.

NOTE 4:	WT2 focuses on general interworking procedures (i.e. MM, SM related procedures). The specific interworking aspects studied in other WTs (if any) need to follow the general interworking procedures.

**** Next Change****

5.X. Key Issue #X: Migration and Interworking

Study migration and interworking, including

1	How to support migration to 6GS.

2	How to support interworking with 5GS (including when using 5G NTN/satellite access).

- how to support transferring from 6GS to 5GS for a UE.

      - After the UE transfers from 6GS to 5GS, the UE can further transfer to EPS immediately.

- how to support transferring from 5GS to 6GS for a UE.

      - Before the UE transfers from 5GS to 6GS, the UE can transfer from EPS to 5GS.

3	Whether and how to support interworking with EPS (including when using 4G NTN/satellite access).

4. Study the architecture for interworking between 6GS and 5GS, including:

- whether and how to define the interface between 6G CN and 5GC.

- whether and how to define the combined NFs dedicated for interworking between 6GS and 5GS.

NOTE 1:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 2: The detailed migration study scope will be coordinated and aligned with RAN.

NOTE 3: The Work Task should consider roaming aspects.

NOTE X: The interworking from 6GS to 5GS or from 6GS to EPS may be used to perform “fallback” procedure for IMS voice service when native IMS voice over PS session in 6GS is not support for an operator.

**** End of Changes ****